3GPP TSG-RAN WG4 Meeting #113            	R4-2418936

Orlando, USA, 18th-22nd, November, 2024

Agenda item:	7.8.4

Source:	Samsung

Title:	Discussion on NTN testing NGSO channel model

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the NGSO channel model for NTN testing was under discussion, the related agreements are captured in the WF [1] as following

- TE emulated channel model for satellite mobility - Methodology for Time varying Doppler and Delay shifts modelling - RAN4 aims to introduce TE-emulated time varying doppler shift and delay shift model for satellite motion with following option - Satellite motion emulated using the two-body Keplerian propagator model, i.e., spherical gravity model - RAN4 further discuss how to derive the Doppler shift and propagation delay variation based on Keplerian model with the necessary parameters and details procedure - Companies are encouraged to provide the two-body Keplerian propagator model to be implemented in the TE, to derive the Doppler shift and propagation delay - Assumption of satellite altitude - Prioritize the LEO-600 orbit for TE emulated channel modelling - FFS on the LEO-1200 orbit for TE emulated channel modelling pending on the test purpose - Assumption of UE motion & UE position - Same UE location information should be set for both DUT and TE before the test start with the following initial position - Assuming UE is stationary during demodulation and RF requirement testing - FFS on the assuming UE is in motion during the RRM requirement testing - Assumption of Elevation angle range - Option 1 - In the configuration for NGSO test cases, the satellite information for the serving cell shall be initiated at a position where the elevation angle measured by the UE at the chosen GNSS position used for the test is below [35] degrees and the satellite is moving away from the UE. - This configuration applies regardless if the UE position is obtained via AT command or via GNSS simulator. - The initial elevation angle might be increased if needed for guaranteeing the GNSS is within the limit of 30 degrees throughout the entire duration of the test. - Option 2: To align with Rel-17 NR NTN channel model discussion, the elevation angle with 30 degrees and 90 degrees can be considered for testing. Large testing period is required for IoT NTN testing - Other options are not precluded - Assumption of feeder link delay - Do not consider feeder link delay - Assumption of updated granularity for Satellite ephemeris information - Companies are encouraged to provide the view for RAN4#113 - Satellite ephemeris information will be updated at least every 10.24 seconds as the worst case according 38.331 - Note: other (smaller) value may be considered depending on the test purpose - Assumption of TE updated granularity for Doppler and timing offset - The Doppler shift and propagation delay should be calculated and updated based on agreed TE emulated channel model - TE vendors are encouraged to provide the feedback about the implementation feasibility to update the Doppler shift and propagation delay per slot - Assumption of Multiple satellite scenario - RAN4 focuses TE emulated channel modeling for one satellite - FFS on the TE emulated channel modelling for multiple satellite scenario, considering the serving satellite changes. - Time varying of doppler shift and propagation delay generation procedure - RAN4 further discuss on general test principle and how to generate channel model to match with satellite motion trajectory with the following options - Option 1: The entire N orbits within a test time of X seconds are generated as follow - The initial ephemeris for each orbit is generated by either GMAT or any numerical method. - The initial ephemeris for each orbit should be chosen such that the elevation angle from the UE perspective is not smaller than Z degrees. The value of Z may vary for different test cases. And if multiple satellites (i.e., N > 1) are needed in RRM mobility/measurement test cases, the value of Z may be different for different satellites. - The ephemeris data after the initial data is generated based on the two-body Keplerian model at every Y sec. The value of Y may differ for NR NTN and IoT NTN. - The propagation delay and Doppler shift at every Y sec are derived based on the projected ephemeris data and given UE location - In case Y is larger than the slot length of interest in the test case due to, e.g., test equipment/channel emulator complexity, the propagation delay and Doppler shift at the beginning of each slot can be generated by applying, e.g., linear interpolation between the propagation delays and Doppler shifts at Y sec intervals. - Other options are not precluded - RAN4 requirements and tests - UE assumption on propagator model for satellite motion for RAN4 simulation purpose - Option 1: Consider Keplerian model for UE assumption on the propagator model - Option 2: Evaluate the performance based on the agreed TE emulated channel model first. It is not necessary to have assumptions on the propagator model for satellite motion at the UE side if the simulation results from companies based on the agreed TE emulated channel model are within some span. - RF requirements - Once sufficient progress made for channel model introduction, RAN4 can start the discussion on corresponding requirements and tests according to WI objective - Inclusion in specification - Inclusion in specification - Specification of Channel Model: RAN4 further discuss how to incorporate the channel model into specification pending on channel model discussion progress

In this contribution, the view on the methodology for time varying Doppler Shift and propagation delay modeling was provided.

# 2	Discussion

Assumption of Satellite altitude

For FR1 NTN scenario, the satellite types with LEO-600 and LEO-1200 are available and considered as the typical deployment. In the last meeting, it was agreed to prioritize LEO-600 orbit for TE emulated channel modelling. And FFS for LEO-1200 pending on the test purpose.

In Rel-17 NR NTN and Rel-18 IoT over NTN WI, it was agreed that there is no time-varying propagation delay modeling, and assuming UE compensation functionality will be covered by other requirements. Therefore, after UE frequency and timing pre-compensation, there is minor different between LEO-600 and LEO-1200. So, only LEO-600 scenario was considered for performance requirement definition.

For modeling the satellite motion trajectory perspective, the main difference between LEO-600 and LEO-1200 is the doppler shift and propagation delay.

From Doppler shift perspective, LEO 600 will suffer the largest one, and up to 24ppm.

From propagation delay perspective, LEO 1200 will suffer the largest one, and up to 6.67ms.

Based on RAN 5 test in TS 38.521-5, for RF test related with frequency error, both LEO 600 and LEO 1200 are included. And for demod test, only LEO 600 is considered.

##### 6.4.1_2.4.2 Test procedure Same test procedure as in clause 6.4.1_1.4.2 with the following exceptions: - In step 1, instead of TS 38.508-1 [12] Table 5.6.2.1-1 use TS 38.508-1 [12] Table 5.6.2.1-3 (ephemeris for NGSO LEO 1200). - Instead of Tables 6.4.1_1.4.3-1a, 6.4.1_1.4.3-2a, and 6.4.1_1.4.3-3a use Tables 6.4.1_2.4.3-1, 6.4.1_2.4.3-2, and 6.4.1_2.4.3-3, respectively. - If the UE supports GSO and NGSO, skip steps 1 to 6 of the test procedure and start with step 7

- In case UE supports only NGSO satellites, ephemeris values in Table 5.6.2.1-3 shall be used except for test cases 8.2.1.2, where ephemeris values in Table 5.6.1.2.1-2 shall be used. - In case UE supports both GSO and NGSO satellites, ephemeris values in Table 5.6.1.2.1-1 shall be used except for test cases 8.2.1.2, where ephemeris values in Table 5.6.1.2.1-2 shall be used.

Observation 1: Both LEO 600 and LEO 1200 are considered into the RF test, only LEO 600 was considered into the demodulation test

Therefore, considering satellite types both LEO-600 and LEO-1200 for TE-emulated time varying doppler shift and delay shift modeling for satellite motion trajectory, and the applicable of channel model for testing can be pending on test purpose.

Proposal 1: Considering satellite types both LEO-600 and LEO-1200 for TE-emulated time varying doppler shift and delay shift modeling for satellite motion trajectory, and the applicable of channel model for testing can be pending on test purpose.

Assumption of UE motion& UE position

Regarding the UE motion, in the last meeting, it was agreed that assuming UE is stationary during the demodulation and RF testing. Based on the RAN5 conformance test, the position of UE is assumed in a fixed position on Earth as following

## 4.13 UE position requirements for NTN testing Clauses 4.14 and 8.1.5B define the common test environment which applies to all test cases executed for eMTC and NB-IoT NTN UEs, unless otherwise specified. - UE's positioning engine (e.g., standalone GNSS receiver) shall be enabled to allow it to acquire the position. UE position during the test is stationary. Below position shall be used to pre-configure UE position: - Latitude: 25.08439333 - Longitude: 121.56076999 - Altitude: 0

For UE motion, considering, it will impact on UE RRM requirement, such as mobility and measurement test cases, where the UE position will be configured based on the AT command

Table A.14.2.1.3.2-2: General test parameters for Intra-frequency SAN time-based conditional handover from FR1 to FR1 Parameter; Unit; Value; Comment. RF Channel Number; 1; One NR NTN satellite RF channel. Initial conditions; Active cell; Cell 1; FDD duplex mode cell. Neighbouring cell; Cell 2; FDD duplex mode cell. Final condition; Active cell; Cell 2. Satellite configuration; Config 1; RMC in [A.x]; For GSO satellite configuration. Config 2; RMC in [A.x]; For NGSO satellite configuration. UE position (N,S, H); [(0, 0, 0)]; Set by AT command.

Observation 2: UE position will be configured by AT command during the RRM test for handover requirement.

While based on the WID, only the UL transmit timing requirement will be specified based on the TE-emulated channel model.

- Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement

Observation 3: Only uplink timing requirement will be specified based on TE-emulated channel model

According to UE transmit timing test case, the UE position is not set by AT command. And also, some margin for GNSS position error has been taken into account. Therefore, it is not necessary to assume the UE motion for TE-emulated time varying doppler shift and propagation delay.

Meanwhile, according to previous agreement

- Predefined files which generated by GMAT can be considered as reference for the comparison purpose of proposed channel model

Where the related predefined files were generated based on UE reference coordinated considered as

- Latitude 25.08439333 degrees;

- Longitude 121.56076999 degrees;

- Altitude 0 m.

Therefore, UE position for uplink timing requirement can also be stationary and configure the UE position as following

- Latitude 25.08439333 degrees;

- Longitude 121.56076999 degrees;

- Altitude 0 m.

Proposal 2: UE position for uplink timing requirement can also be stationary and configure the UE position as following

-	Latitude 25.08439333 degrees;

-	Longitude 121.56076999 degrees;

-	Altitude 0 m.

Assumption of Elevation angle range

Regarding the evaluation angle, it will impact on the delay trajectory and doppler trajectory visible range.  With small evaluation angle, the worst-case scenario can be covered, while it requires long testing time to cover the visible satellite coverage range. The testing time should be considered. With larger evaluation angle, the test coverage time can be reduced, where the doppler shift and delay shift are relatively small.

Therefore, the proper evaluation angle should be selected considering the test time and satellite motion trajectory. Based on the profile provided for doppler shift and propagation delay with satellite type of both LEO 600 and LEO 1200 with 88o orbit inclination in [2] as following

Figure 1:  LEO 600km with 88o orbit inclination

Figure 2:  LEO 1200km with 88o orbit inclination

Observation 4: Based on elevation range of [30°~150°], around 300s elapsed time is required for LEO 600 and around 500s elapsed time is required for LEO 1200. Based on elevation range of [10°~170°], around 500s elapsed time is required for LEO 600 and around 900s elapsed time is required for LEO 1200.

For demodulation test, in Rel-17 NR NTN channel model discussion, the evaluation angle with 30°was selected for NTN-TDLA channel case. To align with Rel-17 NTN channel model discussion, evaluation angle range with [30°~150°] can be considered as the proper range for test with affordable test timing for NR NTN

Meanwhile, since TE-emulated channel model will also be applied for IoT NTN. Different with NR, larger number of repetitions transmission is required, there, the large number of testing time is required. In current IoT NTN test, the maximum number of repetition transmission is configured as 128. Therefore, larger testing time, i.e., [10°~170°] is needed considering to derive the converged requirement for IoT NTN testing.

For in current RF test, the elevation range of [10 °~170°] was considered, considering the maximum doppler shift.

Table 6.4.1_2.4.3-1: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2717617 |  |  |
| positionY-r17 | 4550419 |  |  |
| positionZ-r17 | 852799 |  |  |
| velocityVX-r17 | 6164 |  |  |
| velocityVY-r17 | -19424 |  |  |
| velocityVZ-r17 | 124281 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 10 degrees, one-way delay equal to 6.44 ms and Doppler equal to 22.65 ppm. |  |  |  |

Table 6.4.1_2.4.3-2: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum negative Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2199272 |  |  |
| positionY-r17 | 3404229 |  |  |
| positionZ-r17 | 3535794 |  |  |
| velocityVX-r17 | 35394 |  |  |
| velocityVY-r17 | -74414 |  |  |
| velocityVZ-r17 | 94682 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 169.97 degrees, one-way delay equal to 6.60 ms and Doppler equal to -22.62 ppm. |  |  |  |

Proposal 3: For demodulation test, to align with Rel-17 NTN channel model discussion, the elevation angle with [30°~150°] can be considered for testing. Large testing period, i.e., [10°~170°] can be considered for IoT NTN testing. For RF test, [10°~170°] can be considered

Assumption of updated granularity for Satellite ephemeris information

Regarding the updated granularity for satellite ephemeris information, in current RRM test, 2.56s was considered.

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Interval between adjacent epoch time | s | LEO: 2.56 GEO: 10.24 |
| ntn-UlSyncValidityDuration | s | LEO: 5 GEO: 900 |
| cellSpecificKoffset | slot | LEO: 14 GEO: 258 |
| kmac | slot | Not configured |
| ta-Common |  | 0 |
| ta-CommonDrift |  | 0 |
| ta-CommonDriftVariant |  | 0 |
| ntn-PolarizationDL |  | linear |
| ntn-PolarizationUL |  | linear |
| ephemerisInfo |  | Detailed ephemeris information is provided in TS 38.508-1 [38] |
| ta-Report |  | Not configured |

Proposal 4: Reuse the existing updated granularity for Satellite ephemeris information as 2.56s for RRM test

Assumption of Multiple satellite scenario

From the channel modeling perspective, the difference between one satellite and multiple satellite should be similar. The same model can be used with different satellite position. Therefore, if the TE-emulated channel model for one satellite, it can be easily to extent to multiple satellites scenarios.

Meanwhile, according to WI, only uplink timing requirement will be specified based on these TE-emulated channel model, it was not targeting for RRM requirement with mobility, such as handover. Therefore, there is no need to consider the TE emulated channel model

Proposal 5: No need to consider the TE emulated channel modelling for multiple satellite scenario, considering the serving satellite changes

Methodology for Time varying Doppler and Delay Shifts modeling

In the last meeting, RAN4 has agreed to use the two-body Keplerian propagator model for emulating the satellite motion. And how to derive the doppler shift and propagation delay variation based on Keplerian model with necessary parameters and details procedure will be further discussed.

In order to derive the doppler shift and propagation delay, the estimated satellite location/velocity information should base on the euphemism’s information

According to TS.38.331, the required satellite ephemeris information is included in SIB19 with two potential information: ECEF information (satellite position and velocity vector) and/or Orbit information (Kepler elements) as following

-- ASN1START

-- TAG-EPHEMERISINFO-START

EphemerisInfo-r17 ::=          CHOICE {

positionVelocity-r17           PositionVelocity-r17,

orbital-r17                    Orbital-r17

}

PositionVelocity-r17 ::=       SEQUENCE {

positionX-r17                  PositionStateVector-r17,

positionY-r17                  PositionStateVector-r17,

positionZ-r17                  PositionStateVector-r17,

velocityVX-r17                 VelocityStateVector-r17,

velocityVY-r17                 VelocityStateVector-r17,

velocityVZ-r17                 VelocityStateVector-r17

}

Orbital-r17 ::=                SEQUENCE {

semiMajorAxis-r17              INTEGER (0..8589934591),

eccentricity-r17               INTEGER (0..1048575),

periapsis-r17                  INTEGER (0..268435455),

longitude-r17                  INTEGER (0..268435455),

inclination-r17                INTEGER (-67108864..67108863),

meanAnomaly-r17                INTEGER (0..268435455)

}

PositionStateVector-r17 ::= INTEGER (-33554432..33554431)

VelocityStateVector-r17 ::= INTEGER (-131072..131071)

-- TAG-EPHEMERISINFO-STOP

-- ASN1STOP

Figure 1: The geometrical relationship between satellite and earth

As illustrated in Figure 1, the ephemeris information can be indicated either by satellite position and velocity vector or by orbit parameters (six parameters for Keplerian model), they are equivalent, can be transferred into each other

- The following are the related formulas for transferring of orbital information (Kepler parameters) into the satellite information (position velocity).

- The following table 1 and table 2 are the orbit parameters and position velocity parameters

- Table 1:  orbit parameters

| - Orbital-r17 | - Unit | - Value | - Abbreviation |
| --- | --- | --- | --- |
| - semimajorAxis-r17 | - Meter |  | - a |
| - eccentricity-r17 | - Radian |  | - e |
| - periapsis-r17 | - Radian |  | - ω |
| - longitude-r17 | - Radian |  | - Ω |
| - inclination-r17 | - Radian |  | - i |
| - meanAnomaly-r17 | - Radian |  | - M |
| - gravitation constant | - m^3/s^2 | - 398600.44 | - μ |

- Table 2:  position velocity parameters

| - PositionVelocity-r17 | - Unit | - Value | - Abbreviation |
| --- | --- | --- | --- |
| - positionX-r17 | - meter |  | - X |
| - positionY-r17 | - meter |  | - Y |
| - positionZ-r17 | - meter |  | - Z |
| - velocityVX-r17 | - meter/second |  | - VX |
| - velocityVY-r17 | - meter/second |  | - VY |
| - velocityVZ-r17 | - meter/second |  | - VZ |

- Based on the orbit parameters, the following positioning vector

P=a*(1-e2)

r=P(1+e*cos⁡(M))

Then, the position vector (X, Y, Z) for satellite can be calculated as

X=r*cosΩ*cosω+M-sinΩ*sinω+M*cosi

Y=r*sinΩ*cosω+M+cosΩ*sinω+M*cosi

Z=r*sinω+M*sin⁡(i)

And the position velocity vector (VX, XY, VZ) for satellite in ECEF can be calculated as

VX=-μP*sinM*(cosΩ*sinω+M+sinΩ*cosω+M*cosi)

+μP*e+cosM*(sinΩ*sinω+M-cosΩ*cosω+M*cosi)

VY=μP*sinM*(sin⁡(Ω)*sinω+M-cosΩ*cosω+M*cosi)

-μP*e+cosM*(cosΩ*sinω+M+sinΩ*cosω+M*cosi)

VZ=μP*e+cosM*sinω+M*sini+μP*sinM*cos⁡(i)

- Similarly, the satellite information (position velocity) can also be transferred into the orbital information.

- Given the satellite position r=(X,Y,Z) and velocity vector v=(VX,VY,VZ)

- The inclination i and longitude Ω  can be calculated as

- i=arccos⁡(hx/|h|)

- Ω=arctan⁡(-hx/hy)

- Where h is vector calculated with following

- h=r ×v=(Y*VZ-Z*VY, -X*VZ+Z*VX,(X*VY-Y*VX))

- Then, the semimajorAxis a is calculated as

- E=v22-μ|r|

- a=-μ2E

- Then, the eccentricity e is calculated as

- p=|h|2μ

- e=1-(p/a)

- Where meanAnomaly M is calculated as

- M=arccosp-r/(|r|*e)

- Then, the periapsis ω is calculated as

- ω=u-M

- u=arccos⁡(ON*r/(|r|*|ON|)

- Where ON=(cosΩ,sinΩ, 0)

Observation 5: The satellite position/ velocity vector and orbit parameters (six parameters for Keplerian model) are equivalent, can be transferred into each other

According to RAN5 test, position velocity state vectors are used for Ephemeris info signaling

#### 5.6.2.0 Assumptions for Ephemeris generation This section provides the assumptions used to generate Ephemeris information for GSO and NGSO orbits: 1. Ephemeris files are generated for 3 types of satellites: GSO (inclination equal to 7º), LEO-600 and LEO-1200. 2. UE location is as defined in section 5.6.1. 3. PositionVelocity state vectors are used for EphemerisInfo signaling. 4. For GSO, an altitude of 35,786 km is considered. For NGSO, 2 different altitudes are considered: 600 km and 1200 km. 5. One-way delay between UE and satellite to be in the range of 119.375 ms to 128.79 ms for GSO and between 2 ms (lowest value for LEO orbit 600 km) and 6.67 ms (highest value for LEO orbit 1200 km)

Regarding the initial Ephemeris information, since it was agreed that the predefined files generated by GMAT can be considered as reference for compassion purpose of proposed channel model, so, the initial ephemeris information can be generated by GMAT. Based on the following transfer, the orbit parameters (six parameter) can be generated and used for Keplerian model.

Based on the initial ephemeris information, the estimated ephemeris information can be generated based on the two-body Keplerian model with given time

Given the time t, the meanAnomaly M(t) can be calculated as Mt=M0+nt-t0 n=μa3 Then, based on the Kerplain equation to calculate eccentric anomaly E E=M(t)+e*sin(E) Calculate the Distance to the central body r=a(1-e cos⁡(E)) Calculate the true anomaly θ=2*atan1+e1-e*tan⁡(E2) Position vector (X1, Y1,Z1) in orbit plane X1=r*cosθ Y1=r*sinθ Z1=0 Velocity vector (VX1, VY1,VZ1) in orbit plane VX1=-μ*ar*sin⁡(E) VY1=μ*ar*1-e2*cos⁡(E) VZ1=0 Position vector in (X2, Y2,Z2) in ECEF with rotation operation X2Y2Z2=R3R2R1X1Y1Z1 Velocity vector in (VX2, VY2,VZ2) in ECEF with rotation operation VX2VY2VZ2=R3R2R1VX1VY1VZ1 R3=cos⁡(Ω)-sin⁡(Ω)0sin⁡(Ω)cos⁡(Ω)0001 R2=1000cos⁡(i)-sin⁡(i)0sin⁡(i)cos⁡(i) R1=cos⁡(ω)-sin⁡(ω)0sin⁡(ω)cos⁡(ω)0001 Based on the estimation satellite position vector (X2, Y2,Z2) and velocity vector (VX2, VY2,VZ2) in the time t, the doppler shift and propagation delay can be calculated as following Assuming the UE location is (x,y,z) - UE position-r17; - Value; - Abbreviation. - Latitude; - degree; - la. - Longitude; - degree; - lo. - Altitude; - km; - A. x=N+Acosϕcos⁡(λ) y=N+Acosϕsin⁡(λ) z=(N1-e2+A)sin⁡(ϕ) where ϕ=π180la λ=π180la N=R1-e2sin2⁡(ϕ) Assuming the satellite is moving far from the UE, the frequency doppler can be calculated as Doppler t=-fc*(VX2*X2-x+VY2*Y2-y+VZ2*(Z2-z))(X2-x)2+(Y2-y)2+(Z2-z)2 And propagation delay can be calculated as T(t)=(X2-x)2+(Y2-y)2+(Z2-z)2c - Where c is Speed of light (km/s)

Therefore, the time varying of doppler shift and propagation delay generation procedure can be considered as

Step 1: The initial ephemeris information for each orbit is generated by GMAT based on the following table

Table 1 Parameters related with initial ephemeris information

| - PositionVelocity-r17 | - Unit | - Value | - Abbreviation |
| --- | --- | --- | --- |
| - positionX-r17 | - meter |  | - X |
| - positionY-r17 | - meter |  | - Y |
| - positionZ-r17 | - meter |  | - Z |
| - velocityVX-r17 | - meter/second |  | - VX |
| - velocityVY-r17 | - meter/second |  | - VY |
| - velocityVZ-r17 | - meter/second |  | - VZ |

Step 2: mapping the initial ephemeris information as the Keplerian model input parameter as table 2

| - Orbital-r17 | - Unit | - Value | - Abbreviation |
| --- | --- | --- | --- |
| - semimajorAxis-r17 | - Meter |  | - a |
| - eccentricity-r17 | - Radian |  | - e |
| - periapsis-r17 | - Radian |  | - ω |
| - longitude-r17 | - Radian |  | - Ω |
| - inclination-r17 | - Radian |  | - i |
| - meanAnomaly-r17 | - Radian |  | - M |
| - gravitation constant | - m^3/s^2 | - 398600.44 | - μ |

Step 3: Generate the orbit information based on two-body Keplerian model and derive the ephemeris information at time t;

Step 4: Derive the propagation delay and Doppler shift at time t based on the projected ephemeris data and given UE location;

Proposal 6: The time varying of doppler shift and propagation delay generation procedure can be considered as

Step 1: The initial ephemeris information for each orbit is generated by GMAT based on the following table

Table 1 Parameters related with initial ephemeris information

| - PositionVelocity-r17 | - Unit | - Value | - Abbreviation |
| --- | --- | --- | --- |
| - positionX-r17 | - meter |  | - X |
| - positionY-r17 | - meter |  | - Y |
| - positionZ-r17 | - meter |  | - Z |
| - velocityVX-r17 | - meter/second |  | - VX |
| - velocityVY-r17 | - meter/second |  | - VY |
| - velocityVZ-r17 | - meter/second |  | - VZ |

Step 2: mapping the initial ephemeris information as the Keplerian model input parameter as table 2

| - Orbital-r17 | - Unit | - Value | - Abbreviation |
| --- | --- | --- | --- |
| - semimajorAxis-r17 | - Meter |  | - a |
| - eccentricity-r17 | - Radian |  | - e |
| - periapsis-r17 | - Radian |  | - ω |
| - longitude-r17 | - Radian |  | - Ω |
| - inclination-r17 | - Radian |  | - i |
| - meanAnomaly-r17 | - Radian |  | - M |
| - gravitation constant | - m^3/s^2 | - 398600.44 | - μ |

Step 3: Generate the orbit information based on two-body Keplerian model and derive the ephemeris information at time t;

Step 4: Derive the propagation delay and Doppler shift at time t based on the projected ephemeris data and given UE location

# 3	Conclusion

In this contribution, the view on the NTN testing NGSO channel model is provided.

Observation 1: Both LEO 600 and LEO 1200 are considered into the RF test, only LEO 600 was considered into the demodulation test

Proposal 1: Considering satellite types both LEO-600 and LEO-1200 for TE-emulated time varying doppler shift and delay shift modeling for satellite motion trajectory, and the applicable of channel model for testing can be pending on test purpose.

Observation 2: UE position will be configured by AT command during the RRM test for handover requirement.

Observation 3: Only uplink timing requirement will be specified based on TE-emulated channel model

Proposal 2: UE position for uplink timing requirement can also assume to be stationary and configure the UE position as following

-	Latitude 25.08439333 degrees;

-	Longitude 121.56076999 degrees;

-	Altitude 0 m.

Observation 4: Based on elevation range of [30°~150°], around 300s elapsed time is required for LEO 600 and around 500s elapsed time is required for LEO 1200. Based on elevation range of [10°~170°], around 500s elapsed time is required for LEO 600 and around 900s elapsed time is required for LEO 1200.

Proposal 3: For demodulation test, to align with Rel-17 NTN channel model discussion, the elevation angle with [30°~150°] can be considered for testing. Large testing period, i.e., [10°~170°] can be considered for IoT NTN testing. For RF test, [10°~170°] can be considered

Proposal 4: Reuse the existing updated granularity for Satellite ephemeris information as 2.56s for RRM test

Proposal 5: No need to consider the TE emulated channel modelling for multiple satellite scenario, considering the serving satellite changes

Observation 5: The satellite position/ velocity vector and orbit parameters (six parameters for Keplerian model) are equivalent, can be transferred into each other

Proposal 6: The time varying of doppler shift and propagation delay generation procedure can be considered as

Step 1: The initial ephemeris information for each orbit is generated by GMAT based on the following table

Table 1 Parameters related with initial ephemeris information

| - PositionVelocity-r17 | - Unit | - Value | - Abbreviation |
| --- | --- | --- | --- |
| - positionX-r17 | - meter |  | - X |
| - positionY-r17 | - meter |  | - Y |
| - positionZ-r17 | - meter |  | - Z |
| - velocityVX-r17 | - meter/second |  | - VX |
| - velocityVY-r17 | - meter/second |  | - VY |
| - velocityVZ-r17 | - meter/second |  | - VZ |

Step 2: mapping the initial ephemeris information as the Keplerian model input parameter as table 2

| - Orbital-r17 | - Unit | - Value | - Abbreviation |
| --- | --- | --- | --- |
| - semimajorAxis-r17 | - Meter |  | - a |
| - eccentricity-r17 | - Radian |  | - e |
| - periapsis-r17 | - Radian |  | - ω |
| - longitude-r17 | - Radian |  | - Ω |
| - inclination-r17 | - Radian |  | - i |
| - meanAnomaly-r17 | - Radian |  | - M |
| - gravitation constant | - m^3/s^2 | - 398600.44 | - μ |

Step 3: Generate the orbit information based on two-body Keplerian model and derive the ephemeris information at time t;

Step 4: Derive the propagation delay and Doppler shift at time t based on the projected ephemeris data and given UE location

# References

R4-2416556, Way Forward for [112bis][321] NTN_testing_NGSO_channel_model

R4-2413333