3GPP TSG-RAN WG4 Meeting#113							         		R4-2418942

Orlando, Florida, USA, 18 – 22 November 2024

Title: 	New NS_05 DECT protection in Japan and A-MPR regions

Source: 	Skyworks Solutions, Inc.

Agenda Item:	5.33.1	UE RF related topics	[TEI18]

Document for:	Discussion

# 1	Background

In RAN#112bis, Japanese operators informed RAN4 in [1] of a regulation change due to PHS operation shutdown, but that DECT cordless protection was still needed. This implies a change of NS_05 and UE coexistence requirements. In this contribution, we analyze the new DECT protection requirements versus the SEM and spurious emission requirement to assess which channel and allocation regions may need A-MPR.

# Discussion

## 2.1 New DECT protection requirement

Based on the operators’ input on PHS shutdown in Japan, and the need to still protect DECT cordless phones, the domestic PHS protection requirement will be modified to protect DECT as follows:

- For n1/B1:

  - 5MHz CBW: 1884.5-1910.0 MHz: -30 dBm/MHz and 1910.0-1915.7 MHz: -25 dBm/MHz

  - 10/15/20MHz CBW: 1884.5-1906.6 MHz: -30 dBm/MHz and 1906.6-1915.7 MHz: -25 dBm/MHz

- For other bands: 1884.5-1915.7 MHz: -30 dBm/MHz

Some early analysis of the A-MPR regions required for the new protection requirement was already provided in [1] but we further discuss here how SEM or SE, thus MPR, may already provide the right level of protection in term of both channel and allocation position.

Observations:

- For other bands than band 1/n1 the DECT protection is guaranteed by the spurious emission level if the largest UL channels in operation in Japan are at least CBW+5MHz away from the 1884.5-1915.7 MHz DECT frequency range.

  - Based on our check this is the case for all bands of interest including band 3/n3

- For band 1/n1:

  - The SEM mask covers the -25 dBm/MHz level, and 5MHz lower in frequency, the spurious emission covers the -30dBm/MHz level, thus A-MPR would not be needed for all RB allocations as long as:

    - The 5MHz UL channel is at least 6MHz above 1915.7MHz. Thus, the 5MHz UL channels with Fc would not require any AMPR above 1924.2MHz.

    - The 10/15/20MHz UL channels are at least CBW above 1915.7MHz. Thus:

      - 10MHz UL channel Fc above 1930.7MHz would not require AMPR.

      - 15MHz UL channel Fc above 1938.2MHz would not require AMPR.

      - 20MHz UL channel Fc above 1945.7MHz would not require AMPR.

    - By analogy, accounting for the NR guard band, for lower frequency channels, MPR may be sufficient for any allocation above:

      - 1915.7+6+0.2425=1921.9425MHz for 5MHz CBW

      - 1915.7+10+0.3125=1926.0125MHz for 10MHz CBW

      - 1915.7+15+0.3825=1931.0825MHz for 15MHz CBW

      - 1915.7+20+0.4525=1935.7MHz for 20MHz CBW

  - With simulations/measurements it may be found that channels or allocations at lower frequency may pass but the above is a good start to limit the channel/allocation regions to be explored for the A-MPR work.

  - Since the -25dBm/MHz DECT protection range is always > 5MHz and the -25dBm/MHz SEM region is 5MHz, the -30dBm/MHz is de facto met once the -25dBm/MHz SEM coincides with the -25dBm/MHz DECT protection range.

To better illustrate this, Figure 1 shows the needed DECT protection, the SEM and SE for the different channel bandwidths with pass (dark green)/fail(red) regions for the lowest band 1/n1 channels (orange), and which channels have all their allocations pass (light green). Since the graph has a 1MHz resolution, the illustrated DECT protection range is slightly larger in Figure 1 compared to actual. It also means that the green “fully pass” channels are represented 0.3MHz above where they would sit based on SEM.

Figure 1: band n1 DECT protection requirement versus 5/10/15/20MHz CBW SEM and position.

# Conclusion

To simplify the new NS_05 A-MPR requirement work, in this contribution, we analyzed the new DECT protection requirement and compared it with the SEM and SE requirement and made the following observations on which channels and allocation may require A-MPR.

Observations:

- For other bands than band 1/n1 the DECT protection is guaranteed by the spurious emission level if the largest UL channels in operation in Japan are at least CBW+5MHz away from the 1884.5-1915.7 MHz DECT frequency range.

  - Based on our check this is the case for all bands of interest including band 3/n3

- For band 1/n1:

  - The SEM mask covers the -25 dBm/MHz level, and 5MHz lower in frequency, the spurious emission covers the -30dBm/MHz level, thus A-MPR would not be needed for all RB allocations as long as:

    - The 5MHz UL channel is at least 6MHz above 1915.7MHz. Thus, the 5MHz UL channels with Fc would not require any AMPR above 1924.2MHz.

    - The 10/15/20MHz UL channels are at least CBW above 1915.7MHz. Thus:

      - 10MHz UL channel Fc above 1930.7MHz would not require AMPR.

      - 15MHz UL channel Fc above 1938.2MHz would not require AMPR.

      - 20MHz UL channel Fc above 1945.7MHz would not require AMPR.

    - By analogy, accounting for the NR guard band, for lower frequency channels, MPR may be sufficient for any allocation above:

      - 1915.7+6+0.2425=1921.9425MHz for 5MHz CBW

      - 1915.7+10+0.3125=1926.0125MHz for 10MHz CBW

      - 1915.7+15+0.3825=1931.0825MHz for 15MHz CBW

      - 1915.7+20+0.4525=1935.7MHz for 20MHz CBW

  - With simulations/measurements it may be found that channels or allocations at lower frequency may pass but the above is a good start to limit the channel/allocation regions to be explored for the A-MPR work.

  - Since the -25dBm/MHz DECT protection range is always > 5MHz and the -25dBm/MHz SEM region is 5MHz, the -30dBm/MHz is de facto met once the -25dBm/MHz SEM coincides with the -25dBm/MHz DECT protection range.

# References

[1] R4-2415439 NR A-MPR reevaluation scenarios on n1 for Japan KDDI, NTT Docomo, RAN4#112