3GPP TSG-RAN WG4 Meeting #113	R4-2418967

# Orlando, US, 18 - 22 Nov, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0129 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_enh-Perf) CR on UE demodulation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | K_offset should be specified based on WF R4-2214386. - Agreement - K_offset for 120kHz SCS - 8 for NGSO - 258 for GSO. |
| Summary of change: |  | Specify K_offset in the test parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be inconsist between specification and RAN4 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.2.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <START OF THE CHANGE 1>

11.2.2.1.1.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are specified in Table 11.2.2.1.1.1-3 with the addition of test parameters in Table 11.2.2.1.1.1-2 and the downlink physical channel setup according to Annex A.3.

The test purposes are specified in Table 11.2.2.1.1.1-1.

Table 11.2.2.1.1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models and MCS | 1-1, 1-2, 1-3, 1-4, 2-1, 2-2, 2-3, 2-4 |

Table 11.2.2.1.1.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 160 for CSI-RS resource 1,2,3,4. |
|  | CSI-RS offset | Slots | 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4. |
| Number of HARQ Processes |  |  | 16 for Test 1-1, Test 1-2 32 for Test 1-3 4 with feedback disabled, 12 with feedback enabled in 16 HARQ processes for Test 1-4 in which 4 disabled processes are randomly selected at test configuration |
| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 1) |  |  | 80 for Test 1-1, Test 1-2, Test 1-3 and Test 1-4 2080 for Test 2-1, Test 2-2, Test 2-3 and Test 2-4 |
| cellSpecificKoffset (Note 1) |  | Slots | 64 for Test 1-1, Test 1-2, Test 1-3 and Test 1-4 2064 for Test 2-1, Test 2-2, Test 2-3 and Test 2-4 |
| Maximum number of HARQ transmission |  |  | 4 for Test 1-1, Test 1-2, Test 1-3 1 for Test 1-4 (re-Tx disabled for all HARQ processes) |
| Note 1: cellSpecificKoffset is included in the number of slots between PDSCH and corresponding HARQ-ACK information. |  |  |  |

Table 11.2.2.1.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.3-3.1 FDD | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 3.5 |
| 1-2 | R.PDSCH.3-4.1 FDD | 200 / 120 | 16QAM, 0.48 | NTN-TDLC5-1200 | 1x1 | 70 | 11.2 |
| 1-3 | R.PDSCH.3-3.1 FDD | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 3.5 |
| 1-4 | R.PDSCH.3-3.1 FDD (Note 1) | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 4.2 |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

Table 11.2.2.1.1.1-4: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.3-3.1 FDD | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 3.5 |
| 2-2 | R.PDSCH.3-4.1 FDD | 200 / 120 | 16QAM, 0.48 | NTN-TDLC5-1200 | 1x1 | 70 | 11.2 |
| 2-3 | R.PDSCH.3-3.1 FDD | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 3.5 |
| 2-4 | R.PDSCH.3-3.1 FDD (Note 1) | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 4.2 |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

# <END OF THE CHANGE 1>