3GPP TSG-RAN WG4 Meeting #113	  R4-2418971

Orlando, US, November 18 – 22, 2024

Agenda Item:	7.17.3.1

Source:	Huawei, HiSilicon

Title:	Discussion on testability and interoperability issues in AIML CSI prediction

Document for:	Discussion and Decision

# Introduction

According to WF in RAN4 #112 [1], we continue to discuss the remaining issues related to the test for AIML enabled CSI feedback enhancement.

# Test metric for AI CSI prediction

RAN4 #112 WF [1]: Use relative throughput as requirement metric for CSI prediction - companies are invited to bring contribution on how the requirement could be defined - what is the baseline that throughput would be compared to

For UE-sided CSI prediction, the result of CSI prediction is reported to gNB. In this case, throughput could be taken as a testing metric following RAN4 legacy ‘follow PMI’ test procedure.

Proposal 1: The baseline of the requirement in CSI prediction is the throughput that TE uses a random PMI.

R19 WID on AIML [2]: - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

However, since the performance is related to the match degree between training dataset and testing dataset, how to ensure that the testing dataset aligns well with training dataset is still an open issue. We noticed that in the revised WID [2], other WGs will start to study the necessity of specifying consistency of training/inference from 2025 Q1, RAN4 can wait RAN1 progress for defining the test configurations.

Observation 1: How to ensure that the testing dataset aligns well with training dataset is still an open issue.

Proposal 2: RAN4 will start to discuss the test configuration after other WGs achieve sufficient progress.

# Conclusions

Proposal 1: The baseline of the requirement in CSI prediction is the throughput that TE uses a random PMI.

Observation 1: How to ensure that the testing dataset aligns well with training dataset is still an open issue.

Proposal 2: RAN4 will start to discuss the test configuration after other WGs achieve sufficient progress.

# References

- R4-2410570, “WF on AI/ML”, 3GPP RAN4 #112, May 20th – 24th, 2024.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN #105, September 9-12, 2024.