3GPP TSG-RAN WG4 Meeting #113	  R4-2418975

Orlando, US, November 18 – 22, 2024

Agenda Item:	7.17.5.2

Source:	Huawei, HiSilicon

Title:	Discussion on testability and interoperability issues for positioning accuracy enhancement

Document for:	Discussion and Decision

# Introduction

According to WF in RAN4 #112 [1], we continue to discuss the remaining issues related to the test for AIML enabled positioning enhancement.

# Test metrics for positioning accuracy enhancement

TR 38.843 [2]: For metrics for positioning requirements/tests, the candidate options include - Option 1: positioning accuracy: Ground truth vs. reported - only option available for direct positioning - Option 2: CIR/PDP, channel estimation accuracy - Option 3: ToA, RSTD and RSRP, and RSRPP - Option 4: others (e.g., intermediate KPIs, LoS/NLoS)/combinations of the above

In R18, potential test metrics for AI positioning are identified. The applicability of each test metric needs to be discussed per use case.

R19 SID on AIML [3]: - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

In the revised SID [2], we observe that Case 1, 3a and 3b have the 1st priority, while Case 2a and 2b have the 2nd priority. RAN1 will not discuss any proposals targeting Case 2a and 2b in Q4 of 2024.

RAN4 #110 bis WF [4]: - Requirements for case 1: - postpone discussion until reporting scheme (if defined) is clear. if reporting scheme is introduced, RAN4 will further discuss whether to define requirements or not. - RAN4 will not define any accuracy requirements if no reporting scheme is introduced.

For Case 1, it is agreed in [4] that RAN4 will not define any accuracy requirements if no reporting scheme is introduced. Since UE location is not expected to report to NW, there is no need to define the requirement of positioning accuracy in RAN4.

Proposal 1: For UE-sided direct AI/ML positioning (Case 1), there is no need to define the requirement of positioning accuracy.

RAN4 #112 WF [1]: - Requirements for case 2a: - RAN4 should not define any positioning accuracy requirements because positioning is LMF based. - Reported metrics for case 2a: - RAN4 will take existing requirements for existing metrics as baseline - FFS whether to revisit these requirements - RAN4 will study how to define requirement for any new reported metrics - FFS how to define requirements for LOS/NLOS - Requirements for case 2b: - postpone discussion on what and how to define requirements until RAN1 agrees to what metrics are reported. - Requirements for case 3a/3b: - postpone discussion until RAN1 agrees on any metric to be reported by gNBs.

For Case 2a, the reporting can be the LoS/NLoS indicator or ToA.

- Requirement for Los/NLoS indicator

In legacy, we have no requirement on LoS/NLoS reporting. If AI based LoS/NLoS is introduced, whether and how to differentiate if from non-AI based LoS/NLoS is still under discussion in other WGs. If AI-based LoS/NLoS indicator can not be differentiated from non-AI based LoS/NLoS indicator from spec perspective, RAN4 will not introduce requirement specific for AI.

- Requirement for timing information

There is no sufficient agreement related to AI-based timing information for Case 2a in RAN1. If AI based timing information is introduced, RAN4 needs to discuss what would be the baseline. Since in legacy, we have no requirement of timing information if it is a NLoS path. Other aspect is that how to obtain the ground truth, i.e., the ideal timing information of the virtual LoS path is still an open issue.

Proposal 2: For UE-assisted/LMF-based positioning with UE-side model (Case 2a), RAN4 will not define requirement of AI-based LoS/NLoS indicator.

Proposal 3: For UE-assisted/LMF-based positioning with UE-side model (Case 2a), RAN4 will study the testability of AI-based timing information if introduced by other WGs.

Besides, for Case 2a, the following was agreed in last meeting.

Issue 3-3: Reported metrics for case 2a Agreement: - RAN4 will take existing requirements for existing metrics as baseline - FFS whether to revisit these requirements - RAN4 will study how to define requirement for any new reported metrics - FFS how to define requirements for LOS/NLOS

We suggest RAN4 to clarify what “existing metrics” refers to in the first bullet. In our view, timing or power information for NLOS channel as discussed above should not be considered as “existing metrics”, even they are in the same format of RSTD, UE Rx-Tx or PRS-RSRP. Timing or power information for LOS channel can be considered as “existing metrics”, but we understand measurement prediction, like spatial or temporal prediction as discussed for AL/ML based BM or mobility, is not in scope of AL/ML based positioning. Therefore, it is unclear which exact metrics are considered as “existing metrics” and should meet the existing requirements.

Proposal 4: For UE-assisted/LMF-based positioning with UE-side model (Case 2a), RAN4 to clarify which exact metrics are considered as “existing metrics” and should meet the existing requirements.

For Case 2b, the reporting can be existing metrics such as RSTD, UE Rx-Tx or PRS-RSRP to LMF. In our view, the existing requirements should apply for these metrics because UE measurement behavior is same as assumed in existing requirements.

Proposal 5: For UE-assisted/LMF-based positioning with LMF-side model (Case 2b), existing requirements apply for existing metrics as baseline.

For Case 2b, the reporting can be PDP or CIR. We observed that both sample-based and path-based positioning methods are both on the table. Since there is no agreement on the content and the format of reporting, RAN4 will continue to postpone the discussion of requirement in Case 2b.

Proposal 6: For UE-assisted/LMF-based positioning with LMF-side model (Case 2b), RAN4 will will continue to postpone the discussion of requirement in Case 2b.

For Case 3a/3b, we can follow RAN4 legacy that RAN4 will not define requirement of gNB reporting to LMF.

Proposal 7: For NG-RAN node assisted positioning with gNB-side model (Case 3a) and NG-RAN node assisted positioning with LMF-side model (Case 3b), RAN4 will not define requirement of gNB reporting to LMF.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: For UE-sided direct AI/ML positioning (Case 1), there is no need to define the requirement of positioning accuracy.

Proposal 2: For UE-assisted/LMF-based positioning with UE-side model (Case 2a), RAN4 will not define requirement of AI-based LoS/NLoS indicator.

Proposal 3: For UE-assisted/LMF-based positioning with UE-side model (Case 2a), RAN4 will study the testability of AI-based timing information if introduced by other WGs.

Proposal 4: For UE-assisted/LMF-based positioning with UE-side model (Case 2a), RAN4 to clarify which exact metrics are considered as “existing metrics” and should meet the existing requirements.

Proposal 5: For UE-assisted/LMF-based positioning with LMF-side model (Case 2b), existing requirements apply for existing metrics as baseline.

Proposal 6: For UE-assisted/LMF-based positioning with LMF-side model (Case 2b), RAN4 will will continue to postpone the discussion of requirement in Case 2b.

Proposal 7: For NG-RAN node assisted positioning with gNB-side model (Case 3a) and NG-RAN node assisted positioning with LMF-side model (Case 3b), RAN4 will not define requirement of gNB reporting to LMF.

# References

- R4-2414323, “WF on AI/ML”, 3GPP RAN4 #112, 19th – 23rd August, 2024.

- RP-233133, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, Release 18, 3GPP TR 38.843, V2.0.0, December 11-15, 2023.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN #105, September 9-12, 2024.

- R4-2406617, “WF on AI/ML”, 3GPP RAN4 #110bis, 15th – 19th April, 2024.