3GPP TSG RAN WG4 Meeting #113		R4-2418980

Orlando, Nov.18th – 22nd, 2024

Source: 	Nokia

Title: 	UE RF requirements (MIMO Phase5)

Agenda item:       7.19.2

Document for:     Discussion

# 1	Introduction

Based on the WF on UE RF requirements for MIMO enhancement from RAN4 #112bis meeting [1]. In this paper, we’d like to share our views for 3Tx:

RAN4 WF from #112bis meeting: # <Topic 1> 3Tx ## <Sub-topic 1-1> Power class <Agreement>: PC for 3Tx requirement - PC for 3Tx requirement - PC3 - PC2 - FFS on PC1.5 - FFS on UE type, i.e., FWA and/or handheld UE - FFS on support of full power mode 0 - RAN4 understanding is that TxD for 3Tx is not in the scope of this WI - Full power mode 0 apply to PC2 and PC3 only. - FFS on for which UE types it should apply. <Way forward>: FFS on New capability for power class per band per declared UL power mode for 3Tx ## <Sub-topic 1-2> PA configuration <Agreement>: Assumption for power splitting between antenna ports - Equal power split among non-zero ports according to RAN1 spec <Agreement>: PA configurations - For MPR evaluation, the assumptions of PA architecture are: - For PC3, 3x23dBm - For PC2 - 3x23dBm - 3x26dBm is not precluded and interested company can provide the evaluation ## <Sub-topic 1-3> MPR <Agreement>: Different MPR for different UE type - Define separate MPR requirements for handheld and FWA UE <Way forward>: MPR derivation method for 3Tx - Companies are encouraged to evaluate MPR based on existing MPR requirements, e.g., PC, 2/4Tx, UE type, considering 3PA configuration agreed above - Option 1: Reuse legacy MPR requirements for each power class (OPPO, Huawei, Nokia) - Option 2: Further evaluate with 3PA configuration (Apple) ## <Sub-topic 1-4> Supported layer for mode 0 <Way forward >: FFS on Supported MIMO layer for ULFPTx mode 0 # <Topic 2> Asymmetric TRPs ## <Sub-topic 2-1> UE RF impact <Agreement>: Whether to assume different TCI state between DL and UL - Assume different TCI state between DL and UL <Way forward>: Whether to study FR2 enhancements for asymmetric TRPs scenario in Rel-19 - RAN4 will not study FR2 enhancements until RAN1 considers the scenario in Option 1 - Option 1: Yes, UL-specific TRPs are assumed to have limited DL capability - Option 2: No, need more check - Option 3: Check with RAN1 sending an LS

# 2	Discussion

2.1	Discussion for 3Tx

In Release 17, HPUE for NR FR1 FDD band n77 and n78 for power class 1.5 has been introduced, which covered smartphones and fixed wireless access (FWA) devices. However, 3Tx PC1.5 handheld UE may face power/battery issue support 3-layer ULFPTx mode 0. Therefore, it is reasonable to discuss the capabilities of PC1.5 FWA and handheld/smartphone UEs of how to support 3Tx, for example, for PC1.5 FWA devices, up to 3-layer ULFPTx mode 0 can be supported and for PC1.5 handheld UE, up to 2-layer ULFPTx mode 0 can be supported.

Observation 1: PC1.5 handheld UE may face power/battery issue support 3-layer ULFPTx mode 0.

Proposal 1: For PC1.5 FWA devices, up to 3-layer ULFPTx mode 0 can be supported and for PC1.5 handheld UE, up to 2-layer ULFPTx mode 0 can be supported.

There were no 3Tx UE devices defined in the previous 3GPP releases. The studies and discussion for 3Tx are newly introduced and studied in Rel-19, and the corresponding mechanism how 3Tx should work has been developed in RAN1, which including the codebook up to 3 layers to support 3Tx mode 0. Therefore, in order to let network to configure UE to use 3Tx and work properly, the new UE capability of 3Tx should also be introduced. The details of 3Tx capabilities can be further discussed based on the progress of this WI.

Proposal 2: New UE capability to indicate UE support 3Tx should be introduced. The details of how to define the UE capabilities can be further discussed based on the development of the WI.

Based on R4-2412135 from Samsung [2], the existing MPR gap between 2 Tx and 4Tx for PC1.5 is up to 1.5 dB gap for FWA devices as an example, it is possible to define new MPR requirement for 3Tx if necessary. The first step can be that to reuse existing MPR tables for 2Tx/4Tx as much as possible. Then, if it is needed, the new MPR can be introduced for 3Tx in specific scenarios.

Observation 2: The MPR gap can be observed based on the current MPR tables defined for 2Tx and 4Tx for different power class UEs.

Proposal 3: The first step can be that to reuse existing MPR tables for 2Tx/4Tx as much as possible. Then, if it is needed, the new MPR can be introduced for 3Tx in specific scenarios.

# 3	Conclusions

In our contribution, related uplink 3Tx topics are discussed. Here is the list of our proposals:

Observation 1: PC1.5 handheld UE may face power/battery issue support 3-layer ULFPTx mode 0.

Proposal 1: For PC1.5 FWA devices, up to 3-layer ULFPTx mode 0 can be supported and for PC1.5 handheld UE, up to 2-layer ULFPTx mode 0 can be supported.

Proposal 2: New UE capability to indicate UE support 3Tx should be introduced. The details of how to define the UE capabilities can be further discussed based on the development of the WI.

Observation 2: The MPR gap can be observed based on the current MPR tables defined for 2Tx and 4Tx for different power class UEs.

Proposal 3: The first step can be that to reuse existing MPR tables for 2Tx/4Tx as much as possible. Then, in the further study, if it is needed, the new MPR can be introduced for 3Tx in specific scenarios.

# References

- R4-2417116, WF on UE RF requirements for MIMO enhancement

- R4-2412135, Views on UE RF impact of NR MIMO Phase 5, Samsung.