3GPP TSG-RAN WG4 Meeting #113	R4-2418983

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	Other RX requirements for LP-WUR

Agenda item:	7.23.2.3

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following agreements were made

Issue 3-2-2: Maximum input level requirements Agreements: - The same maximum input power of [-25 dBm] could be starting point for LP-WUR. - FFS whether this value should be scaled-down based on the number of LP-WUS RBs. Issue 3-2-3: Reference channel for LP-WUR requirements Agreements: - RAN4 should define reference measurement channel for LP-WUR RF requirements. - FFS details.

Discussion

Spurious emission

Spurious emissions are the narrow band CW emissions appearing at the UE antenna connector. Receiver spurious emissions mainly come from the LO in the receiver. As illustrated in Figure 1 marked with the green line can the LO leakage end at the antenna or antenna connector. The main LO frequency will  dominate inband spurious emissions and the harmonic will dominate outband spurious emissions. In a design with potential low isolation from LO to RF input an LNA will be required to attenuate the inband spurious and a band pass filter to ensure outband spurious is suppressed. Given that the current requirements are regulatory in nature, they should apply to the LP-WUR.

Reuse the spurious emission requirements from TS 38.101-1 for LP-WUR.

Figure 1Ilustration of a DC receiver.

Maximum input level

LP-WUR and main receiver (MR) will observe the same radio environment. Thus, for LP-WUR to reliably operate and co-exist with legacy NR signals, it should be able to handle large input signals.

The value of maximum input level depends on the maximum gNB output power and the minimum coupling loss (MCL) between the gNB and the UE. The maximum gNB output power has not changed and is same for LP-WUS and NR signal. Further, there is no change on the assumptions regarding MCL. Thus, the value of maximum input level for MR can be used for LP-WUR.

There hasn’t been any change in assumptions regarding MCL and gNB output power.

For the main receiver, the requirement power level varies based on channel bandwidth and modulation type. Further, it is a single value of -25 dBm for channel bandwidths of 3, 5, 10, 15, and 20 MHz, for 64 QAM modulation. There is no impact of SCS on the requirement. As possible bandwidths being considered for LP-WUS in FR1 are ≤5MHz, a single value is sufficient and there is no need to scale the current value.

Possible bandwidths being considered for LP-WUS in FR1 are ≤5MHz.

SCS has no impact on the requirement.

No need to scale-down the requirement based on LP-WUS RBs.

Use maximum input level of -25 dBm for LP-WUR.

Blocking requirements

Blocking characteristics are measure of the receiver ability to receive the wanted signal in presence of interference on frequencies other than its designated frequencies. Given LP-WUR will observe the same radio environment as MR, it should be able to withstand interferer with similar level as of MR. Thus, the interferer power levels, and configuration should be the same as of MR.

Reuse Table 7.6.2-2, 7.6.2-4, 7.6.3-2, 7.6.3-4 for in-band and out-of-band blocking levels for LP-WUR.

It is known that dynamic range of the LP-WUR will be smaller than the dynamic range of main radio. Thus, it is possible to relax the desired signal level in blocking test cases.

Agree to use larger signal power in blocking test cases compared to MR blocking test cases.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Reuse the spurious emission requirements from TS 38.101-1 for LP-WUR.

Observation 1: There hasn’t been any change in assumptions regarding MCL and gNB output power.

Observation 2: Possible bandwidths being considered for LP-WUS in FR1 are ≤5MHz.

Observation 3: SCS has no impact on the requirement.

Proposal 2: No need to scale-down the requirement based on LP-WUS RBs.

Proposal 3: Use maximum input level of -25 dBm for LP-WUR.

Proposal 4: Reuse Table 7.6.2-2, 7.6.2-4, 7.6.3-2, 7.6.3-4 for in-band and out-of-band blocking levels for LP-WUR.

Proposal 5: Agree to use larger signal power in blocking test cases compared to MR blocking test cases.

- References

- R4-2417112, WF on UE RF requirements for LP-WUS, Vivo, RAN4#112bis, Hefei China, 14th – 28th Oct, 2024