3GPP TSG-RAN WG4 Meeting #113	R4-2418987

Orlando, FL, November 18th – 22nd 2024

Agenda Item:	7.17.3.1

Source:	Ericsson

Title:	Discussion on CSI prediction

Document for:	Discussion

# 1	Introduction

During RAN#105, normative work on CSI prediction was agreed as a part of AI/ML for air interface WI [1], and accordingly RAN4#112bis started the discussion on CSI prediction.

Although the objective calls the CSI prediction, RAN1 only discusses the PMI prediction by reusing the Rel-18 Enhanced Type II codebook for predicted PMI. In this contribution, we also discuss the PMI prediction based on AI/ML.

# 2. Performance requirement metric

In RAN4#112bis, the following WF was agreed [2]:

Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy

When defining the performance requirement metric, it is important to bear in mind that the basic purpose of the requirement is to enable interoperability by means of ensuring that UEs present consistent behavior and performance. We tend to agree that the AI-based CSI (PMI) reporting performance should not be worse than the non-AI-based PMI reporting performance, but we think the purpose of the performance requirement discussed in RAN4 is not to establish a “gain” compared to the legacy non-AI-based PMI reporting. The reason is because RAN1 and RAN have already identified that gain is achieved, and the purpose of RAN4 requirements is to establish consistent behavior.

The goal with the RAN4 requirement is not to demonstrate AI performance relative to performance with non-AI (“show a gain”), but it is to establish a minimum performance level for the AI/ML-based CSI (PMI) prediction.

RAN4#112 has already agreed that the metric will be based on relative throughput [3], but the baseline for the relative throughput (i.e., denominator of throughput ratio) was not discussed in RAN4#112bis.

Issue 4-5: CSI prediction accuracy metric Agreement: Use relative throughput as requirement metric for CSI prediction - companies are invited to bring contribution on how the requirement could be defined: - what is the baseline that throughput would be compared to

One option for the baseline is Rel-16 eType II codebook. However, it may not be the case because the supporting of Rel-16 eTypeII codebook is optional capability. Furthermore, there is no need to use an eTypeII codebook since the purpose is not to establish a “gain” from AI. Another option is Rel-18 eTypeII-Doppler codebook, because RAN1 assumes the AI-based CSI prediction uses this codebook. However it is not clear yet how the UE switches the reporting algorithm between Rel-19 (AI-based) and Rel-18 (non-AI-based), we may need to wait for RAN1 conclusion on monitoring. In our understanding, defining the throughput relative to random PMI is sufficient and satisfactory to ensure proper performance with the predicted CSI.

For the performance metric, use throughput with AI predicted CSI (PMI) relative to random PMI with Rel-15 Type I Single-Panel Codebook.

# 3. Range of testing conditions

The existing non-AI framework has requirements defined for a couple of specific TDL channel models and SNR levels. For example, RAN4 usually use the low-dispersion and low-Doppler spread TDL channel models such as TDL-A with delay spread of 30ns and Doppler spread of 5Hz or 10Hz for FR1 PMI reporting tests.

For AI, on the other hand, there is a need to understand whether a model that is trained and can work well in one specific channel and SNR can also work consistently in other scenarios (generality). The scenario in which the PMI prediction operates may change for any of the following reasons:

- Channel variation: depending on the location the multipath profile of the channel may look different.

- SNR level (SNR test point): depending on the location, the SNR may differ.

- SNR changes: due to the activity in neighbor cells, the SNR may shift rapidly with time, in particular for cell-edge UEs.

- Doppler variation: the UE Doppler may differ depending on the location.

- Differences in the antenna array design for the BS serving the UE.

Examining each of these in detail as follows:

Channel variation: A study is needed whether the prediction performance for an AL prediction model varies significantly depending on the multi-path channel profile. If this is the case, then requirements are needed for a variety of multi-path profiles. Moreover RAN4 may need to study the AI model(s) trained with CDL-based dataset work well for TDL-based test setup, and vice versa.

SNR level: A study is needed whether the prediction performance is consistent across the SNR levels. If not, the requirements are needed for several SNR test points. For example, the exiting NR PMI reporting tests set the test point to one SNR point achieving 90% of the peak rate with fixed MCS. We may need to consider lower SNR test points, e.g., 90% of the peak rate with lower MCS such as QPSK/16QAM.

SNR changes: If the prediction performance does not vary with SNR then no consideration is needed. However, if the performance does depend on SNR, then the requirement may need to also specify performance when SNR changes significantly due to e.g. varying neighbor cell scheduling. A realistic model for the SNR change profile would be needed. Note although we have no such requirements in NR and LTE, 3G HSPA CQI reporting test verifies the reported CQI with varying SNR scenario (See TS 25.101 9.3.1.3).

Doppler variation: A study is needed on the sensitivity of the prediction to Doppler. Requirements may be needed for several levels of Doppler spread. In our understanding, RAN1 study the CSI prediction with 30km/h and 60km/h. However RAN4 may need to consider more scenarios such as higher/lower UE mobility such as 120km/h (high speed vehicles) or 3km/h (pedestrian) when defining the requirements.

BS antenna design: RAN1 is now evaluating whether the NW-side conditions such as antenna tilt configuration or antenna array design impacts on the AI-based CSI prediction.

Study the variation of performance of CSI prediction with different channel model, SNR level, changes in SNR and Doppler.

Depending on the results of the study, define sufficient requirement variations that proper model training is achieved, i.e., verify a model/functionality can perform well across variations in cell conditions.

Consider whether to make conformance to CSI prediction with lower/higher Doppler scenario, according to RAN1 study.

# 4. “Monitoring” considerations

“Monitoring” is a term used to refer to the action on either the UE or network side of determining whether the model, in current operation can continue to meet performance expectations. It is important to consider that monitoring should not just indicate whether the model is functioning or not in a binary sense. If the predicted CSI is obviously trash and the performance is poor, then it is obvious that a switch back to non-AI functionality (fallback) is needed as illustrated in Figure 1. The challenge with monitoring is to detect when the performance has degraded to a non-negligible, but not catastrophic extent (e.g. 5-10% degradation).

It is important that monitoring can observe and quantify partial reduction in performance (not just a large or catastrophic loss).

Figure 1	Monitoring of AI/ML-based CSI prediction model.

Such wrong predictions could lead to unacceptable degradations in network performance. However, identifying that degradations have come from a specific AI model may be difficult to achieve. What may also be difficult is to identify whether changing to non-AI may actually improve performance or not. The reason is that UE performance (i.e., data throughput) depends not only on the reported CSI, but on a very large range of factors, including but not limited to e.g. the radio environment, interference conditions, RF performance, adjacent channel interferers, network traffic and scheduling and potentially interaction between a variety of AI algorithms in future UEs.

It is non-trivial to ensure that a monitoring function can determine the performance of an AI model in isolation, since in the real world a very large number of interacting factors influence performance.

Monitoring of CSI prediction is simpler than 2-sided monitoring since a single model needs to be monitored. Nevertheless, RAN4 may need some attention. We expect RAN1 will specify the performance monitoring mechanism for CSI prediction, but RAN1 start the discussion from 2025 February according to their work plan.

From a RAN4 perspective, though, the following considerations are important although we need to wait for the RAN1 conclusion. If monitoring is on the UE side, that is, the UE sends measurements, or monitoring reports to the network (e.g., NMSE or SGCS), then it is important that the measurements or reports are consistent and comparable between different UE types like other CSI quantities such L1-RSRP/L1-SINR as defined in TS 38.215. For measurements, this may necessitate setting requirements on e.g. accuracy and reporting delay in a similar style to RAN4 RRM measurement requirements. For UE monitoring reports, it is important that different UEs provide similar reports when their models have a similar level of reliability. However, since the reliability of different UEs models may differ, a discussion is needed on how requirement definition and testing could be carried out in a way that forces UEs to specific reliability levels and then tests the output. Also, a definition of metrics for testing of the output is needed.

Monitoring requirements should define the constancy/accuracy of monitoring reports from the UE if they are defined.

Testability of monitoring report accuracy needs discussion, since the reliability of underlying models will differ between UEs and a test of accuracy needs to force UEs into known reliability levels.

If monitoring is entirely on the network side, then it may be discussed in RAN1. Since the network needs to manage a large amount of UEs, the complexity of monitoring, ability to diagnose specific models within the UE need improvement and also identify what improvement is needed may be challenges.

In particular, to perform network sided monitoring, the UE may be required to report two CSI values: one for predicted CSI value (CSI value for time t+δ, predicted in report time t), and another one for the measured CSI value (CSI value based on the CSI-RS measurement at time t+δ). The measured CSI would be considered as ‘ground truth’, and the NW may judge the prediction performance by comparing two CSI values. It may be also debated whether it is useful to set requirements to ensure that both the reported CSI measured at t+δ for time t+δ and the CSI predicted at t for time t+δ meet performance requirements when the UE is operating in this mode.

Figure 2	Example of network-sided monitoring.

For network sided monitoring, if the UE reports both predicted and measured CSI, there may be some need to consider setting conformance requirements on the performance of both measured and predicted when operating in this monitoring mode.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The goal with the RAN4 requirement is not to demonstrate AI performance relative to performance with non-AI (“show a gain”), but it is to establish a minimum performance level for the AI/ML-based CSI (PMI) prediction.

Observation 2	It is important that monitoring can observe and quantify partial reduction in performance (not just a large or catastrophic loss).

Observation 3	It is non-trivial to ensure that a monitoring function can determine the performance of an AI model in isolation, since in the real world a very large number of interacting factors influence performance.

Observation 4	Monitoring requirements should define the constancy/accuracy of monitoring reports from the UE if they are defined.

Observation 5	Testability of monitoring report accuracy needs discussion, since the reliability of underlying models will differ between UEs and a test of accuracy needs to force UEs into known reliability levels.

Observation 6	For network sided monitoring, if the UE reports both predicted and measured CSI, there may be some need to consider setting conformance requirements on the performance of both measured and predicted when operating in this monitoring mode.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For the performance metric, use throughput with AI predicted CSI (PMI) relative to random PMI with Rel-15 Type I Single-Panel Codebook.

Proposal 2	Study the variation of performance of CSI prediction with different channel model, SNR level, changes in SNR and Doppler.

Proposal 3	Depending on the results of the study, define sufficient requirement variations that proper model training is achieved, i.e., verify a model/functionality can perform well across variations in cell conditions.

Proposal 4	Consider whether to make conformance to CSI prediction with lower/higher Doppler scenario, according to RAN1 study.

# References

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm.

- R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm Incorporated.

- R4-2414323, “WF on AI/ML”, Qualcomm Incorporated.