3GPP TSG-RAN WG4 Meeting #113	R4-2418992

Orlando, Florida, USA, 18th to 22nd  November 2024

Agenda Item:	7.2.4.1

Source:	Ericsson

Title:	On 15 GHz co-existence simulation assumptions and results discussion

Document for:	Discussion

# 1	Introduction

ITU-R WP5D sent a LS [1] to 3GPP RAN seeking information on terrestrial component IMT-2030 parameters to be used for sharing and compatibility studies in preparation for WRC-27, for frequency ranges 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 15350 MHz.

In the 3GPP RAN4#110-bis meeting, the Work Plan [2] for delivery of information to ITU-R LS for those respective frequency ranges was agreed, following a phase approach to send information in 3 phases as below -

- Track 1: 4400 to 4800 MHz, the estimated date for completion is latest by May 2024 (RAN4#111).

- Track 2: 7125 to 8400 MHz, the estimated date for completion is latest by August 2024 (RAN4#112).

- Track 3: 14800 to 15350 MHz, the estimated date for completion is latest by November 2024 (RAN4#113).

WF [3] [4] and TR 38.922 v0.3.0 [5] captured initial agreements on co-existence simulation assumptions for this frequency range.

This contribution provides further discussion points on co-existence simulation assumptions and BS antenna configuration relevant for 14800 to 15350 MHz frequency range.

# Discussion

In the previous 3GPP RAN4#112bis meeting, a couple of agreements were made on the co-existence simulation assumptions for Urban scenarios which are captured in the TR 38.922 v0.3.0 [5]. However, some issues remain open and are discussed in the following sub-sections.

#### 2.1	Channel Bandwidth

In our companion contribution [7], we have presented results assuming both 100 MHz and 200 MHz. It can be observed that the ACIR requirements are more or less in similar ballpark for the two different bandwidth values. At the same time, the maximum BS and UE transmission powers (46 dBm and 23 dBm, respectively) have been defined independently of the channel bandwidth, and the simulation assumption considers that the total power is transmitted over the entire bandwidth. In a real deployment it can be considered however that multiple users can be scheduled over the wide bandwidth and the bandwidth allocated to a UE can be adapted to improve its coverage.

ACIR requirements are in similar ballpark when considering 100 or 200 MHz, both in Uplink and Downlink.

UL coverage could be challenging and multiple UEs can be scheduled to address coverage issues.

It is important to communicate to ITU-R some combinations of power/bandwidth which are feasible in terms of coverage and propose them as typical values in this band. Otherwise, if power and bandwidth remain untied, this leads to unclear Power Spectral Density (PSD), which plays a significant role in coexistence simulations, especially when there are coverage issues.

The additional information in Channel Bandwidth should be updated with the following text in the TR – “Higher channel bandwidths are not precluded. The typical channel bandwidth is associated with the maximum transmission power as it impacts the Power Spectral Density (PSD), when full channel bandwidth is transmitted.”

Following a de-facto approach, to enable wider channel bandwidth with increasing frequency, it is proposed to adopt 200 MHz as typical channel bandwidth, with higher channel bandwidths not precluded.

RAN4 to adopt 200 MHz as typical channel bandwidth, and also state in TR that higher channel bandwidths are not precluded, and that in case of coverage issues, more UEs can be considered to be scheduled on the wide bandwidth.

#### 2.2	Indoor UE ratio

In our companion contribution [7], we have presented results assuming both options of indoor probability 0% and 20%. Indoor probability 20% brings more challenges in terms of UL coverage and a slightly lower UL ACIR requirement. Indoor users are the driving force that decides the requirements in UL.

Indoor users are the driving force that decide the ACIR requirements in UL.

As highlighted in our companion contribution [7] - with the assumption of 0% Indoor Users for similar options in BS array size and CBW, the coverage gets better and it results in a higher ACIR requirements which can also be considered as the worst-case.

RAN4 to keep both options 20% and 0% Indoor users for co-existence simulations, and ACIR requirements can be derived considering 0% Indoor Users as the worst-case.

#### 2.3	BS & UE ACLR/ ACS

In TR 38.921 [6], the agreed values for ACLR/ ACS for 10 GHz have been specified in Table 2.3-1 below. The values were based on ACIR requirements of 30 dB in DL and 23.9 dB in UL, as per the co-existence simulation results.

Table 2.3-1: ACLR and ACS values for 10-10.5 GHz [6]

| (dB) | ACLR | ACS |
| --- | --- | --- |
| BS | 37 | 40 |
| UE | 24 | 31 |

Based on the simulation results presented in our companion contribution [7], the ACIR requirements in DL and UL are 24 dB and 14 dB respectively, when considering BS array of 3072 AEs and channel bandwidth of 200 MHz and 0% indoor users as the worst-case scenario.

Based on co-existence simulation results of 24 dB ACIR in DL and assuming 25 dB UE ACS into the formula would correspond to BS ACLR of 31 dB.

Similarly, with 14 dB ACIR in UL and assuming 15 dB UE ACLR into the above formula would result in BS ACS of 21 dB.

The proposed values for BS-UE ACLR/ ACS for 14800 – 15350 MHz are summarized in below table”

Table 2.3-2: ACLR and ACS values for 14800 – 15350 MHz

| (dB) | ACLR | ACS |
| --- | --- | --- |
| BS | 31 | 21 |
| UE | 15 | 25 |

RAN4 to consider BS ACLR as 31 dB and BS ACS as 21 dB as a starting point for discussions.

RAN4 to consider UE ACLR as 15 dB and UE ACS as 25 dB as a starting point for discussions.

#### 2.4	Non-AAS BS architecture in LS Template

The LS Template has IMT parameters requested for both AAS and non-AAS BS.

With the antenna proposals for larger antenna array and advanced antenna configurations, non-AAS parameters might be unnecessary and therefore non-AAS BS architecture sections can be removed from the template.

RAN4 to remove non-AAS BS architecture sections from the LS template.

# Conclusion

In the previous sections we made the following observations:

Observation 1	ACIR requirements are in similar ballpark when considering 100 or 200 MHz, both in Uplink and Downlink.

Observation 2	UL coverage could be challenging and multiple UEs can be scheduled to address coverage issues.

Observation 3	Indoor users are the driving force that decide the ACIR requirements in UL.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The additional information in Channel Bandwidth should be updated with the following text in the TR – “Higher channel bandwidths are not precluded. The typical channel bandwidth is associated with the maximum transmission power as it impacts the Power Spectral Density (PSD), when full channel bandwidth is transmitted.”

Proposal 2	RAN4 to adopt 200 MHz as typical channel bandwidth, and also state in TR that higher channel bandwidths are not precluded, and that in case of coverage issues, more UEs can be considered to be scheduled on the wide bandwidth.

Proposal 3	RAN4 to keep both options 20% and 0% Indoor users for co-existence simulations, and ACIR requirements should be derived considering 0% Indoor Users as the worst-case.

Proposal 4	RAN4 to consider BS ACLR as 31 dB and BS ACS as 21 dB as a starting point for discussions.

Proposal 5	RAN4 to consider UE ACLR as 15 dB and UE ACS as 25 dB as a starting point for discussions.

Proposal 6	RAN4 to remove non-AAS BS architecture sections from the LS template.

# References

- R4-2400333, “ITU-R WP5D LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz, and 14.8-15.35 GHz”.

- R4-2405874, “Work Plan on IMT parameters relevant for sharing and compatibility for 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 1530 MHz frequency ranges”, 3GPP RAN4#110-bis, Ericsson

- R4-2417107, “WF on 15 GHz BS parameters”, 3GPP RAN4#112bis, ZTE Corporation

- R4-2417108, “WF on 15 GHz UE parameters”, 3GPP RAN4#112bis, Qualcomm

- R4-2417200, “3GPP TR 38.922 v0.3.0: Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz, and 14800-15350 MHz”, 3GPP RAN4#112bis

- 3GPP TR 38.921 v18.0.1: Study on IMT parameters for 6425 – 7025, 7025 – 7125 and 10000 - 10500 MHz”, 3GPP SA#103

- R4-2418993, “On 15 GHz co-existence simulation results”, 3GPP RAN4#113, Ericsson