3GPP TSG-RAN WG4 Meeting #113							R4-2418993

Orlando, Florida, USA, 18th to 22nd  November 2024

Agenda Item:	7.2.4.1

Source:	Ericsson

Title:	On 15 GHz co-existence simulation results

Document for:	Information

# 1	Introduction

ITU-R WP5D sent a LS [1] to 3GPP RAN seeking information on terrestrial component IMT-2030 parameters to be used for sharing and compatibility studies in preparation for WRC-27, for frequency ranges 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 1530 MHz.

In 3GPP RAN4#110-bis meeting, the Work Plan [2] for delivery of information to ITU-R LS for those respective frequency ranges was agreed, following a phase approach to send information in 3 phases as below -

- Track 1: 4400 to 4800 MHz, completed by May 2024 (RAN4#111).

- Track 2: 7125 to 8400 MHz, completed by August 2024 (RAN4#112).

- Track 3: 14800 to 15350 MHz, the estimated date for completion is latest by November 2024 (RAN4#113).

WF [1] [2] and TR 38.922 v0.3.0 [3] captured initial agreements on co-existence simulation assumptions for companies to present results in October meeting.

This contribution provides the simulation results for Urban Macro as per the latest agreements.

# 2	Discussion

In this section some simulation results for Urban macro are presented, following the captured agreements, and the options listed in the sections below. The simulation assumptions for the Urban Macro scenario have been agreed in WF [1][2] and captured in TR [3].

## 2.1	Urban Macro uncoordinated scenario

Relevant simulation assumptions, after discussions in previous meetings, are the following:

- ISD (Inter-Site Distance) = 450 m

- UE max Transmission power = 23 dBm

- Indoor probability: 20%/ 0%

- Diversity gain: 0 dB

- Number of active UEs is 1

- Channel bandwidth: 100/200 MHz

- UE omnidirectional antenna with 0 dBi gain

- BS antenna size – 3072/ 4096 Antenna Elements (AEs)

In the following sections we discuss simulation results considering the currently available simulation assumptions and options for Indoor probability, Channel bandwidth (CBW) and BS array size.

### 2.1.1	Downlink

Figure 2.1.1-1 and Figure 2.1.1-2 show the DL SINR for indoor probability 20% and 0%, respectively, considering the two different arrays with 3072 and 4096 AE, and for CBW of 100 and 200 MHz. It can be observed that both scenarios are feasible from a coverage perspective, with the bigger array option further improving the coverage compared to the 3072 AE option. When CBW is lower, the PSD is 3 dB higher, due to the simulation assumption that the total transmission power is used over the full bandwidth, and this results in slightly better coverage.

The DL scenario is feasible from a coverage perspective for all the combination of parameters under study – BS array size of 3072/ 4096 AEs, CBW of 100/ 200 MHz and Indoor probability of 20%/ 0%.

Figure 2.1.1-1 DL SINR for BS array size - 3072 and 4096 AEs, bandwidth 100 and 200 MHz, indoor probability 20%

Figure 2.1.1-2 DL SINR for BS array size - 3072 and 4096 AEs, bandwidth 100 and 200 MHz, indoor probability 0%

DL ACIR requirements are determined by the 5%tile throughput loss and are in similar ballpark when considering either of 100 MHz or 200 MHz CBW. For all the cases studied, the DL ACIR requirements are 20-21 dB with Indoor probability 20% and 24 dB with indoor probability 0%. The results have been presented in Table 2.1.1.1-1 with BS array size of 3072 AEs and in Table 2.1.1.2-1 with BS array size of 4096 AEs.

#### 2.1.1.1	Results when BS array size is 3072 antenna elements

Table 2.1.1.1-1 DL ACIR for BS array size - 3072 AEs, bandwidth 200 and 100 MHz, indoor probability 20% and 0%

| BS array size – 3072 AEs | CBW | Indoor UE probability | Absolute ACIR (dB) | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 100 MHz | 20% | Average throughput loss in % | 0.013 | 0.012 | 0.01 | 0.009 | 0.008 |  |  |
|  |  |  | 5%-tile throughput loss in % | 6.92 | 6.23 | 5.43 | 5.1 | 4.7 |  |  |
|  |  | 0% | Average throughput loss in % | 0.015 | 0.014 | 0.0125 | 0.011 | 0.01 | 0.009 | 0.008 |
|  |  |  | 5%-tile throughput loss in % | 12 | 10 | 9 | 8.4 | 7.8 | 6.6 | 6 |
|  | 200 MHz | 20% | Average throughput loss in % | 0.0134 | 0.0121 | 0.01 | 0.009 | 0.008 |  |  |
|  |  |  | 5%-tile throughput loss in % | 8 | 7 | 6.24 | 5.25 | 4.6 |  |  |
|  |  | 0% | Average throughput loss in % | 0.015 | 0.014 | 0.0125 | 0.01 | 0.01 | 0.009 | 0.008 |
|  |  |  | 5%-tile throughput loss in % | 11 | 10 | 8 | 7.5 | 7.2 | 6.7 | 5.5 |

#### 2.1.1.2	Results when BS array size is 4096 antenna elements

Table 2.1.1.2-1 DL ACIR for BS array size – 4096 AEs, bandwidth 200 and 100 MHz, indoor probability 20% and 0%

| BS array size – 4096 AEs | CBW | Indoor UE probability | Absolute ACIR (dB) | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 100 MHz | 20% | Average throughput loss in % | 0.011 | 0.01 | 0.008 | 0.007 | 0.006 |  |  |
|  |  |  | 5%-tile throughput loss in % | 5.7 | 5.4 | 5.3 | 4.4 | 4 |  |  |
|  |  | 0% | Average throughput loss in % | 0.013 | 0.011 | 0.01 | 0.009 | 0.008 | 0.007 | 0.006 |
|  |  |  | 5%-tile throughput loss in % | 11.5 | 10 | 9 | 8.2 | 7.1 | 6.6 | 5.7 |
|  | 200 MHz | 20% | Average throughput loss in % | 0.011 | 0.01 | 0.0088 | 0.008 | 0.006 |  |  |
|  |  |  | 5%-tile throughput loss in % | 6 | 5.4 | 5 | 4.4 | 3.66 |  |  |
|  |  | 0% | Average throughput loss in % | 0.0125 | 0.01 | 0.009 | 0.008 | 0.0075 | 0.007 | 0.006 |
|  |  |  | 5%-tile throughput loss in % | 10 | 9 | 8 | 7.4 | 6.3 | 5.9 | 5.1 |

DL ACIR requirements are 20-21 dB with Indoor probability 20% and 24 dB with indoor probability 0%.

DL ACIR requirements are on the lower side when 20% Indoor probability is considered with larger antenna size.

### 2.1.2	Uplink

Figure 2.1.2-1 and Figure 2.1.2-2 show the UL SINR for indoor probability 20% and 0%, respectively, considering the two different BS arrays with 3072 and 4096 AEs, and CBW of 100 and 200 MHz. It can be observed that both scenarios are feasible from a coverage perspective, with the bigger array option further improving the coverage compared to the 3072 AE option. When CBW is lower, the PSD is 3 dB higher, due to the simulation assumption that the total transmission power is used over the full bandwidth, and this results in slightly better coverage. Depending on the assumptions considered for the UE or BS architecture, and the CBW, the percentage of users in outage oscillates around 4%. When UL coverage conditions are challenging it should be possible in a real deployment to adapt the bandwidth and concentrate the UL transmission power in a fraction of the bandwidth and schedule more users in the remaining bandwidth.

The UL scenario is feasible from a coverage perspective for all the combination of parameters under study – BS array size of 3072/ 4096 AEs, CBW of 100/ 200 MHz and Indoor probability of 20%/ 0%.

A percentage of users of 4.3% is in outage in UL 3072 AE and 200 MHz CBW and indoor probability 20%. For users in challenging coverage conditions, it should be possible to schedule more users at the same time in the bandwidth, to concentrate the UL transmission power in a fraction of the bandwidth and improve the PSD.

Figure 2.1.2-3 and Figure 2.1.2-4 show the UL transmission power for indoor probability 20% and 0%, respectively, considering the two different BS arrays with 3072 and 4096 AE and CBW of 100 and 200 MHz. It can be observed that when indoor users are present in the scenario, the percentage of users transmitting at maximum power increases and is below 30% for 200 MHz and around 20% for 100 MHz. A bigger array reduces the percentage of users in need to transmit at maximum power.

Figure 2.1.2-1 UL SINR for BS array size - 3072 and 4096 AEs, bandwidth 100 and 200 MHz, indoor probability 20%

Figure 2.1.2-2 UL SINR for BS array size - 3072 and 4096 AEs, bandwidth 100 and 200 MHz, indoor probability 0%

Figure 2.1.2-3 UL power for BS array size - 3072 and 4096 AE, bandwidth 100 and 200 MHz, indoor probability 20%

Figure 2.1.2-4 UL power for BS array size - 3072 and 4096 AE, bandwidth 100 and 200 MHz, indoor probability 0%

UL requirements are determined by the 5%tile throughput loss and are in similar ballpark when considering either of 100 MHz or 200 MHz. For all the cases studied, the UL ACIR requirements are 11-12 dB with indoor probability 20% and 14-15 dB with indoor probability 0%. The results have been presented in Table 2.1.2.1-1 with BS array size of 3072 AEs and in Table 2.1.2.2-1 with BS array size of 4096 AEs.

#### 2.1.2.1	Results when BS array size is 3072 antenna elements

Table 2.1.2.1-1 UL ACIR for BS array size – 3072 AEs, bandwidth 200 and 100 MHz, indoor probability 20% and 0%

| BS array size – 3072 AEs | CBW | Indoor UE probability | Absolute ACIR (dB) | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 100 MHz | 20% | Average throughput loss in % | 0.001 | 0.009 | 0.008 | 0.007 | 0.006 | 0.005 | 0.0043 |
|  |  |  | 5%-tile throughput loss in % | 8.77 | 7.46 | 6.1 | 5.1 | 4.76 | 3.76 | 3.5 |
|  |  | 0% | Average throughput loss in % | 0.02 | 0.018 | 0.016 | 0.014 | 0.012 | 0.011 | 0.01 |
|  |  |  | 5%-tile throughput loss in % | 14.1 | 13 | 12 | 11 | 9 | 8.1 | 7.5 |
|  | 200 MHz | 20% | Average throughput loss in % | 0.0088 | 0.0078 | 0.0068 | 0.006 | 0.0052 | 0.0046 | 0.004 |
|  |  |  | 5%-tile throughput loss in % | 6 | 5.66 | 5 | 4.6 | 4.22 | 3.6 | 3.3 |
|  |  | 0% | Average throughput loss in % | 0.02 | 0.017 | 0.015 | 0.013 | 0.012 | 0.01 | 0.009 |
|  |  |  | 5%-tile throughput loss in % | 12.52 | 11 | 10 | 8.2 | 7.2 | 6 | 5 |

#### 2.1.2.2	Results when BS array size is 4096 antenna elements

Table 2.1.2.2-1 DL ACIR for BS array size – 4096 AEs, bandwidth 200 and 100 MHz, indoor probability 20% and 0%

| BS array size – 4096 AEs | CBW | Indoor UE probability | Absolute ACIR (dB) | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 100 MHz | 20% | Average throughput loss in % | 0.008 | 0.007 | 0.0065 | 0.0055 | 0.005 | 0.0044 | 0.004 |
|  |  |  | 5%-tile throughput loss in % | 7.37 | 7 | 6 | 4.8 | 4 | 3.5 | 3 |
|  |  | 0% | Average throughput loss in % | 0.015 | 0.014 | 0.0125 | 0.011 | 0.009 | 0.008 | 0.007 |
|  |  |  | 5%-tile throughput loss in % | 11.9 | 10.5 | 9.5 | 8 | 7.18 | 6.5 | 6 |
|  | 200 MHz | 20% | Average throughput loss in % | 0.007 | 0.0064 | 0.0056 | 0.005 | 0.0044 | 0.0038 | 0.0035 |
|  |  |  | 5%-tile throughput loss in % | 5.55 | 4.73 | 3.93 | 3.45 | 2.66 | 2.08 | 2 |
|  |  | 0% | Average throughput loss in % | 0.015 | 0.0135 | 0.0125 | 0.011 | 0.009 | 0.007 | 0.006 |
|  |  |  | 5%-tile throughput loss in % | 10.7 | 9 | 8.5 | 7.5 | 6.6 | 6 | 5 |

UL ACIR requirements are in the range 11-12 dB with indoor probability 20% and 14 dB with indoor probability 0%.

UL ACIR requirements are on the lower side when 20% Indoor probability, as the Indoor Users decide the requirements.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The DL scenario is feasible from a coverage perspective for all the combination of parameters under study – BS array size of 3072/ 4096 AEs, CBW of 100/ 200 MHz and Indoor probability of 20%/ 0%.

Observation 2	DL ACIR requirements are 20-21 dB with Indoor probability 20% and 24 dB with indoor probability 0%.

Observation 3	DL ACIR requirements are on the lower side when 20% Indoor probability is considered with larger antenna size.

Observation 4	The UL scenario is feasible from a coverage perspective for all the combination of parameters under study – BS array size of 3072/ 4096 AEs, CBW of 100/ 200 MHz and Indoor probability of 20%/ 0%.

Observation 5	A percentage of users of 4.3% is in outage in UL considering the above-mentioned assumptions and indoor probability 20%. For users in challenging coverage conditions, it should be possible to schedule more users at the same time in the bandwidth, to concentrate the UL transmission power in a fraction of the bandwidth and improve the PSD.

Observation 6	UL ACIR requirements are in the range 11-12 dB with indoor probability 20% and 14 dB with indoor probability 0%.

Observation 7	UL ACIR requirements are on the lower side when 20% Indoor probability, as the Indoor Users decide the requirements.

# References

- R4-2417107, “WF on 15 GHz BS parameters”, 3GPP RAN4#112bis, ZTE Corporation

- R4-2417108, “WF on 15 GHz UE parameters”, 3GPP RAN4#112bis, Qualcomm

- R4-2417200, “3GPP TR 38.922 v0.3.0: Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz, and 14800-15350 MHz”, 3GPP RAN4#112bis