3GPP TSG-RAN WG4 Meeting # 113	R4-2418999

# Orlando, US, 18th – 22th November, 2024

Source:	LG Electronics

Title:	Discussion on Power domain enhancements for single carrier

Agenda item:	7.1.1.2.1

Document for:	Discussion

Introduction

In 3GPP RAN4 #112-bis, WF[1] was approved. In this paper, we provide our views on Power domain enhancements for single carrier.

- Topic #1: Power domain enhancements for single carrier #### Issue 1-1-1: Approaches for scenarios 1-1 and 1-2 (see Reference for description of scenarios) - WF - Strive for a unified solution which can accommodate both scenario 1 and scenario 2 under the framework of converting outer RB allocation to inner RB allocation. - It does not preclude separate solutions for each applicable scenario - No relaxation of ACLR/SEM/SE values Issue 1-2-1: Clarification of multi-CC scenario - WF - The approach of converting outer RB allocation to inner RB allocation for MPR reduction should consider multi-carrier spectrum scenario from NW perspective while the UE is configured for single CC operation. Issue 1-2-2: Approaches of converting outer RB allocation to inner RB allocation - WF - The approach should consider the following aspects - unified solution could be applicable for both scenarios 1 and scenario 2 - unified solution considering the spectrum allocation status of operators, e.g multi-CC case from NW perspective, to better leverage the feature of MPR reduction - Solution could benefit RedCap and non-RedCap with UE CBW less or even identical to BS CBW - Solution could include both symmetric and asymmetric CBW extension - Whether the extended UE CBW could exceed the BS CBW for different scenarios - Signalling overhead is considered for the proposed approach(s) #### Issue 1-2-3: Where to use IBE in the larger BS CBW - WF - IBE should be used between edges of UE CBW and extended UE CBW - FFS whether IBE could be used in the guard band of UE CBW - FFS the full RB allocation scenario - FFS the impact to testability #### Issue 1-2-4: Boundary to apply ACLR and SEM - Proposals - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW. - Proposal 2: ACLR, SEM and spurious emissions would be defined based on BS channel bandwidth. - WF - FFS in next meeting - ITU regulation of 250% necessary bandwidth should be considered in further analysis #### Issue 1-2-5: Boundary to apply SE - Proposals - Proposal 1: The application range of SE should be altered with the shifting of the edge of the UE CBW. - Proposal 2: SE is applied at BS channel bandwidth. - WF - FFS in next meeting - ITU regulation of 250% necessary bandwidth should be considered in further analysis #### Issue 1-2-6: Which CBW is utilized as the basis for the OOBE requirement and the applicable boundary - Proposals - Proposal 1: The OOBE requirement and applicable boundary should be based on UE CBW. - Proposal 2: The OOBE requirement and applicable boundary is based on BS CBW. - WF - FFS in next meeting #### Issue 1-2-7: Ratio of extended CBW to UE CBW and to larger BS channel BW - WF - To discuss the following aspects for the extended UE CBW: - Whether the extended CBW with new boundary of FOOB should comply with the recommendation by ITU-R on necessary bandwidth in terms of SE for the original UE BW - Whether default or fixed extension, e.g. 1/2 UE CBW could be stipulated in the spec or the extension ratio could be a UE capability to fulfil the conversion of outer to inner - Whether the extended UE CBW could belong to the regular defined channel bandwidth - Whether some premise should be established for the extension size, e.g. - minimum extended CBW compared to original UE CBW - minimum BS CBW compared to UE CBW - frequency separation of the RB allocation to the BS edge - Other identified issues are not precluded #### Issue 1-2-8: Asymmetrical extended CBW approach - WF - Asymmetrical extension is considered, FFS the solution. #### Issue 1-2-10: Signaling aspects - WF - To further discuss the signalling aspects in conjunction with the solution to be adopted for MPR reduction. Companies are encouraged to provide thinking on signalling impact together with the proposed solution

Discussion

Approaches of converting outer RB allocation to inner RB allocation

- WF - The approach should consider the following aspects - unified solution could be applicable for both scenarios 1 and scenario 2 - unified solution considering the spectrum allocation status of operators, e.g multi-CC case from NW perspective, to better leverage the feature of MPR reduction - Solution could benefit RedCap and non-RedCap with UE CBW less or even identical to BS CBW - Solution could include both symmetric and asymmetric CBW extension - Whether the extended UE CBW could exceed the BS CBW for different scenarios - Signalling overhead is considered for the proposed approach(s)

In the last RAN4 #112-bis meeting, it was agreed to move to a unified method, converting outer RB allocation to inner RB allocation, and it was agreed that the method can satisfy the above aspects. In the previous discussion, it was discussed under limited conditions (redcap UE, BS CHBE ≥ 2*UE CHBW). However, we need to include a lot of aspects as discussed in the last meeting. We have limited time to consider many aspects.so it need to strive to reuse the existing MPR value without additional simulation.

Proposal 1: Strive to reuse the existing MPR value without additional simulation.

Network indication method can be one of simple method without additional simulation and used to cover outer RB allocation to inner RB allocation while including the above aspects in WF. So far as we have discussed with extended UE CHBW. In this discussion, unified solution converting outer RB allocation to inner RB allocation is coming. Because MPR value in the spec was made under the condiction that there is adjacent allocated channel. At this aspect, if network indicate some information (e.g. extended UE CHBW and RBoffset) to UE. UE can calculate the new MPR value with converting outer RB allocation to inner RB allocation.

Figure1. Extension UE CHBW

Figure2. Operation based on BS indication

If the RB allocation region is divided based on the new NRB_new as above, the MPR value can be calculated with the inner region expanded than the previous one. NRB_new is calculated as follow NRB_new=RBoffset_left+ RBoffset_right . Then, if the network indicates the NRB_newand RBoffset_left values to the UE, the UE can apply the MPR value of the new region based on the expanded UE CHBW with improved values. The above method can reduces the MPR by the number of unallocated RBs next to the UE CHBW including many aspect in WF. The outer RB allocation can be modified to an inner RB allocation if the Network indicate NRB_newand RBoffset_leftto the UE.

We consider network indication when converting outer RB allocation to inner allocation as below

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB_new – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart + RBOffset  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_new/2)

Proposal 2: Consider network indication when converting outer RB allocation to inner allocation as below;

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB_new – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart + RBOffset  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_new/2)

Boundary to apply ACLR,  SEM, and SE

#### Issue 1-2-4: Boundary to apply ACLR and SEM - Proposals - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW. - Proposal 2: ACLR, SEM and spurious emissions would be defined based on BS channel bandwidth. - WF - FFS in next meeting - ITU regulation of 250% necessary bandwidth should be considered in further analysis #### Issue 1-2-5: Boundary to apply SE - Proposals - Proposal 1: The application range of SE should be altered with the shifting of the edge of the UE CBW. - Proposal 2: SE is applied at BS channel bandwidth. - WF - FFS in next meeting ITU regulation of 250% necessary bandwidth should be considered in further analysis

Until the last meeting, it have discussed that only one UE CHBW is configured in the BS CHBW which mean we assume single carrier operation. But RAN4 agree to consider many aspects in last meeting. So i think we need to extend our consideration including many aspects. In case of ACLR, SEM, and SE issue, if unified solution is based on network indication. ACLR, SEM, and SE should be based on extended UE CHBW.

Proposal 3: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW.

Proposal 4: The application range of SE should be altered with the shifting of the edge of the UE CBW.

Regarding the ITU regulation of 250% necessary bandwidth. It is related to spurious emission mask. Spurious emission boundary was also defined by reffering to the ITU regulations [2].

<ITU-R SM.329-10>

Spurious emission boundary of ITU was decided that the boundary is 250% of channel bandwidth away from centre frequency. As shown in follow table. According to ITU-R SM.329-10. SE should be consider ITU regulation of 250% necessary bandwidth.

Observation 1: The SE is set based on 250% of the necessary bandwidth in ITU regulation.

And then when we compare SE boundary between ITU regulation and 3GPP, each spurious emission boundary is shown as Figure 2, if the centre frequency is 3GHz and channel bandwidth is 30MHz. the boundary of OOB is 3.015GHz+5MHz so the boundary of OOB is 3.035GHz in 3GPP requirement. but conversely if according to ITU regulation. As a result, the boundary of OOB is 3.075GHz.

Figure 2. Example of spurious emission boundary with ITU regulation and 3GPP requirement

the boundary of SE is different between ITU and 3GPP. The SE boundary is tighter than ITU regulation.

Observation 2: the boundary of SE is different between ITU and 3GPP. The SE boundary is tighter than ITU regulation.

- 2. Conclusion

In this contribution, we want to share some views on Power domain enhancements for single carrier and proposals are made as following:

Proposal 1: Strive to reuse the existing MPR value without additional simulation.

Proposal 2: Consider network indication when converting outer RB allocation to inner allocation as below;

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB_new – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart + RBOffset_left  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_new/2)

Proposal 3: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW.

Proposal 4: The application range of SE should be altered with the shifting of the edge of the UE CBW.

Observation 1: The SE is set based on 250% of the necessary bandwidth in ITU regulation.

Observation 2: the boundary of SE is different between ITU and 3GPP. The SE boundary of 3GPP is tighter than ITU regulation.

- 3. Reference

[1] R4-2417204, “WF on Power domain enhancements”

[2] ITU-R SM.329-10, “Recommendation ITU-R SM.329-10 - Unwanted emissions in the spurious domain”