# 3GPP TSG-RAN WG4 Meeting #113	R4-2419007

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.23.4.1

Source: 	Ericsson

Title:	Simulation results on LP-WUS RRM measurement

Document for:	Information

- Introduction

In this contribution, the LP-WUS RRM measurement performance simulation results will be shown. The detailed simulation assumptions are listed in table below.

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | Agreement: 2.6 GHz initially |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | same SCS as LP-SS |
| Subcarrier spacing | 30KHz |
| Measurement period (in number of measurement samples) |  |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS initially TBD for 15KHz SCS |
| SSS | 30KHz for SSS, TBD for 15KHz |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-1; OOK-4 with M = 2 OOK-4 with M = 4 | when Cell 1 uses OOK-1; OOK-1 or NR signal is used for Cell 2 when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 or NR signal is used for Cell 2 |
| LP-SS pattern |  | [1010] | [0101] |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm OOK based receiver [5 10] ppm | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: company report |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | SNR setting for serving and interference cell are derived based on agreement of Issue 2-1-1-1 |  |
|  |  | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-2.7] - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-11.7] - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
|  |  | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
|  |  | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
| Ês/Iot | dB | -3; -0.5dB; 2dB | N/A |
| Propagation conditions | - | AWGN TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 3: UE-specific parameters

| [Receiver Filter] | [3th/5th Order Butterworth with 3.96MHz bandwidth] |
| --- | --- |
| [Receiver ADC bit width] | [4 or 8-bitADC] |
| [Receiver Sampling Rate for LP-SS only] | [3.84 or 7.68MHz] |

The agreed target accuracy for RSRP and RSRQ is as follow.

Issue 2-1-4: Accuracy baseline for simulation Agreement: Candidate absolute baseband accuracy to derive the number of samples for simulation alignment purpose only: - For FR1: - For OOK based LP-WUR, {±3.5, ±2.5} dB for LP-RSRP and {±3.5, ±2.5} dB for LP-RSRQ - 1st priority: 3.5dB, 2nd priority: 2.5dB - For OFDM based LP-WUR, {±3.5, ±2.5} dB for SS-RSRP and {±3.5, ±2.5} dB for SS-RSRQ - 1st priority: 3.5dB, 2nd priority: 2.5dB - Note 1: the numbers can be updated if needed based on the simulation results and/or the progress in RF session Note 2: the numbers for accuracy requirement is a separate discussion, and different numbers for OOK based and OFDM based LP-WUS for accuracy requirement are not precluded.

- Simulation results

Case 1: SINR=-3dB, SNR=-2.4dB

| Channel | LP-SS signal | Averaging of 1 sample |  | Averaging of 2 samples |  | Averaging of 3 samples |  | Averaging of 4 samples |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 90% RSRP | 90% RSRQ | 90% RSRP | 90% RSRQ | 90% RSRP | 90% RSRQ | 90% RSRP | 90% RSRQ |
| AWGN | OOK-1, 2 OFDM symbols/seq | 1.5 | 1.5 | 1.18 | 1.18 | 1.04 | 1.04 | 0.95 | 0.95 |
|  | OOK-4, M=2 1 OFDM symbol/seq | 2.9 | 2.9 | 2.28 | 2.28 | 1.99 | 1.99 | 1.86 | 1.86 |
|  | OOK-4, M=4 1 OFDM symbol/seq | 2.88 | 2.88 | 2.22 | 2.22 | 1.96 | 1.96 | 1.81 | 1.81 |
| TDLC-300 | OOK-1, 2 OFDM symbols/seq | 1.97 | 1.97 | 1.43 | 1.43 | 1.23 | 1.23 | 1.13 | 1.13 |
|  | OOK-4, M=2 1 OFDM symbol/seq | 3.79 | 3.79 | 2.67 | 2.67 | 2.3 | 2.3 | 2.06 | 2.06 |
|  | OOK-4, M=4 1 OFDM symbol/seq | 3.85 | 3.85 | 2.67 | 2.67 | 2.3 | 2.3 | 2.01 | 2.01 |

Case 2: SINR=-3dB, SNR=-2.7dB

| Channel | LP-SS signal | Averaging of 1 sample |  | Averaging of 2 samples |  | Averaging of 3 samples |  | Averaging of 4 samples |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 90% RSRP | 90% RSRQ | 90% RSRP | 90% RSRQ | 90% RSRP | 90% RSRQ | 90% RSRP | 90% RSRQ |
| AWGN | OOK-1, 2 OFDM symbols/seq | 1.18 | 1.18 | 0.88 | 0.88 | 0.76 | 0.76 | 0.68 | 0.68 |
|  | OOK-4, M=2 1 OFDM symbol/seq | 2.05 | 2.05 | 1.54 | 1.54 | 1.32 | 1.32 | 1.19 | 1.19 |
|  | OOK-4, M=4 1 OFDM symbol/seq | 2.06 | 2.06 | 1.53 | 1.53 | 1.3 | 1.3 | 1.17 | 1.17 |
| TDLC-300 | OOK-1, 2 OFDM symbols/seq | 1.6 | 1.6 | 1.12 | 1.12 | 0.96 | 0.96 | 0.86 | 0.86 |
|  | OOK-4, M=2 1 OFDM symbol/seq | 2.82 | 2.82 | 1.91 | 1.91 | 1.61 | 1.61 | 1.4 | 1.4 |
|  | OOK-4, M=4 1 OFDM symbol/seq | 2.86 | 2.86 | 1.94 | 1.94 | 1.64 | 1.64 | 1.42 | 1.42 |

Case 2: SINR=-3dB, NR interference

| Channel | LP-SS signal | Averaging of 1 sample |  | Averaging of 2 samples |  | Averaging of 3 samples |  | Averaging of 4 samples |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 90% RSRP | 90% RSRQ | 90% RSRP | 90% RSRQ | 90% RSRP | 90% RSRQ | 90% RSRP | 90% RSRQ |
| AWGN | OOK-1, 2 OFDM symbols/seq | 1.13 | 1.13 | 0.8 | 0.8 | 0.65 | 0.65 | 0.56 | 0.56 |
|  | OOK-4, M=2 1 OFDM symbol/seq | 1.64 | 1.64 | 1.15 | 1.15 | 0.93 | 0.93 | 0.8 | 0.8 |
|  | OOK-4, M=4 1 OFDM symbol/seq | 1.74 | 1.74 | 1.2 | 1.2 | 0.99 | 0.99 | 0.84 | 0.84 |
| TDLC-300 | OOK-1, 2 OFDM symbols/seq | 1.36 | 1.36 | 0.95 | 0.95 | 0.78 | 0.78 | 0.67 | 0.67 |
|  | OOK-4, M=2 1 OFDM symbol/seq | 2.2 | 2.2 | 1.5 | 1.5 | 1.22 | 1.22 | 1.05 | 1.05 |
|  | OOK-4, M=4 1 OFDM symbol/seq | 2.33 | 2.33 | 1.58 | 1.58 | 1.28 | 1.28 | 1.11 | 1.11 |

Observation 1: Base on our simulation, 2 samples is enough to achieve the LP-SS RSRP accuracy ±3.5dB.

Observation 2: Base on our simulation, 2 samples is enough to achieve the LP-SS RSRQ accuracy ±3.5dB.

- Reference

[1]. R4-2416861, “WF for RRM requirements for LP-WUS/WUR”, vivo