# 3GPP TSG-RAN WG4 Meeting #113	R4-2419009

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.22.2.2

Source: 	Ericsson

Title:	Discussion on RS and channel adaptation in Rel-19 NES

Document for:	Discussion

# Introduction

In last RAN4 meeting, we achieved some agreements as follow[1].

### Sub-topic 1-1 SSB adapation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism) Issue 1-2-1: Requirements need updating Agreement: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement Note: Above list could be further updated based on RAN4 discussion.

In last RAN1 meeting, the following agreements related to SSB adapataion have achieved.

Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior). - No spec impact

In this contribution, we continue to discuss on the Rel-19 NES operation in Obj. 3 from RAN4 perspective.

# Adaptation of common signal/channel transmissions (CS/CC adaptation)

Scenario

In RAN1 #116 meeting, RAN1 agreed to further study the SSB adaptation in time domain to both PCell and SCell. In last RAN1 meeting, RAN1 agreed that the SSB adaptation is at least the SSB not in sync raster in SCell, but whether the SSB adaptation can be applied to PCell is still FFS. Thus, at least RAN4 will define the SSB adaptation on SCell for NCD-SSB.

RAN1 #116 Agreement For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following: - Applicability to UE in idle/inactive and/or connected mode - Applicability to PCell and/or SCell(s) RAN1 #118 Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Proposal 1: RAN4 to define the NCD-SSB adaptation requirement for SCell first and wait RAN1’s conclusion on whether SSB adaptation can be applied to PCell.

In RAN1 #116bis meeting, NW controlled SSB adaptation is agreed. The UE triggered SSB adaptation is FFS. Thus, we propose RAN4 to focus on NW controlled SSB adaptation firstly.

RAN1 #116 Agreement For the adaptation mechanisms of SSB in time-domain, study further following mechanisms: - Adaptation mechanism indicated or configured by gNB without UE trigger - Adaptation triggered by UE (if any) FFS: Details of associated signaling/indication/configuration RAN1 #116bis Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger

Proposal 2: RAN4 to focus on NW controlled SSB adaptation firstly.

In last meeting, RAN1 had a conclusion that no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19. In our understanding, no further related procedure will be developed later in RAN1. Thus, RAN4 also not to define any related SSB adaptation requirement for IDLE mode.

Proposal 3: RAN4 not to define SSB adaptation requirement for IDLE mode.

Requirement

In last meeting, an LS related to SSB adaptation in L3 measurement has sent out to RAN2. Thus, RAN4 can wait RAN2 ‘s feedback and focus on other issues. If new SMTC will be introduced, UE shall follow the SMTC adaptation. Otherwise, UE may follow the same SMTC although the SSB periodicity changes.

Proposal 4: RAN4 to wait RAN2’s LS about SMTC adaptation in L3 measurement.

In RAN4 #112 meeting, a previous agreement is as follow.

Issue 3-1-2: SSB adaptation impact in RRM requirement - Agreement: SSB adaptation impact in RRM requirement - RAN4 to discuss the L1/L3 measurement requirement impact as starting point.

We think it’s reasonable to introduce SSB adaptation requirement for the L1/L3 measurement procedure. However, we doubt the use case of SSB adapatation during some one shot procedures, such as SCell activation, TCI state switching. NW can easily indicate the SSB adaptation before or after the procedure. Thus, we propose not to consider SSB adaptation during SCell activation, TCI state switching.

Proposal 5: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching.

To achieve the power saving, NW may adapt the SSB periodicities. The case is similar as OD-SSB Case 2. During the transition measurement period, UE can combine the SSB before and after adaptation since the only difference is the periodicities. Subsequent to this measurement period, the measurement period requirements are corresponding to the updated SSB periodicity after transition. Thus, we peopose UE shall combine the SSB measurements before and after SSB adaptation with different periodicities for L1 measurement.

Proposal 6: During L1 measurement, UE shall combine the SSB measurements before and after SSB adaptation with different periodicities.

Proposal 7: During L1 measurement, subsequent to the transition measurement period, the measurement period requirements will follow the SSB periodicity after SSB adaptation.

- Summary

In this contribution, we have discussed the RS adaptation requirements. Based on the discussions, we have made following proposals and observations:

Proposal 1: RAN4 to define the NCD-SSB adaptation requirement for SCell first and wait RAN1’s conclusion on whether SSB adaptation can be applied to PCell.

Proposal 2: RAN4 to focus on NW controlled SSB adaptation firstly.

Proposal 3: RAN4 not to define SSB adaptation requirement for IDLE mode.

Proposal 4: RAN4 to wait RAN2’s LS about SMTC adaptation in L3 measurement.

Proposal 5: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching.

Proposal 6: During L1 measurement, UE shall combine the SSB measurements before and after SSB adaptation with different periodicities.

Proposal 7: During L1 measurement, subsequent to the transition measurement period, the measurement period requirements will follow the SSB periodicity after SSB adaptation.

- References

- R4-2416860, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei