3GPP TSG-RAN WG4 Meeting #113	R4-2419012
Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.1.1.2.1

Source: 	OPPO

Title: 	Discussion on MPR reduction for single carrier

Document for:	Discussion

- Introduction

In RAN4 #112bis meeting, the power domain enhancements for single carrier were discussed and some agreements were approved in WF [1].

Sub-topic 1-1: Approaches to enable MPR reduction for scenario 1 Issue 1-1-1: Approaches for scenarios 1-1 and 1-2 (see Reference for description of scenarios) - Strive for a unified solution which can accommodate both scenario 1 and scenario 2 under the framework of converting outer RB allocation to inner RB allocation. - It does not preclude separate solutions for each applicable scenario - No relaxation of ACLR/SEM/SE values Issue 1-2-1: Clarification of multi-CC scenario - The approach of converting outer RB allocation to inner RB allocation for MPR reduction should consider multi-carrier spectrum scenario from NW perspective while the UE is configured for single CC operation. Issue 1-2-2: Approaches of converting outer RB allocation to inner RB allocation - The approach should consider the following aspects - unified solution could be applicable for both scenarios 1 and scenario 2 - unified solution considering the spectrum allocation status of operators, e.g multi-CC case from NW perspective, to better leverage the feature of MPR reduction - Solution could benefit RedCap and non-RedCap with UE CBW less or even identical to BS CBW - Solution could include both symmetric and asymmetric CBW extension - Whether the extended UE CBW could exceed the BS CBW for different scenarios - Signalling overhead is considered for the proposed approach(s) Issue 1-2-3: Where to use IBE in the larger BS CBW - IBE should be used between edges of UE CBW and extended UE CBW - FFS whether IBE could be used in the guard band of UE CBW - FFS the full RB allocation scenario - FFS the impact to testability Issue 1-2-4: Boundary to apply ACLR and SEM - FFS in next meeting - ITU regulation of 250% necessary bandwidth should be considered in further analysis Issue 1-2-5: Boundary to apply SE - FFS in next meeting - ITU regulation of 250% necessary bandwidth should be considered in further analysis Issue 1-2-6: Which CBW is utilized as the basis for the OOBE requirement and the applicable boundary - FFS in next meeting Issue 1-2-7: Ratio of extended CBW to UE CBW and to larger BS channel BW - To discuss the following aspects for the extended UE CBW: - Whether the extended CBW with new boundary of FOOB should comply with the recommendation by ITU-R on necessary bandwidth in terms of SE for the original UE BW - Whether default or fixed extension, e.g. 1/2 UE CBW could be stipulated in the spec or the extension ratio could be a UE capability to fulfil the conversion of outer to inner - Whether the extended UE CBW could belong to the regular defined channel bandwidth - Whether some premise should be established for the extension size, e.g. - minimum extended CBW compared to original UE CBW - minimum BS CBW compared to UE CBW - frequency separation of the RB allocation to the BS edge - Other identified issues are not precluded Issue 1-2-8: Asymmetrical extended CBW approach - Asymmetrical extension is considered, FFS the solution. Issue 1-2-10: Signaling aspects - To further discuss the signalling aspects in conjunction with the solution to be adopted for MPR reduction. - Companies are encouraged to provide thinking on signalling impact together with the proposed solution

This contribution further discusses power domain enhancement for single carrier.

- Discussion

To convert outer RB allocation to inner RB allocation, it was agreed to shift the starting frequency of ACLR and SEM by extended UE CBW and no relaxation of ACLR/SEM/SE values. If we change the measurement bandwidth of ACLR from based on the original UE CBW to extended UE CBW, the ΔfOOB of SEM also need to change from based on the original UE CBW to extended UE CBW, it will cause further relaxation of the out of band emission. Therefore, the measurement bandwidth of ACLR should use the original UE CBW as the basis, as Figure 2-1:

Figure 2-1 the emission boundary area for extended UE CBW

Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW.

Proposal 2: The original UE CBW should be utilized as the basis for the OOBE requirement.

About the boundary to apply ACLR/SEM and SE, although there is no any agreement, it was proposed ITU regulation of 250% necessary bandwidth should be considered in further analysis.

Therefore, we have to go back to what the regulation recommends of “250% rule” before discussing the boundary to apply ACLR and SEM and SE.

According to Recommendations ITU-R SM.1541 and ITU-R SM.329, normally the boundary between the OOB and spurious domains is the frequency separated from the centra frequency of the emission by 250% of the necessary bandwidth of the emission.

That is, the starting frequency for the spurious emission is 2.5 times the necessary bandwidth separated from the centra frequency of the necessary bandwidth, and is 2 times the necessary bandwidth separated from the edge of the necessary bandwidth.

In my understanding, the necessary bandwidth means the bandwidth of the transmission, in 3GPP, it should be the channel bandwidth.

We review the boundary between FR1 NR out of band and general spurious emission domain, it’s specified the starting frequency of the spurious emission is BWchannel+5MHz from the edge of the channel bandwidth for the channel bandwidth equal to larger than 5MHz in Spec 38.101-1. Therefore, we can get the boundary between FR1 NR out of band and general spurious emission domain is 1.5+5/ BWchannel separated from the centra frequency of the channel bandwidth, it doesn’t meet “250% rule” for the channel bandwidth larger than 5MHz, as listed in Table 2-1.

Obsevation1: The boundary between the OOB and spurious domain doesn’t meet “250% rule” for the channel bandwidth larger than 5MHz in TS 38.101-1.

In TR 38.803, it said such large offsets to the boundary between the OOB and spurious domain may not be necessary for an NR specification, so it’s allowed not to meet “250% rule” for NR.

Come back to this topic, to convert outer RB allocation to inner RB allocation, it was agreed to shift the starting frequency of ACLR and SEM by extended UE CBW. But the UE is configured still on the original UE CBW, the boundary between the OOB and spurious domain from the edge of extended UE CBW should be UE CBW+5MHz. we can get the SE boundary from the the centra frequency of the channel bandwidth should be (0.5*Extended UE CBW+ UE CBW+5MHz)/UE CBW.

If assuming the extended UE CBW is 2 * UE CBW, it will be 2+5/BWchannel, that is larger than the values (1.5+5/ BWchannel) of NR single CC although it still doesn’t meet “250% rule”, as listed in table 2-1:

Table 2-1 Compared the boundary

| BWchannel | Spurious emission separated from the centra frequency of the channel bandwidth |  |
| --- | --- | --- |
|  | NR single CC 1.5+5/ BWchannel | Extended UE CBW 2+5/ BWchannel |
| 5MHz | 250% | 300% |
| 10MHz | 200% | 250% |
| 15MHz | 180% | 230% |
| 20MHz | 175% | 225% |
| 40MHz | 162.5% | 212.5% |
| 50MHz | 160% | 210% |

Proposal 3: The SE limits apply for the frequency ranges that are more than UE CBW+5MHz from the edge of the extended UE CBW.

In lasting meeting, it was agreed that

- Strive for a unified solution which can accommodate both scenario 1 and scenario 2 under the framework of converting outer RB allocation to inner RB allocation and there is no relaxation of ACLR/SEM/SE values.

- The approach of converting outer RB allocation to inner RB allocation for MPR reduction should consider multi-carrier spectrum scenario from NW perspective while the UE is configured for single CC operation.

Therefore, the approach of converting outer RB allocation to inner RB allocation for scenario 2 can also apply to scenario 1 by of shifting the starting frequency of ACLR and SEM to the adjacent carrier, as shown in Figure 2-2:

Figure 2-2 The shifting emission boundary area for scenario 1

We can see from Figure 2-3, the scenario 2 is just one special case of scenario 1 when the extended UE CBW can meet the condition of converting outer RB allocation to inner RB allocation in the BS CBW.

Figure 2-3 The shifting emission boundary area for scenario 2

When the extended UE CBW can’t meet the condition of converting outer RB allocation to inner RB allocation in the BS CBW, we can combine the method of scenario 1 with the method of scenario 2 as shown in Figure 2-4:

Figure 2-3 The shifting emission boundary area combined with scenario 2 and scenario 1

In scenario 2, we call shifting the starting frequency of ACLR and SEM as extended UE CBW. In scenario 1, how to call the shifting frequency range need further discuss, due to the UE CBW is not allowed to exceed the BS CBW in principle.

Actually, the extended UE CBW is just a virtual concept, and used to limit the starting frequency of the ACLR and SEM, the UE is configured only in the original UE CBW. And the shifting frequency range in above figure for scenario 1 is for the same purpose. To facilitate the discussion and the requirements definition, the extended UE CBW can be adopted as the unified terminology for the shifting frequency range for scenario 1 and scenario 2. So it’s necessary to give the assumption that the extended UE CBW is allowed to exceed the BS CBW for different scenarios.

Proposal 4: As an assumption for the requirements that the extended UE CBW is allowed to exceed the BS CBW.

In last meeting, it was agreed that asymmetrical extended UE CBW approach is considered in this topic, the solution need further study.

Figure 2-4 Asymmetrical extended UE CBW

As shown in Figure 2-4, the extended UE CBW can’t extend beyond the left boundary of the BS CBW due to certain limitations, this results in not all outer RB allocation being converted to inner RB allocation.

To resolve this issue for asymmetrical extended UE CBW, the NW can limit the RB allocations in UE CBW to make the UE only use the inner RB allocation as shown in Figure 2-5. The NW can increase the RB allocations for the UE when the extended UE CBW is allowed to extend beyond the left boundary of the BS CBW.

Figure 2-5 Limit the RB allocations of UE CBW for asymmetrical extended UE CBW

The NW can also reduce the UE CBW based on the restrictions, so that the extended UE CBW is symmetrical on both sides of UE CBW, as shown in Figure 2-6:

Figure 2-6 Reduce the UE CBW for asymmetrical extended UE CBW

For asymmetrical extended UE CBW, whether to limit the RB allocations of UE CBW or reduce the UE CBW, it should depend on the NW scheduling, and doesn’t need to be specified in Spec. What RAN4 needs to do is define a minimum extended UE CBW compared to original UE CBW.

In previous meeting, we provided the related simulation results for 20MHz channel bandwidth CP-OFDM QPSK modulation in [2] research the relationship of the MPR reduction and the extended UE CBW. And we got that:

- When extended UE CBW is 2 times of the UE CBW, all outer RB allocation converts to inner RB allocation and the MPR can be improved up to 2.5dB.

- When extended UE CBW is 1.5 times of the UE CBW, the MPR can still be improved up to 2dB although not all outer RB allocation are converted to inner RB allocation.

- When extended UE CBW is 1.25 times of the UE CBW, only small part of regions for outer RB allocation can get improvement.

To guarantee all outer RB allocation can convert to inner RB allocation, 2 times of the UE CBW is needed.

Proposal 5:  RAN4 don’t need specify the solution for the asymmetrical extended UE CBW.

Proposal 6: RAN4 only define the minimum extended UE CBW compared to original UE CBW that can apply to both of symmetrical extended UE CBW and asymmetrical extended UE CBW.

Proposal 7: The minimum extended UE CBW should be 2 times of the original UE CBW.

- Conclusion

This paper analyzed the power enhancements by emission relaxation in the scenario of single operator in a band, and proposed:

Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW.

Proposal 2: The original UE CBW should be utilized as the basis for the OOBE requirement.

Proposal 3: The SE limits apply for the frequency ranges that are more than UE CBW+5MHz from the edge of the extended UE CBW.

Proposal 4: As an assumption for the requirements that the extended UE CBW is allowed to exceed the BS CBW.

Proposal 5:  RAN4 don’t need specify the solution for the asymmetrical extended UE CBW.

Proposal 6: RAN4 only define the minimum extended UE CBW compared to original UE CBW that can apply to both of symmetrical extended UE CBW and asymmetrical extended UE CBW.

Proposal 7: The minimum extended UE CBW should be 2 times of the original UE CBW.

- Reference

[1] R4-2417204 WF on power domain enhancement for UE, Huawei, RAN4 #112bis

[2] R4-2412351 R19 MPR reduction for single carrier, OPPO, RAN4 #112