3GPP TSG-RAN WG4 Meeting #113	R4-2419014
Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.6.3

Source: 	OPPO

Title: 	Discussion on impacts on UE RF requirements

Document for:	Discussion

- Introduction

In RAN4 #112bis meeting, the impacts on UE RF requirements and DL performance were discussed in WF [1]:

Agreements: Issue 3-1: Discussion on assumptions on test configurations for evaluation - Agreement: (a) RAN4 agree on the general assumptions that are same for legacy NR RF performance evaluation: - UE self-band uplink Tx re-use same assumptions for NR PC3 PA i.e., ACLR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dB - Reuse the Rx chain image rejection, which is 25dB, as the starting point. - Agreement: - The configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be re-used for performance evaluation, for ΔRIBNC, REFSENS, both out-of-gap and in-gap ACS, IBB and NBB requirements for selected example combos - Agreement: - Priority to one uplink carrier - Agreement: - Evaluation may consider impairment factors such as ACLR, phase noise or IMDx of aggressor ability, duplexer isolation…etc - The example bands should be in the scope of existing intra-band non-contiguous CA operating bands - No relaxation on any Tx RF requirements should be assumed in this study item - Both PC3 and PC2 should be considered - Prioritize PC3 - The minimum Wgap for ACS, narrow band blocking and in-band blocking verification should meet: - Wgap ≥ 2∙¦FInterferer (offset),j¦ – BWChannel(j) - It is the condition to apply the requirements - Both the symmetric CC scenario and the asymmetric CC scenario should be considered in the FC SI, and companies are encouraged, but not limited, to study the CC scenarios shared by the operators - [Adjusted requirements for fragmented carriers, if any, shall be separated from the existing requirements] - RAN4 shall not re-discuss the power imbalance assumption of the wanted carriers further in relation to the SI on fragmented carriers Issue 3-2: Example band consideration for evaluation - Agreement: - RAN4 doesn’t consider 3MHz channel bandwidth for fragmented carriers - The example band(s) should already have NC DLCA specified - Prioritize the frequency bands of ≤2700MHz - Remove n39 and include n26 in the example bands: Consider n2/n25, n3, n7, n66, n41, n39, n26 as the example band Open issues: Issue 3-3: Observations of impacts on DL Rx requirements - RAN4 discuss if following general aspects can be agreeable: - The largest interferer power in-gap shouldn’t larger than -25dBm for fragmented carriers using a single Rx RF chain - It is proposed to consider only same SCS for both CCs to ease receiver design for the common RF chain - ΔRIBNC for SCC may need to be updated to reflect the filter rejection reduction of common RF chain compared to separate RF chains for each intra-band non-contiguous CA band combination. Furthermore, ΔRIBNC needs to apply to PCC too, although the value of ΔRIBNC can be different for PCC and SCC - RAN4 shall not consider requirements for fragmented carriers which will result in performance degradation as compared to legacy NC IB CA or – only in case of a strong in-gap interferer – single CC operation

In this contribution, we further discuss the impact on UE DL RF requirements reducing the UE Rx chain for fragmented carriers.

- Discussion

In last meeting, it was agreed that a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers to reduce the number of Rx RF chains, shown as Figure 2-1:

Figure 2-1 a fully shared RF chain for fragmented carriers

And a partially shared RF chain architecture in TR 36.823 is the reference architecture for existing DL NCCA requirements as shown in Figure 2-2.

Figure 2-2 a partially shared RF chain for existing DL NCCA

Based on above two architectures for the DL intra-band NC CA, i.e., CA_n25(2A), we can see that:

- Each LPF after the mixer in the partially shared RF chain can reuse the filter bandwidths for the single carriers.

- The LPF after the mixer in the fully shared RF chain need to be adjusted to wider bandwidth to cover two DL CCs and the Wgap between two CCs.

Therefore, for the fully shared RF chain, many additional filter bandwidths are needed to support different aggregated channel bandwidth for fragmented carriers.

Before discussion the rejection performance of the LPF, we can first review how the FDD band REFSENS relaxation requirements (ΔRIBNC) for SCC were defined for intra-band NC CA with one uplink in LTE due to the Tx leakage. From the TR 36.823, we can see

- The impact factors of the transmitter leakage, receiver IM2 noise, LO phase noise from both the transmitter and receiver are considered.

- Only the uplink allocation and duplexer isolation were given in the Tx noise evaluation, the rejection of LPF after the mixer was not mentioned.

And NR REFSENS relaxation requirements (ΔRIBNC) for SCC for intra-band NC CA with one uplink reused the definition of LTE.

Then, compared the fully shared RF chain architecture for the fragmented carrier with the Rx architecture of DL intra-band contiguous CA.

It was assumed DL intra-band contiguous CA are implemented by one RF chain in LTE and NR, therefore, the LPF after the mixer for DL intra-band contiguous CA will cover all the DL aggregated CCs, the bandwidths of LPF are adjusted to wider compared with the LPF for single carrier, as shown in figure 2-3:

Figure 2-3 the LPF of single carrier and DL intra-band C CA

But there is no mention of the deterioration of LPF rejection due to the wider bandwidth in the definition of the Rx requirements of DL intra-band contiguous CA.

Based on above analysis, the REFSENs relaxation of DL intra-band C CA and DL intra-band NC CA don’t consider the rejection of the LPF.

In additional, the fragmented carriers using a fully share Rx chain can reuse the LPF of single carriers and DL intra-band contiguous CA in the implementation when the aggregated bandwidths equal to the bandwidths of single carriers and DL intra-band contiguous CA:

Figure 2-4 the LPF of DL fragmented carriers and DL intra-band C CA

Therefore, we don’t need to evaluate the rejection of LPF with the wider bandwidth for fragmented carriers using a fully share Rx chain.

Proposal 1: The rejection of LPF with the wider bandwidth for fragmented carriers doesn’t need to be evaluated.

Since the UE self-band uplink Tx re-use same assumptions for NR PC3 and the duplexer isolation doesn’t change, the impact of Tx noise to DL SCC and DL PCC of fragmented carriers are the same with DL intra-band NC CA with one PC3 uplink.

Proposal 2: ΔRIBNC for SCC and PCC of fragmented carriers caused by Tx leakage can keep the same with DL intra-band NC CA with one uplink.

The REFSENS relaxation of DL intra-band C CA and DL intra-band NC CA are defined based on the same SCS (15kHz) for both of PCC and SCC in current Spec. The requirements for fragmented carriers can also consider the same SCS for both of PCC and SCC.

Proposal 3: Considering 15kHz SCS for both of PCC and SCC of fragmented carriers.

As the agreement in last meeting, to reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers. Correspondingly, the LPF after the mixer will be adjusted from two LPFs to one wider bandwidth to cover two CC including the gap between two CCs, it will result in no any rejection in the gap between two CCs, as shown in Figure 2-5.

Figure 2-5 The interferer in the gap between two CCs

Therefore, the impact of in-gap interferer and its image and IMDs associated with the interferer to the Rx requirements for PCC and SCC should be considered.

In LTE, 25dB receiver image rejection was defined for DL intra-band C CA due to two component carriers are received the finite receiver image rejection will manifest itself such way that power from PCell will leak to SCell and vice versa, as shown in Figure 2-6:

Figure 2-6 The receiver image for intra-band C CA

From the definition in TS 36.101:

- Receiver image rejection ratio is the ratio of the wanted received power on a sub-carrier being measured to the unwanted image power received on the same sub-carrier when both sub-carriers are received with equal power at the UE antenna connector

We can see the ratio is per sub-carrier, here we use band n25 with 15kHz SCS as the example, the REFSENS and REFSENS/RB is shown in bellow table 2-1:

Table 2-1 REFSENS and REFSENS/RB for n25

| Operating Band |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| n25 | REFSENS | -96.5 | -93.3 | -91.5 | -90.3 |
|  | REFSENS/RB | -110.5 | -110.5 | -110.5 | -110.5 |

Now we evaluate the impact of PCC image to the REFSENS of SCC for fragmented carriers, from above table, we can get the leakage power from PCC to SCC is -135.5 dBm/RB. The REFSENS/RB of SCC will still be -110.5dBm, the impact of receiver image to REFSENs can be ignored if 25dB rejection is assumed.

Observation 1: 25dB receiver image rejection for REFSENs is enough, no REFSENS reduction is needed.

Proposal 4: No REFSENS reduction is needed for both of PCC and SCC of fragmented carriers assuming 25dB receiver image rejection.

The anther impact of the image for fragmented carriers is the interferer image that falling into the wanted received power for the ACS and IBB requirements due to there is no in-gap filter.

As the analysis in [2], there are two cases for ACS and IBB testing:

Case 1: when testing in-gap ACS/IBB1/IBB2 on CC0, if BW of CC0 is larger than CC1, the image of in-gap interference may also fall into co-channel of CC0 as illustrated in Figure 3.

Case 2: On the other hand, when measuring ACS/IBB1/IBB2 of CC0, the image of interference may fall into the co-channel range of CC1 as illustrated in Figure 4.

Here we excerpt the figure from [2] to further analyse the image impact.

Since the conformance test does not measure REFSENS of CC1 when testing ACS/IBB1/IBB2 of CC0, here we only analyse case 1 based on example band n25.

Here we summarized the parameters for ACS and IBBs test cases for single CC with 20MHz channel bandwidth as shown in Table 2-2:

Table 2-2 The parameters for ACS and IBBs test cases for n25 with 20MHz channel bandwidth

|  | ACS |  | IBB1 | IBB2 |
| --- | --- | --- | --- | --- |
|  | Case 1 | Case 2 |  |  |
| Pwanted(dBm) | -76.3 | -50.5 | -81.3 | -81.3 |
| Pinterferer(dBm) | -50.8 | -25 | -56 | -44 |
| BWinterferer | 5MHz | 5MHz | 5MHz | 5MHz |
| Pwanted- Pinterferer (dB) | -25.5 | -25.5 | -25.3 | -37.3 |

We assumed CC0 is 20MHz, when testing in-gap ACS on CC0, the image of interferer will fall into CC0 as Figure 3, based on Table 2-2 assuming 25dB receiver image rejection, we can get the related image power of the interferer and the worsen power in transmission bandwidth configuration as shown in table 2-3:

Table 2-3 The parameters with the image impact for ACS and IBBs test cases for n25 with  20MHz channel bandwidth

|  | ACS |  | IBB1 | IBB2 |
| --- | --- | --- | --- | --- |
|  | Case 1 | Case 2 |  |  |
| Pwanted(dBm) | -76.3 | -50.5 | -81.3 | -81.3 |
| Pimage | -75.8 | -50 | -81 | -69 |
| Pwanted+Pimage (dBm) | -73 | -47.2 | -78 | -68.8 |
| Pinterferer(dBm) | -50.8 | -25 | -56 | -44 |
| (Pwanted+Pimage)- Pinterferer (dB) | -22.2 | -22.2 | -22 | -24.8 |

Compared Table 2-3 with table 2-2, we can see the deterioration of ACS and IBB performance and it cannot meet current requirements

Table 2-4 the deterioration of ACS and IBB performance

|  | ACS |  | IBB1 | IBB2 |
| --- | --- | --- | --- | --- |
|  | Case 1 | Case 2 |  |  |
| Pwanted- Pinterferer (dB) | -25.5 | -25.5 | -25.3 | -37.3 |
| (Pwanted+Pimage)- Pinterferer (dB) | -22.2 | -22.2 | -22 | -24.8 |
| Deterioration (dB) | 3.3 | 3.3 | 3.3 | 12.5 |

Observation 2: Assuming 25dB receiver image rejection, ACS and IBB1 performance decreased by 3.3 dB and IBB2 performance decreased by 12.5 dB.

If we keep parameters for ACS and IBBs test cases unchanged, the receiver image rejection needs up to 46dB to meet current requirements for ACS and IBBs as Table 2-5 calculated, it’s hard to achieve for current component performnce:

Table 2-5 The Receiver image rejection without deterioration of ACS and IBB performance

|  | ACS |  | IBB1 | IBB2 |
| --- | --- | --- | --- | --- |
|  | Case 1 | Case 2 |  |  |
| Pwanted(dBm) | -76.3 | -50.5 | -81.3 | -81.3 |
| Pimage | -82.2 | -82.5 | -90 | -90 |
| Pwanted+Pimage (dBm) | -75.3 | -50.5 | -81.3 | -81.3 |
| Pinterferer(dBm) | -50.8 | -25 | -56 | -44 |
| (Pwanted+Pimage)- Pinterferer (dB) | -25.5 | -25.5 | -25.3 | -37.3 |
| Receiver image rejection | 31.4 | 57.5 | 34 | 46 |

Proposal 5: The relaxation of the in-gap ACS/IBB is needed.

- Conclusion

This paper discussed the RF requirements for fragmented carriers and proposed:

Proposal 1: The rejection of LPF with the wider bandwidth for fragmented carriers doesn’t need to be evaluated.

Proposal 2: ΔRIBNC for SCC and PCC of fragmented carriers caused by Tx leakage can keep the same with DL intra-band NC CA with one uplink.

Proposal 3: Considering 15kHz SCS for both of PCC and SCC of fragmented carriers.

Observation 1: 25dB receiver image rejection for REFSENs is enough, no REFSENS reduction is needed.

Proposal 4: No REFSENS reduction is needed for both of PCC and SCC of fragmented carriers assuming 25dB receiver image rejection.

Observation 2: Assuming 25dB receiver image rejection, ACS and IBB1 performance decreased by 3.3 dB and IBB2 performance decreased by 12.5 dB.

Proposal 5: The relaxation of the in-gap ACS/IBB is needed.

Reference

[1] R4-2417201, WF on fragmented DL, MediaTek Inc., RAN4#112bis 2024-10

[2] R4-2416164, Discussion of impacts on UE RF requirements and DL performance impacts, MediaTek Inc., RAN4#112bis 2024-10