3GPP TSG-RAN WG4 Meeting #113	R4-2419016
Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.8.2.2.2

Source: 	OPPO

Title: 	Discussion on Rx requirements for NR-NTN HPUE

Document for:	Discussion

- Introduction

In last RAN4 meeting, the HPUE Rx RF requirements with NR NTN operating in FR1-NTN bands were discussed and some agreements are approved in WF [1]:

Agreements: Issue 3-1: RSD form in spec - Use similar way in TN spec to derive RSD, if needed. Issue 3-2: RSD requirements for PC2 - For n256: [0dB] RSD for PC2 NR-NTN for 1Tx and 2Tx - FFS on the duplexer isolation and PA interference Issue 3-4: REFSENS/RSD testing - For PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration with PC3 NR NTN UE as starting point. Open issue: Issue 3-3: RSD requirements for other PC (PC1.5 and PC1) - FFS RSD for PC1/PC1.5 for applicable UE architectures.

This contribution further discussed the UE Rx RF requirements for NR NTN HPUE.

- Discussion

In last meeting, it was agreed [0dB] RSD for PC2 NR-NTN for 1Tx and 2Tx in band n256, but the duplexer isolation and PA interference need further study.

Compared NTN band n256 to TN band n1, the gap between UL frequency and DL frequency of n256 is 60MHz that is larger than the 30MHz gap of n1.

|  | UL frequency | DL frequency |  |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |

Therefore, the duplexer isolation of band n256 is better than that of band n1, and the related impact of Tx noise to the Rx receiver of band n256 is smaller than that of band n1.

Proposal 1: 0dB RSD can be specified for PC2 NR NTN UE in n256.

For n1, 0 dB RSD for PC2 is defined for different channel bandwidth. And in last meeting, it agreed 1 Tx and 2 Tx for PC2 NR NTN UE are to be considered for requirements, that is similar with the architecture of TN PC2 UE. Band n256 can reuse the RSD of n1.

For band n255, the gap between UL frequency and DL frequency is 65.5 MHz, there is no influence of Tx transmission to the REFSENS of PC3, compared band n1, band n255 can also reuse the RSD of n1.

Proposal 2: 0dB RSD can be specified for PC2 NR NTN UE in n255.

In our companion contribution, we proposed PC 1 and PC1.5 only apply to non-handheld UE, the RSD is not needed for PC1 and PC 1.5.

Proposal 3: no RSD is needed for non-handheld UE including PC 1 and PC 1.5.

- Conclusion

In this paper we give further discussion on the RX requirement for HPUE NTN. The proposals are as following:

Proposal 1: 0dB RSD can be specified for PC2 NR NTN UE in n256.

Proposal 2: 0dB RSD can be specified for PC2 NR NTN UE in n255.

Proposal 3: no RSD is needed for non-handheld UE including PC 1 and PC 1.5.

- Reference

[1] R4-2417122, WF on co-existence study and HPUE for NR-NTN, Samsung, RAN4#112bis 2024-10