3GPP TSG-RAN WG4 Meeting #113                                         R4-2419025

Orlando, US, November 18 - 22, 2024

Source: 	Huawei, HiSilicon

Title: 	On power domain enhancements for single carrier

Agenda Item:	7.1.1.2.1

Document for:	Approval

Introduction

During RAN4#112bis, power domain enhancement for single carrier has been discussed. Conclusions and FFS points can be found in [1]. In this contribution, we would like to share our views on this topic concentrating on the general solution.

Discussion

- On the unified solution for MPR reduction

As captured in WF, RAN4 is tasked to find a unified solution which can satisfy operator demands projected in all three scenarios.

- Approaches for scenarios 1-1 and 1-2 (see Reference for description of scenarios) - WF - Strive for a unified solution which can accommodate both scenario 1 and scenario 2 under the framework of converting outer RB allocation to inner RB allocation. - It does not preclude separate solutions for each applicable scenario - No relaxation of ACLR/SEM/SE values Clarification of multi-CC scenario - WF - The approach of converting outer RB allocation to inner RB allocation for MPR reduction should consider multi-carrier spectrum scenario from NW perspective while the UE is configured for single CC operation. Approaches of converting outer RB allocation to inner RB allocation - WF - The approach should consider the following aspects - unified solution could be applicable for both scenarios 1 and scenario 2 - unified solution considering the spectrum allocation status of operators, e.g multi-CC case from NW perspective, to better leverage the feature of MPR reduction - Solution could benefit RedCap and non-RedCap with UE CBW less or even identical to BS CBW - Solution could include both symmetric and asymmetric CBW extension - Whether the extended UE CBW could exceed the BS CBW for different scenarios - Signalling overhead is considered for the proposed approach(s)

The principle here is to fully utilize the available spectrum by taking adjacent frequency situation into account and construct the extended UE CBW accordingly.

Proposal 1: The principle of the unified solution is to fully utilize the available spectrum by taking adjacent frequency situation into account and construct the extended UE CBW accordingly.

- In terms of the extended RBs in the extended UE CBW, clarify that they are not allowed for any UL/DL scheduling or measurement.

Since the network would possess the aforementioned information, the key point here is to let the UE be aware of the position relationship between the original UE CBW and the extended UE CBW, so that the outer RB allocations where inner MPR is expected to be applied are clear to the UE.

Observation 1: The length and location of the extended UE CBW should be indicated to UE in order to let it know where inner MPR is expected to be applied besides the original Inner RB allocations.

According to TS 38.331, the existing UE CBW configuration procedure comprises of following two circumstances:

- If the UE supports the cell-specific carrier bandwidth indicated in SIB1, then this carrier bandwidth would be determined accordingly;

- Or if not, a UE-specific carrier bandwidth would be configured by the network via RRC reconfiguration signalling eventually.

The location of the actual carrier bandwidth is determined by IE offsetToCarrier in SCS-SpecificCarrier, which is the offset to the absolute frequency location of Point A.

Since the offset is determined and configured by the network, there could be a possibility that the ‘lowest RB’ of the extended UE CBW is even lower than the Point A.

Figure 1. The location of the ‘RB0’ of the extended UE CBW may not be higher than Point A

Observation 2: As the value of offsetToCarrier is determined and configured by the network dedicated for the UE CBW, there could be a chance that the ‘RB0’ of the extended UE CBW is located lower than Point A.

In this regard, we think a proper way to indicate the location and length of the extended UE CBW to UE without changing the current specification too much could be a configuration on the number of the extended RBs at each side of UE CBW rather than introduce another offset to cover the aforementioned possibility, because the range of extended RB number is fixed as the required extended RB for 100MHz UE CBW would be the one specified for 50MHz With that, all three scenarios can be covered, examples are shown as in below table.

Table 1. Illustrations on the unified solution

| Region | Effect |  |
| --- | --- | --- |
| China (Scenario 1-1) | Band n41 |  |
|  | Band n78 |  |
| Netherlands (Scenario 1-2) | Band n78 |  |
| Spain (Scenario 2) | Band n78 |  |

With the location and width of the extended UE CBW indicated to UE, the ‘new’ Inner region (i.e. the area surrounded by the red dashed line as in Table 1) can be derived as the intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW.

In conclusion, we propose the following way to incorporate the indication of location & width of the extended UE CBW with the specified procedure of UE CBW configuration i.e. Step 1 and 2.

- The specified procedure for UE CBW configuration

- UE capability reporting on the support of Rel-19 MPR reduction for single CC

- Depending on the adjacent frequency situation, NW determines and configures the extended RBs to each side of the UE CBW. Note that they can be different values and varied from zero to any one specified in Table 5.3.2-1 in TS 38.101-1 but no need to exceed the one corresponding to 50MHz, which is the half of the maximum UE CBW 100MHz.

Proposal 2: Introduce new RRC signalling to indicate the extended RB number at each side of the UE CBW, then the intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE.

- The extended RB refers to the same subcarrier spacing as UE CBW;

- The extended RB number for each side can be different and the value could vary from zero to the one specified in Table 5.3.2-1 in TS 38.101-1, but it is no need to exceed the one corresponding to 50MHz, which is the half of the maximum UE CBW 100MHz.

- Consideration on the requirements

- How to relax the out-of-band requirements

For the out-of-band (OOB) and spurious domain definition, 3GPP adopted a more strict requirement than the one defined by ITU-R which is introduced as 250% of the necessary bandwidth from the center of transmission as the boundary according to SM.329 [2]. In this sense, we think from regulatory point of view there would be no issue to consider shifting the applicable range of OOB and spurious emision together to half of the UE CBW away. But if the applicable range and/or shifting distance will be based on BS CBW, then there could be a risk on regulatory violation. Thus we have the following preferences with the intention of maximizing the gain:

- ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW, since this is way more general than only consider the applicable range from the edge of BS CBW.

- The application range of SE should be altered with the shifting of the edge of the UE CBW, which has no violation to the regulatory.

- The applicable range of ACLR, SEM and SE requirements should still be based on UE CBW, because the UE CBW is where UL transmission will be scheduled but not the extended UE CBW.

Proposal 3: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW.

Proposal 4: The application range of SE should be altered with the shifting of the edge of the UE CBW.

Proposal 5: The applicable range of ACLR, SEM and SE should still be based on UE CBW.

- How to apply the IBE in the extended UE CBW

The related FFS point is reproduced as below:

- Where to use IBE in the larger BS CBW - WF - IBE should be used between edges of UE CBW and extended UE CBW - FFS whether IBE could be used in the guard band of UE CBW - FFS the full RB allocation scenario - FFS the impact to testability

As it was agreed to use IBE between the edges of UE CBW and the extended UE CBW, the detailed should be further checked. Firstly, it should be maintained that no requirement will be applied to the guard band within UE CBW.

Proposal 6: No RF requirement applies to the guard band within UE CBW.

Based on that, we propose the following design regarding how to apply the IBE for the extended UE CBW. The following two figures demonstrate how the design works.

For the full RB allocation scenario, the RB index for the first RB (pair) adjacent to the UE CBW becomes +1(-1) and it is applied for all extended RBs, so the guard band within the UE CBW is not counted. The actual equation derived from the general part of IBE would be:

max-25, 10log10EVM-3, -57dBm+10log10SCS15kHz-PRB

Figure 2. Illustration on the full RB allocation scenario where IBE applies to the extended UE CBW

For the partial RB allocation scenario, the first RB adjacent to the UE CBW can be counted as the next RB index to the one used by the very last RB within the scheduled block at the same side. Similarly, the index can be applied to all the extended RBs at each side, respectively.

Figure 3. Illustration on the partial RB allocation scenario where IBE applies to the extended UE CBW

Proposal 7: For the partial RB allocation case, the RB index that used for IBE value derivation applies to all the extended RBs at one side is numbering right after the adjacent usable RB within UE CBW at that side.

Proposal 8: For the full RB allocation case, the RB index that used for IBE value derivation applies to all the extended RBs is set to +1(-1). The equation for IBE derivation is:

max-25, 10log10EVM-3, -57dBm+10log10SCS15kHz-PRB

It is self-evident that such approach has no fundamental impact to TE implementation because for FR2 the 200MHz UE channel bandwidth is already specified.

Observation 3: The proposed method to apply IBE to the extended UE CBW has no fundamental impact to TE implementation because the >100MHz UE channel bandwidth is already specified in FR2.

Conclusion

In this contribution we discussed on the Rel-19 MPR reduction for single CC. We have the following observations and proposals:

Observation 1: The length and location of the extended UE CBW should be indicated to UE in order to let it know where inner MPR is expected to be applied besides the original Inner RB allocations.

Observation 2: As the value of offsetToCarrier is determined and configured by the network dedicated for the UE CBW, there could be a chance that the ‘RB0’ of the extended UE CBW is located lower than Point A.

Observation 3: The proposed method to apply IBE to the extended UE CBW has no fundamental impact to TE implementation because the >100MHz UE channel bandwidth is already specified in FR2.

Proposal 1: The principle of the unified solution is to fully utilize the available spectrum by taking adjacent frequency situation into account and construct the extended UE CBW accordingly.

- In terms of the extended RBs in the extended UE CBW, clarify that they are not allowed for any UL/DL scheduling or measurement.

Proposal 2: Introduce new RRC signalling to indicate the extended RB number at each side of the UE CBW, then the intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE.

- The extended RB refers to the same subcarrier spacing as UE CBW;

- The extended RB number for each side can be different and the value could vary from zero to the one specified in Table 5.3.2-1 in TS 38.101-1, but it is no need to exceed the one corresponding to 50MHz, which is the half of the maximum UE CBW 100MHz.

Proposal 3: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW.

Proposal 4: The application range of SE should be altered with the shifting of the edge of the UE CBW.

Proposal 5: The applicable range of ACLR, SEM and SE should still be based on UE CBW.

Proposal 6: No RF requirement applies to the guard band within UE CBW.

Proposal 7: For the partial RB allocation case, the RB index that used for IBE value derivation applies to all the extended RBs at one side is numbering right after the adjacent usable RB within UE CBW at that side.

Proposal 8: For the full RB allocation case, the RB index that used for IBE value derivation applies to all the extended RBs is set to +1(-1). The equation for IBE derivation is:

max-25, 10log10EVM-3, -57dBm+10log10SCS15kHz-PRB

- References

- R4-2417204, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #112bis, Hefei, China, October 14-18, 2024.

- Annex

- Scenario classification based on the worldwide spectrum allocation

As RAN4 agreed to study below three scenarios, it would be beneficial to look closely at each one and take the worldwide spectrum allocation and deployment into account, so that the corresponding solution can be more targeted.

In the following part, we list all the suitable examples under each scenario. What needs to be noted is that the spectrum allocation/deployment may not be static by the time passage where spectrum policy and business decision could weight in, so we only explicitly mark the region but waive the operator.

The distinction between Scenario 1-1 and Scenario 1-2

For these two scenarios, we understand the key criteria to distinguish Scenario 1-1 from Scenario 1-2 is whether there is any other operator(s) close to the holding spectrum. Specifically, the Scenario 1-1 stands for the case where a sufficient gap (at each side) exists between two allocations of spectrum (not shorter than the half of the indicated maximum UE channel bandwidth or the indicated OOB boundary shifting offset for UE to apply inner MPR to out RB allocations). Otherwise, it should be Scenario 1-2. Note that the case where the holding spectrum is larger than 100MHz should be included as well, because besides the gap there are operator’s own frequencies that can be used as offset for shifting the OOB boundary.

Observation: The key criteria to distinguish Scenario 1-1 from Scenario 1-2 is whether there is sufficient gap between operator holding spectrums.

- The gap (at each side) is sufficient if it is larger than the half of the indicated maximum UE channel bandwidth or the indicated OOB boundary shifting offset for UE to apply inner MPR to out RB allocations.

- The case where the concerned holding spectrum is larger than 100MHz should be included as well.

Accordingly, we can screen out the matched ones based on the current spectrum allocation and list them in Table 2. One interesting finding is that for a same band e.g. n78, it can belong to different scenario due to certain spectrum allocation and deployment.

Table 1. On how the scenario belonging would change for band n78 in different region

| Region | Spectrum | Matched Scenario |
| --- | --- | --- |
| China |  | 1-1 (300MHz) |
| Romania |  | 1-2 (115MHz, at least 15MHz at each side can be used as offset) |

Table 2. On the operator holding spectrum that fits Scenario 1-1 or Scenario 1-2

| Region | NR band | Holding spectrum for UL | Matched Scenario | Note |
| --- | --- | --- | --- | --- |
| Australia | n7 | 2510MHz ~ 2550MHz 40MHz | 1-2 | Insufficient gap adjacent to this CBW (>2550MHz is occupied by adjacent operator) |
| Britain | n40 | 2350MHz ~ 2390MHz 40MHz | 1-1 | Single operator |
| China | n28 | 703MHz ~ 733MHz 30MHz | 1-1 | Co-constructed and shared by operators |
|  | n5 | 825MHz ~ 835MHz 10MHz | 1-1 | Single operator |
|  | n39 | 1885MHz ~ 1915MHz 30MHz | 1-2 | Insufficient gap (5MHz) adjacent to this CBW |
|  | n1 | 1920MHz ~ 1965MHz 45MHz | 1-2 | Co-constructed and shared by operators Insufficient gap (15MHz) adjacent to this CBW |
|  | n34 | 2010MHz ~ 2025MHz 15MHz | 1-1 | Single operator |
|  | n40 | 2300MHz ~ 2370MHz 70MHz | 1-1 | Single operator |
|  | n41 | 2515~2675MHz 160MHz | 1-1 | Single operator |
|  | n78 | 3300~3600MHz 300MHz | 1-1 | Co-constructed and shared by operators |
|  | n79 | 4800~4960MHz 160MHz | 1-1 | Co-constructed and shared by operators |
| Croatia | n78 | 3480MHz ~ 3600MHz 120MHz | 1-2 | Insufficient gap adjacent to this CBW (>3600MHz is occupied by adjacent operator) |
| Denmark | n1 | 1920MHz ~ 1940MHz 20MHz | 1-2 | Insufficient gap adjacent to this CBW (>1940MHz is occupied by adjacent operator) |
|  | n40 | 2300MHz ~ 2400MHz 100MHz | 1-1 | Single operator |
|  | n78 | 3410MHz ~ 3540MHz 130MHz | 1-2 | Insufficient gap adjacent to this CBW (>3540MHz is occupied by adjacent operator) |
|  | n78 | 3540MHz ~ 3660MHz 120MHz | 1-2 | Insufficient gap adjacent to this CBW (<3540MHz is occupied by adjacent operator) |
| Finland | n78 | 3410MHz ~ 3540MHz 130MHz | 1-2 | Insufficient gap adjacent to this CBW (>3540MHz is occupied by adjacent operator) |
| Greek | n78 | 3410MHz ~ 3450MHz 40MHz | 1-2 | Insufficient gap adjacent to this CBW (>3450MHz is occupied by adjacent operator) |
| Hungary | n78 | 3410MHz ~ 3470MHz 60MHz | 1-2 | Insufficient gap adjacent to this CBW (>3470MHz is occupied by adjacent operator) |
| Ireland | n40 | 2300MHz ~ 2360MHz 60MHz | 1-1 | Single operator |
| Lithuania | n40 | 2310MHz ~ 2390MHz 80MHz | 1-1 | Single operator |
| Luxembourg | n78 | 3420MHz ~ 3530MHz 110MHz | 1-2 | Insufficient gap adjacent to this CBW (>3540MHz is occupied by adjacent operator) |
|  | n78 | 3540MHz ~ 3650MHz 110MHz | 1-2 | Insufficient gap adjacent to this CBW (<3530MHz and >3650MHz are occupied by adjacent operator, respectively) |
|  | n78 | 3650MHz ~ 3750MHz 100MHz | 1-2 | Insufficient gap adjacent to this CBW (<3650MHz is occupied by adjacent operator and 3800MHz is the edge of n78) |
| Netherland | n34 | 2010MHz ~ 2025MHz 15MHz | 1-1 | Single operator |
| Norway | n78 | 3400MHz ~ 3480MHz 80MHz | 1-1 | Sufficient gap (100MHz) adjacent to this CBW |
|  | n78 | 3580MHz ~ 3700MHz 120MHz | 1-2 | Insufficient gap adjacent to this CBW (>3700MHz is occupied by adjacent operator) |
| Romania | n78 | 3700MHz ~ 3750MHz 50MHz | 1-1 | Sufficient gap (>=50MHz) adjacent to this CBW |
|  | n78 | 3490MHz ~ 3530MHz 40MHz | 1-2 | Insufficient gap adjacent to this CBW (>3530MHz is occupied by adjacent operator) |
|  | n78 | 3530MHz ~ 3645MHz 115MHz | 1-2 | Insufficient gap adjacent to this CBW (<3530MHz and >3700MHz are occupied by adjacent operator, respectively) |
| Slovenia | n34 | 2010MHz ~ 2025MHz 15MHz | 1-1 | Single operator |
| Spain | n28 | 703MHz ~ 733MHz 30MHz | 1-1 | Co-constructed and shared by operators |
|  | n78 | 3420MHz ~ 3500MHz 80MHz | 1-2 | Insufficient gap adjacent to this CBW (<3420MHz is occupied by adjacent operator) |
| Sweden | n78 | 3400MHz ~ 3500MHz 100MHz | 1-2 | Insufficient gap adjacent to this CBW (>3500MHz is occupied by adjacent operator) |
|  | n78 | 3500MHz ~ 3620MHz 120MHz | 1-2 | Insufficient gap adjacent to this CBW (<3500MHz is occupied by adjacent operator) |
| Switzerland | n78 | 3500MHz ~ 3580MHz 80MHz | 1-2 | Insufficient gap adjacent to this CBW (>3580MHz is occupied by adjacent operator) |

On Scenario 2

For this scenario, there is a previous agreement that the shifted boundary of where ACLR and SEM applied cannot exceed the BS CBW in order to avoid additional interference to adjacent operators. Hence, any spectrum allocation with no gap to other operator in adjacent channels can be incorporated to Scenario 2. But one should focus on the case where the operator holding spectrum is larger than 100MHz instead and it doesn’t need to confine to single carrier operation at BS side because:

- The multi-carriers deployment could have a disposition to configure 100MHz to one CC as it could be a practical decision after considering achievable peak throughputs and overall coverage performance along with the maximum CBW capability normally indicated by an eMBB UE i.e. 100MHz.

- But if it is arbitrarily confined to single carrier, to accommodate the required offset e.g. 1/2 UE CBW into holding spectrum, the gNB may have to configure the UE with a smaller CBW if it wants to enable this feature.

- The reduced MPR is still the same as the original one for inner region, which can be achieved under inner RB allocations scheduling.

- Since a smaller UE CBW is configured, the original outer RB allocation cannot be scheduled anymore.

Figure 1. Illustration on the drawbacks when confining to single carrier at BS side

Observation: As depicted in Figure 1, for Scenario 2, one should focus on the case where the operator holding spectrum is larger than 100MHz and it is multi-carriers operation at BS side. Because:

- For >100MHz contiguous spectrum, it could have a disposition to configure 100MHz to one CC as it could be a practical decision after considering achievable peak throughputs and overall coverage performance along with the maximum CBW capability normally indicated by an eMBB UE i.e. 100MHz.

- But if it is arbitrarily confined to single carrier, to accommodate the required offset e.g. 1/2 UE CBW into holding spectrum, the gNB may have to configure the UE with a smaller CBW rather than the supported maximum UE CBW, if it wants to enable this feature.

- The reduced MPR is still the same as the one that can be achieved on the very same RB allocations, which are identical to the inner RB allocations within the original maximum UE CBW.

- Since a smaller UE CBW is configured, the original outer RB allocation cannot be scheduled anymore.

- Last but not the least, the motivation of this feature is to unleash the UE MPR potential but not power saving.

Then we can list the qualified cases in below table.

Table 3. On the operator holding spectrum that fits Scenario 2

| Region | NR band | Holding spectrum for UL | Note |
| --- | --- | --- | --- |
| Any kind of spectrum allocation that the BS sees fit for extended UE CBW |  |  | Under a certain spectrum allocation, the BS may need to reconfigure the UE CBW in order to accommodate the extended UE CBW. |
| Finland | n78 | 3540MHz ~ 3670MHz 130MHz | No gap adjacent to this CBW (<3540MHz and >3670MHz are occupied by adjacent operator, respectively) |
| Greek | n78 | 3450MHz ~ 3600MHz 150MHz | No gap adjacent to this CBW (<3450MHz and >3600MHz are occupied by adjacent operator, respectively) |
| Hungary | n78 | 3540MHz ~ 3660MHz 120MHz | No gap adjacent to this CBW (<3540MHz and >3660MHz are occupied by adjacent operator, respectively) |
| Slovenia | n78 | 3560MHz ~ 3700MHz 140MHz | No gap adjacent to this CBW (<3560MHz and >3700MHz are occupied by adjacent operator, respectively) |
| Spain | n78 | 3600MHz ~ 3710MHz 110MHz | No gap adjacent to this CBW (<3600MHz and >3710MHz are occupied by adjacent operator, respectively) |
| Switzerland | n78 | 3580MHz ~ 3700MHz 120MHz | No gap adjacent to this CBW (<3580MHz and >3700MHz are occupied by adjacent operator, respectively) |

System level simulation on including BS side multi-carriers operation for MPR reduction

In TR 36.942, the assumption for determining co-existence requirements i.e. UE ACLR who dominates the ACIR is captured as a 3%~5% throughput loss tolerance. By taking this experience value as metric, we evaluate the co-existence performance on applying MPR reduction to BS side multi-carriers operation with following considerations:

- For the BS side multi-carriers operation, it is two contiguous CCs (same RB length) in the same band.

- For a victim PC3 UE on CC1, the aggressor is another PC3 UE on CC2 that will apply MPR reduction by shifting the ACLR boundary overlapping with CC1.

- As the ACLR boundary shifting away, the interference in the first adjacent channel (the overlapping part in CC1) is increased. Following figure can better explain it.

Figure 2. Illustration on why the interference in the 1st adjacent channel is increased

- As depicted in Figure 2, since the PSD would be lower in the “new” 1st adjacent channel and the same ACLR value applies in that integral region, there must be a room for the aggressor to deliver higher output power by utilizing reduced MPR. Thus the interference landed in CC1 will increase.

- By observing the throughput loss on CC1 with different shifted ACLR boundary, we can identify the offset when the loss hits e.g. 3%.

|  | Throughput loss. ACLR boundary; @50% CDF; @90% CDF. Shift to 1/5 UE CBW; 0.88%; 1.43%. Shift with 1/2 UE CBW; 2.26%; 2.78%. |
| --- | --- |

Figure 3. Comparison on the CDF curve of throughput when different ACLR boundary is applied

From the above results, we can find that the throughput loss due to reduced MPR is acceptable when the ACLR integral region is overlapping with the adjacent CC.

Observation: Taking 3% throughput loss as the metric, the interference to the 1st adjacent channel (overlapping with another CC right next to the concerned CC) caused by a UE enables MPR reduction is acceptable.