3GPP TSG-RAN WG4 Meeting #113                                         R4-2419032

Orlando, US, November 18 - 22, 2024

Source: 	Huawei, HiSilicon

Title: 	On RF requirements for Rel-19 NR-MIMO

Agenda Item:	7.19.2

Document for:	Approval

Introduction

In previous meeting, RAN4 further discussed on this WI. The progress on FR1 3Tx and newly proposed scenario for mTRP operation in FR2 can be found in [1].

- Power class <Agreement>: PC for 3Tx requirement - PC for 3Tx requirement - PC3 - PC2 - FFS on PC1.5 - FFS on UE type, i.e., FWA and/or handheld UE - FFS on support of full power mode 0 - RAN4 understanding is that TxD for 3Tx is not in the scope of this WI - Full power mode 0 apply to PC2 and PC3 only. - FFS on for which UE types it should apply. <Way forward>: FFS on New capability for power class per band per declared UL power mode for 3Tx - Supported layer for mode 0 <Way forward >: FFS on Supported MIMO layer for ULFPTx mode 0 Asymmetric TRPs <Way forward>: Whether to study FR2 enhancements for asymmetric TRPs scenario in Rel-19 - RAN4 will not study FR2 enhancements until RAN1 considers the scenario in Option 1 - Option 1: Yes, UL-specific TRPs are assumed to have limited DL capability - Option 2: No, need more check - Option 3: Check with RAN1 sending an LS

In this contribution, we would like to provide our views on this topic.

Discussion

On FR1 3Tx operation

- Full power transmission mode

It was agreed to further study whether to introduce new capability for power class per declared full power mode for 3Tx. We think one fundamental issue needs to be clarified is under what circumstances that a 3Tx UE would shift its advertised power class and expect different configuration on the full power mode. Note that back in Rel-18 a similar discussion happened but it was dedicated for the so called “power class fallback” scenario. So it could make the Rel-19 discussion more efficient if the proponent can elaborate more about the concerned scenario.

Observation 1: Clarification on the concerned scenario where a 3Tx UE would shift its advertised power class and expect different configuration on the full power mode could make the discussion more efficient in Rel-19.

As for the layer aspect, we think to introduce the test for single layer and omit two layers is a preferable way since it can reduce the test burden without leave any room for unqualified UE.

Proposal 1: Suggest to only specify single layer precoders for 3Tx ULFPTx mode 0 verification.

- MPR requirements for 3Tx

During the discussion in last meeting, we understand the centre of the controversy is how to consider PC1.5 for 3Tx. As it would be somehow related to the assumed PA configurations behind it, we think neither 3x29dBm nor 3x26dBm can be ready from industry perspective especially for handheld UE. Thus we have the following proposal.

Proposal 2: For Rel-19 RAN4 3Tx discussion under Rel-19 NR_MIMO_Ph5 WI, suggest to preclude PC1.5 at least for handheld UE.

On the asymmetric DL sTRP/UL mTRP

Another interesting issue is whether to additionally consider SSB transmission capability for the “UL-specific TRP” in RAN4. While we agree to make further decision in RAN4 after RAN1 reply LS can be formulated, if in RAN1 discussion the TRP is capable of DL transmission but it may or may not configure the UE for corresponding reception, then we think such kind of deployment is already covered by the spec then seemingly no dedicated requirement for such scenario is needed.

Observation 2: If in RAN1 discussion the TRP is capable of DL transmission but it may or may not configure the UE for corresponding reception, then we think such kind of deployment is already covered by the spec then seemingly no dedicated requirement for such scenario is needed.

Conclusion

In this contribution we discussed on the RF requirements for Rel-19 NR MIMO. We have the following observation and proposals:

Observation 1: Clarification on the concerned scenario where a 3Tx UE would shift its advertised power class and expect different configuration on the full power mode could make the discussion more efficient in Rel-19.

Observation 2: If in RAN1 discussion the TRP is capable of DL transmission but it may or may not configure the UE for corresponding reception, then we think such kind of deployment is already covered by the spec then seemingly no dedicated requirement for such scenario is needed.

Proposal 1: Suggest to only specify single layer precoders for 3Tx ULFPTx mode 0 verification.

Proposal 2: For Rel-19 RAN4 3Tx discussion under Rel-19 NR_MIMO_Ph5 WI, suggest to preclude PC1.5 at least for handheld UE.

- References

- R4-2417116, “WF on UE RF requirements for Rel-19 MIMO enhancement”, Samsung, 3GPP TSG RAN4 Meeting #112bis, Hefei, China, October 14-18, 2024.