3GPP TSG-RAN WG4 Meeting #113                                         R4-2419034

Orlando, US, November 18 - 22, 2024

Source: 	Huawei, HiSilicon

Title: 	On potentially new requirements for SBFD

Agenda Item:	7.20.2.1

Document for:	Approval

Introduction

During RAN4#112bis, potentially new requirements for SBFD have been discussed. Following conclusions and FFS points are reproduced from [1].

##### Transient period requirement applicability on frequency/antenna Way Forward: - FFS the text proposal for the clause of transmitter transient period for SBFD, e.g., the example provided in R4-2416498. ##### Location of transient period between SBFD and non-SBFD Way Forward: - FFS location of transient period between SBFD and non-SBFD: - Option 1: No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not. - Option 2: Place the transient period outside of both SBFD and non-SBFD slots. - Option 3: The location of the transient period between SBFD and non-SBFD shall be within the SBFD symbol). - In-channel adjacent subband leakage ratio requirements ##### Necessity of in-channel adjacent subband leakage ratio requirements [Background] In RAN4#112, the following WF is approved: - Way forward: - FFS the necessity of in-channel adjacent subband leakage ratio requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband leakage ratio. - Option 2: New requirement for in-channel adjacent subband leakage ratio should be defined. - Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.. Way Forward: - FFS based on the way forward approved in RAN4#112. - In-channel adjacent subband selectivity/blocking requirements ##### Necessity of in-channel adjacent subband selectivity/blocking requirements [Background] In RAN4#112, the following WF is approved: - Way forward: - FFS the necessity of in-channel adjacent subband selectivity requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband selectivity. - Option 2: New requirement for in-channel adjacent subband selectivity should be defined. - Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline. - Way forward: - FFS the necessity of in-channel adjacent subband blocking requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband blocking. - Option 2: New requirement for in-channel adjacent subband blocking should be defined.. Way Forward: - FFS based on the way forward approved in RAN4#112. - Consider the possibility of specifying RX requirement to cover both in-channel adjacent subband selectivity/blocking requirements

In this contribution, we would like to share our views regarding this topic.

Discussion

The requirement for transient period for SBFD

In previous meeting, a conclusion on this issue was achieved. While there will be transient period between SBFD and non-SBFD slot, it is no need to specify transmitter transient period requirement between different SBFD configurations. The remaining issue is to determine where the transient period will be. Our preference is to select SBFD slot to accommodate the transient period for the necessary RF resource rearrangement, so the TDD slot can be intact.

Proposal 1: The location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol.

In-channel adjacent subband requirement for SBFD

On one hand, we do share the understanding that in-channel adjacent subband requirement could be needed in order to guarantee certain performance level under self-interference. On the other hand, the potential solutions to overcome that would highly depend on vendors but very hard to be reflected in this new requirement since it must be based on specific implementation assumptions. Somehow, the REFSENS requirement for SBFD will reflect the expected suppression on the self-interference from feasible solutions.

Observation 1: For in-channel adjacent subband leakage ratio derivation, it is difficult for RAN4 to agree on a reference scheme for self-interference suppression. While the REFSENS for SBFD will reflect such suppression to some extent.

We also have similar feelings for other intra-operator gNB-gNB interference e.g. inter-site or co-site inter-sector cases. It could be hard to align companies understanding regarding power level, interference handling solutions or other issues that will also be involved not only in requirements derivation but also in test setup.

Proposal 2: Prefer not to introduce new requirement for in-channel adjacent subband leakage ratio.

Proposal 3: Prefer not to introduce new requirement for in-channel adjacent subband selectivity or blocking.

On the potential RAN4 impact for UL resource muting

According to the WID [2], the potential transmit signal quality/MPR requirement impact due to UL resource muting will be checked. Details on this CLI handling solution are reproduced from WID as follows.

To our understanding, this technique would have no impact to the existing transmit quality/MPR requirement. The reason is comb pattern is nothing new from Rel-15. For instance, DMRS multiplexing is specified but there is no dedicated minimum RF requirement for it so that doesn’t mean a UE can meet the aforementioned RF requirements may fail the very same requirements when transmission in comb pattern is scheduled. Further, if the total transmission power remain the same, which is already supported by 3dB power boosting as specified in the spec, there is no performance loss can be expected, either. However, we can provide simulation results based on further RAN1 inputs on the simulation assumptions if possible.

Observation 2: For UL resource muting, there is no transmit quality or MPR requirement impact can be identified for PUSCH as the spec is already support DMRS multiplexing as the data could be mapped according to comb-2 pattern. Further measurements or simulation can be conducted if RAN1 inputs on necessary assumptions is possible.

Conclusion

In this contribution we discussed on the general aspects for SBFD. We have the following observation and proposals:

Observation 1: For in-channel adjacent subband leakage ratio derivation, it is difficult for RAN4 to agree on a reference scheme for self-interference suppression. While the REFSENS for SBFD will reflect such suppression to some extent.

Observation 2: For UL resource muting, there is no transmit quality or MPR requirement impact can be identified for PUSCH as the spec is already support DMRS multiplexing as the data could be mapped according to comb-2 pattern. Further measurements or simulation can be conducted if RAN1 inputs on necessary assumptions is possible.

Proposal 1: The location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol.

Proposal 2: Prefer not to introduce new requirement for in-channel adjacent subband leakage ratio.

Proposal 3: Prefer not to introduce new requirement for in-channel adjacent subband selectivity or blocking.

- References

- R4-2416584, “Way Forward for [112bis][306] NR_duplex_evo_General”, Samsung, 3GPP TSG-RAN4 Meeting #112bis, Hefei, China, October 14-18, 2024.

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, 3GPP TSG RAN Meeting #104, Shanghai, China, June 17-20, 2024.