# 3GPP TSG-RAN WG4 Meeting # 113	R4-2419037

Maastricht, Netherland, August 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on MSD for CA_n40A-n41C with intra-band UL CA_n41C

Agenda Item:	6.3.3

Document for:	Approval

Introduction

In the RAN4#112bis meeting, RAN4 reached the way forward [1] on how to specify the MSD for CA_n40A-n41C with intra-band UL CA_n41C with some options.

Way forward on MSD for DL_n40A-n41C_UL_n41C: Proposal 3: For the MSD value DL_n40A-n41C_UL_n41C, the following inputs from companies can be further checked considering the assumption of filter performance and PA interference. - 3.1) 1RB+1RB test case 1: UL band; DL band; UL BW; SCS of UL band; UL RB Allocation; DL BW; MSD; UL/DL fc condition; UL/DL harmonic order. (MHz); (kHz); LCRB; (MHz); (dB). CA_n40-n41; n40; N/A; 5; N/A; 2397.5; 42.5; TDD; IMD3x. n41; 2555; 80; 1 (RBSTART=0); 2555; N/A; TDD; N/A. 2635; 80; 1 (RBSTART=107); 2635; N/A; TDD; N/A. Note x: this DL band is also subject to IMD5 that is not specified Note y: this second test point is optional. - 3.2) For fully UL RB allocation test case 2, RAN4 further discuss whether MSD due to IMD3 or ACLR1 cross band isolation is considered for DL_n40A-n41C_UL_n41C. - Option 1: UL band; DL band; UL BW; SCS of UL band; UL RB Allocation; DL BW; MSD; UL/DL fc condition; UL/DL harmonic order. (MHz); (kHz); LCRB; (MHz); (dB). CA_n40-n41; n40; N/A; 5; N/A; 2397.5; 34.9; TDD; IMD3xy. n41; 2555; 80; 216 (RBSTART=1); 2555; N/A; TDD; N/A. 2635; 80; 216 (RBSTART=0); 2635; N/A; TDD; N/A. Note x: this DL band is also subject to IMD5 that is not specified Note y: this second test point is optional. - Option 2: UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). CA_n41C; n40; 2595; 190; 30; 270 (RBstart=0) + 243 ((RBstart=0); 2397.5; 5; 34; ACLR1. - Option 3: The cross band isolation MSD caused by fully allocated maximum aggregated BW of UL CA_n41C falls into n40 is 31.5dB and 21.7dB for CA ACLR1 and CA ACLR2, respectively. - Other proposal and value are also welcome. NOTE: In the next meeting, RAN4 can further evaluate the MSD values based on the following channel bandwidth configurations: Band n41: one 100MHz BW CC at 2565MHz centre frequency and another one 60MHz BW CC at 2645MHz centre frequency with 30kHz SCS. Band n40: one 5MHz BW CC at 2397.5MHz centre frequency.

In the previous meeting, we provided the contribution [2] to evaluate the MSD value for CA_n40A-n41C with fully allocated RBs. In this paper, we’d like to share our views on the MSD test definition with the fully allocated maximum aggregated BW.

Discussion

From technical perspective, the MSD with fully allocated maximum aggregated BW for intra-band UL CA is similar to the traditional MSD due to the cross band isolation for single UL carrier. Thus, it’s inappropriate to introduce such MSD test configuration into the IMD MSD table.

Proposal 1: The following MSD test configuration due to cross band isolation for DL_n40A-n41C_UL_n41C can be introduced into the spec.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| CA_n41C | n40A | 2565 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 34 | ACLR1 |
|  |  | 2645 | 60 | 30 | 160 (RBstart=0) |  |  |  |  |

Summary

Proposal 1: The following MSD test configuration due to cross band isolation for DL_n40A-n41C_UL_n41C can be introduced into the spec.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| CA_n41C | n40A | 2565 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 34 | ACLR1 |
|  |  | 2645 | 60 | 30 | 160 (RBstart=0) |  |  |  |  |

- References

[1] R4-2417181, WF on MSD requirements with intra-band contiguous UL CA_n40A-n41C and CA_n41C-n79A, Huawei, HiSilicon

[2] R4-2416029, Discussion on MSD for CA_n40A-n41C with intra-band UL CA_n41C, Huawei, HiSilicon

- Annex

For CA_n40A-n41C with UL intra-band CA_n41C, if RAN4 choose the fully allocated maximum aggregated BW, the DL band n40 will be affected by the first adjacent channel leakage of UL intra-band CA_n41C. The PA non-linearity in 1st adjacent channel leakage (30dB) and band n41 Tx filter performance at DL frequency range of band n40 will dominate the REFSENS degradation. As RAN4 has specified the MSD due to cross band isolation from ACLR2 for the fallback CA_n40A-n41A, RAN4 can consider the similar method and discuss whether to specify the MSD due to cross band isolation from ACLR1 for CA_n40A-n41C with UL intra-band CA_n41C.

Generally, separate antenna RF architecture is used for DL_n40A-n41C_UL_n41C as shown below.

Figure 1 separate antenna RF architecture for DL_n40A-n41C_UL_n41C

And the MSD calculation for CA_n40A-n41C cross band isolation is shown below. The PA noise PSD at PA output port is -29.79dBm/MHz for the first adjacent channel region. Band n40 filter rejection at 2515~2675MHz is assumed as 30dB. And n41 Tx filter rejection at 2300~2400MHz is assumed as 20 / 30 dB. 10dB antenna isolation is assumed. It can be found that there is no MSD for full RB allocation of UL n41C.

table 1 MSD calculation for 20dB n41 Tx filter rejection at 2300~2400MHz

|  | parameters | n40 main path | n40 diversity path |
| --- | --- | --- | --- |
| transmit power for n41C, dBm |  | 23 | 23 |
| Tx band n41C BW(MHz, Lcrb) | 190 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n41C uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| n40 receival singal at ANT port, dBm |  | 13 | 13 |
| n40 filter rejection at 2515~2675MHz, dB | 30 |  |  |
| signal After n40 filter, dBm |  | -21 | -21 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -92 | -92 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -88 | -88 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n40 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -98.47 | -98.47 |
| ACLR1, dB | 30 |  |  |
| ACLR2, dB | 43 |  |  |
| n41 PA leakage PSD at PA output port at 1st adjacent channel, dBm/MHz |  | -29.79 |  |
| n41 PA noise PSD at PA output port, dBm/RxBW |  | -23.26 |  |
| n41 Tx filter rejection at 2300~2400MHz, dB | 20 |  |  |
| n41 PA noise power at Rx n105 ant port at 2300~2400MHz, dBm |  | -53.26 | -53.26 |
| Total noise level at ANT port |  | -53.25 | -53.25 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -54.25 | -54.25 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(5MHz BW), dBm | -55.26 |  |  |
| MSD | 44.74 |  |  |

table 2 MSD calculation for 30dB n41 Tx filter rejection at 2300~2400MHz

|  | parameters | n40 main path | n40 diversity path |
| --- | --- | --- | --- |
| transmit power for n41C, dBm |  | 23 | 23 |
| Tx band n41C BW(MHz, Lcrb) | 190 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n41C uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| n40 receival singal at ANT port, dBm |  | 13 | 13 |
| n40 filter rejection at 2515~2675MHz, dB | 30 |  |  |
| signal After n40 filter, dBm |  | -21 | -21 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -92 | -92 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -88 | -88 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n40 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -98.47 | -98.47 |
| ACLR1, dB | 30 |  |  |
| ACLR2, dB | 43 |  |  |
| n41 PA leakage PSD at PA output port at 1st adjacent channel, dBm/MHz |  | -29.79 |  |
| n41 PA noise PSD at PA output port, dBm/RxBW |  | -23.26 |  |
| n41 Tx filter rejection at 2300~2400MHz, dB | 30 |  |  |
| n41 PA noise power at Rx n105 ant port at 2300~2400MHz, dBm |  | -63.26 | -63.26 |
| Total noise level at ANT port |  | -63.24 | -63.24 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -64.24 | -64.24 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(5MHz BW), dBm | -65.25 |  |  |
| MSD | 34.75 |  |  |

If n41 Tx filter rejection at 2300~2400MHz is assumed as 20 ~ 30 dB, the MSD calculation is about 34dB~44dB for DL_n40A-n41C_UL_n41C.