3GPP TSG-RAN WG4 Meeting # 113	R4-2419054

# Orlando, US, 18th – 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0130 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_solutions-Core) CR for TS 38.101-5 to clarify the UE speed application for the real scenario (R17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MediaTek, Spreadtrum Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2024-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 concluded the frequency error requirements including doppler pre-compensation as 0.1ppm based on the assumption that an NR NTN UE is capable of acquiring the exact GNSS location and satellite ephemeris. RAN4 never discuss the frequency error requirements for NTN UE when UE can’t acquire the exact GNSS location and satellite ephemeris. The shorter GNSS update period can help UE acquire the exact GNSS location timely. However, the shorter GNSS update period UEs have to maintain, the higher power consumption UEs have to bear. Based on the calculation, for 10km/h UE speed scenario, the additional frequency error from doppler pre-compensation will be less than 0.01ppm even if 90s GNSS update period is assumed. Additionally, in the most of scenarios for handheld UE communicating with satellite, the UE speed is less than 10km/h. It’s better to clarify the UE speed scenarios for current frequency error requirements, so that UE implementation can balance the GNSS update period, the targeted accuracy of GNSS location, the performance of UL frequency error and UE power consumption. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clarify that current 0.1ppm frequency error requirements for handheld NR NTN UE are only applicable to UE speed less than or equal to 10km/h. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It’s unclear what is the UE speed assumption for NTN UE to maintein the current frequency error requirements. For some extreme scenairos, e.g. 500km/h or 1000km/h, it’s very hard for NTN UE to maintein such requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change for TS 38.101-5>>

6.4.1	Frequency error

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. For NR NTN handheld UE, current frequency error requirements are only applicable to UE speed less than or equal to 10km/h.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift.]

<<End of Change>>