# 3GPP TSG-RAN WG4 Meeting # 113	R4-2419057

# Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 	(NR_NTN_enh-Core) Discussion on potential solution on Doppler shift issues for guard band and transmission bandwidth configuration

Agenda Item:	5.21.1

Document for:	Approval

Introduction

In the RAN4#110 meeting, although the draft big CR [1] was endorsed in RAN4#110 meeting, doppler shift issues were not extensively discussed. Especially, RAN4 didn’t fully discuss the relationship among the guard band and transmission bandwidth configuration under large doppler shift condition. In RAN4#110bis meeting, this issue was raised by one paper [2]. Finally, a WF [3] was approved to further discuss this issue.

- Way forward Further discuss if extra guard-band is required/should be specified when VSAT UE is pre-compensating Doppler.

In the RAN4#112 meeting, one discussion paper [4] was proposed to address this issue. Finally, RAN4 identified this issue and capture some potential solutions to address it in the way forward [5].

- Identified issue Issue: There is a potential risk for NTN UE to comply with the out-of-band emission requirements when the edge RB falls into the guard band due to Doppler pre-compensation or Doppler shift in NGSO deployment. - Way forward Proposal: The following solutions are considered to address the issue if identified that the edge RB fall into the guard band due to the doppler pre-compensation in NGSO deployment. Solution 1: the first RB and the last RB in the Maximum transmission bandwidth configuration can’t be deployed by NGSO SAN when the edge RB fall into the guard band due to the doppler shift pre-compensation. Solution 2: UE can report a static capability whether UE is capable of supporting the deployment in NGSO scenario that the edge RB fall into the guard band. Solution 3: UE can indicate a dynamic information to network whether the edge RB can be deployed or not based on whether the doppler shift is negligible or not. Solution 4: A guard band at least equivalent to maximum doppler shift expected the NGSO constellation between the channel edge of UE channel bandwidth and spectrum block edge is accounted for as a part of system deployment configuration by the operator. NOTE 1: other solutions are not precluded.

In this paper, we’d like to trade-off these potential solutions for this issue.

Re-discussion on this issue

Doppler shift issues were discussed for FR1-NTN in Rel-18, which was raised by RAN5 LS [6]. In the end, the following testing configuration related to satellite access in table 1 were captured into the TS 38.101-5 for FR1-NTN UE. However, there is no discussion on this issue for Ka band VSAT. For NGSO scenario, the doppler shift could be up to 24ppm especially for LEO scenario. For 30GHz UL frequency, the doppler shift is about 720kHz. One RB could be shifted into the guard for 60kHz SCS when VSAT is handling the doppler pre-compensation as figure 1.

Figure 1 Edge RB falling into Guard band due to doppler shift

One solution is to follow the approach for FR1-NTN UE. All requirements for transmitter and receiver characteristics, other than frequency error, shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.

However, in the real deployment scenario, the doppler conditions are not zero especially for LEO scenario. It’s unclear how to specify or reuse current RF requirements when Edge RB falling into Guard band due to doppler shift.

Observation 1: The issue is identified that one RB could be shifted into the guard for 60kHz SCS when VSAT is handling the doppler pre-compensation.

Currently, the Out-of-Band Emission requirements specified in clause 6.5.2 and 9.5.2 from TS 38.101-5 are clearly defined that the Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions.

- 6.5.2 Out of band emission - 6.5.2.1 General The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an adjacent channel leakage power ratio. To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

- 9.5.2 Out of Band Emissions - 9.5.2.1 General The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an adjacent channel leakage power ratio. Additional requirements to protect specific bands are also considered. The requirements in clause 9.5.2.2 only apply when both UL and DL of a VSAT UE are configured for single CC operation, and they are of the same bandwidth. All out of band emissions for FR2-NTN are TRP.

This definition is quoted from ITU regulation. If we read the SM.329, the similar definition can be found below.

- 1.3bis Out-of-band domain2 (of an emission): The frequency range, immediately outside the necessary bandwidth but excluding the spurious domain, in which out-of-band emissions generally predominate. NOTE 1 Out-of-band emissions, defined based on their source, occur in the out-of-band domain and, to a lesser extent, in the spurious domain. Spurious emissions likewise may occur in the out-of-band domain as well as in the spurious domain.

Observation 2: Both 3GPP spec and ITU regulation specified the out-of-band domain as the frequency range immediately outside the channel bandwidth.

From NTN UE perspective, if the digital filter is designed based on the spectrum utilization and guard band specified in clause 5.3.2 and 5.3.3 from TS 38.101-5, that means NTN UE can’t meet current out-of-band domain requirements including ACLR and out-of-band emission requirements which are specified immediately outside the channel bandwidth when the edge RB fall into the guard band due to the doppler shift pre-compensation.

Observation 3: based on the spectrum utilization and guard band specified in clause 5.3.2 and 5.3.3 from TS 38.101-5, that means NTN UE can’t meet current out-of-band domain requirements including ACLR and out-of-band emission requirements which are specified immediately outside the channel bandwidth when the edge RB fall into the guard band due to the doppler shift pre-compensation.

Trade-off on the potential solutions

In order to solve this issue, four solutions were proposed in the RAN4#112 meeting.

Solution 1: the first RB and the last RB in the Maximum transmission bandwidth configuration can’t be deployed by NGSO SAN when the edge RB fall into the guard band due to the doppler shift pre-compensation. Solution 2: UE can report a static capability whether UE is capable of supporting the deployment in NGSO scenario that the edge RB fall into the guard band. Solution 3: UE can indicate a dynamic information to network whether the edge RB can be deployed or not based on whether the doppler shift is negligible or not. Solution 4: A guard band at least equivalent to maximum doppler shift expected the NGSO constellation between the channel edge of UE channel bandwidth and spectrum block edge is accounted for as a part of system deployment configuration by the operator. NOTE 1: other solutions are not precluded.

In addition, we also check current LTE 36.102 spec. It seems that we have two clarifications (In clause 6.5B.3.1 and clause 6.5B.3.2) which are conflicted with each other for NB-IoT NTN.

- 6.5B.3.1 General The out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an Adjacent Channel Leakage power Ratio. - 6.5B.3.2 Spectrum emission mask [NOTE2: When the UE is operating in an NGSO deployment, to support coexistence, it is assumed that a guardband at least equivalent to the maximum doppler shift expected for the NGSO constellation between the channel edge of the channel bandwidth operated by the UE and the spectrum block edge has been accounted for as part of system deployment configuration by the operator.]

For the general definition about out-of-band emissions, the region for out-of-band emission requirements are immediately outside the assigned channel bandwidth. However, we also clarify an assumption with square brackets (that means this clarification is not an official RAN4 agreement) that a guardband between channel edge of the channel bandwidth and the spectrum block edge has been accounted for as part of system deployment. These two clarifications are conflicted with each other. UE is unclear and confused what current out-of-band emission requirements are. Apart from that, the ACLR requirements for NB1 and NB2 are quoted below.

- 6.5B.3.4 Adjacent Channel Leakage Ratio for category NB1 and NB2 Adjacent Channel Leakage power Ratio is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The assigned category NB1or NB2 channel power and adjacent channel power are measured with filters and measurement bandwidths specified in Table 6.5B.3.4-1. If the measured adjacent channel power is greater than –50dBm then the category NB1 or NB2 UE ACLR shall be higher than the value specified in Table 6.5B.3.4-1. GSMACLR requirement is intended for protection of GSM system. UTRAACLR requirement is intended for protection of UTRA and E-UTRA systems. Table 6.5B.3.4-1: category NB1 and NB2 UE ACLR requirements GSMACLR; UTRAACLR. ACLR; 20 dB; 37 dB. Adjacent channel center frequency offset from category NB1 or NB2 Channel edge; ±200 kHz; ±2.5 MHz. Adjacent channel measurement bandwidth; 180 kHz; 3.84 MHz. Measurement filter; Rectangular; RRC-filter α=0.22. Category NB1 and NB2 channel measurement bandwidth; 180 kHz; 180 kHz. Category NB1 and NB2 channel Measurement filter; Rectangular; Rectangular.

We think the ACLR requirements are reused for NB-IoT NTN UE, but for LEO scenario, we don’t think NB-IoT NTN UE can meet the ACLR requirements in case some REs fall into the adjacent channel due to the doppler pre-compensation.

Observation 4: for solution 4, without network signaling, UE can’t distinguish whether the requirements about out-of-band emissions are defined immediately outside the assigned channel bandwidth or a guardband at least equivalent to the maximum doppler shift was reserved by the network.

For solution 1 and solution 4, these two solutions are similar from spectrum utilization perspective as illustrated below.

Figure 2 The comparison between solution 1 and solution 4

For solution 1, one edge RB in the transmission bandwidth configuration was reserved from system perspective. However, the additional guard band between CBW and out-of-band region was reserved from system perspective for solution 4. From spectrum utilization perspective, both solution 1 and solution 4 are similar that the system/network have to reserve additional spectrum for doppler shift/pre-compensation in case edge RB falls into the guard band without meeting current requirements.

Observation 5: From spectrum utilization perspective, both solution 1 and solution 4 are similar that the system/network have to reserve additional spectrum for doppler shift/pre-compensation in case edge RB falls into the guard band without meeting current requirements.

For solution 2 and solution 3, these two solutions are slightly different from system perspective for different scenarios as illustrated below.

figure 3 The comparison between solution 2 and solution 3

For solution 2, UE just need to decide whether to report the static capability and network can consider different deployment strategies for the UEs with different capabilities. For solution 3, UE can report the dynamic information to the network based on the actual scenario and network can fully utilize this information to deploy the corresponding resources. From network perspective, both solution 2 and solution 3 can help improve the spectrum utilization comparing to the solution 1 and 4. However, solution 2 seems simpler than the solution 3 even if some UEs with lower capability can’t fully use the edge RB for some scenarios.

Observation 6: From network perspective, both solution 2 and solution 3 can help improve the spectrum utilization comparing to the solution 1 and 4.

If we can only select one solution, we prefer to go with solution 2 that it’s allowed for some UEs to report whether UE is capable of supporting the deployment in NGSO scenario when the edge RB fall into the guard band.

If companies insist to consider solution 4, we can also compromise to introduce both solution 2 and solution 4 with clear network signaling.

Proposal 1: Solution 2 is selected to address this issue. It’s allowed for some UEs to report one capability whether UE is capable of supporting the deployment in NGSO scenario when the edge RB fall into the guard band.

Proposal 2: Send LS to RAN2 and request RAN2 design the corresponding UE capability.

Summary

Observation 1: The issue is identified that one RB could be shifted into the guard for 60kHz SCS when VSAT is handling the doppler pre-compensation.

Observation 2: Both 3GPP spec and ITU regulation specified the out-of-band domain as the frequency range immediately outside the channel bandwidth.

Observation 3: based on the spectrum utilization and guard band specified in clause 5.3.2 and 5.3.3 from TS 38.101-5, that means NTN UE can’t meet current unwanted emission requirements which are specified immediately outside the channel bandwidth when the edge RB fall into the guard band due to the doppler shift pre-compensation.

Observation 4: for solution 4, without network signaling, UE can’t distinguish whether the requirements about out-of-band emissions are defined immediately outside the assigned channel bandwidth or a guardband at least equivalent to the maximum doppler shift was reserved by the network.

Observation 5: From spectrum utilization perspective, both solution 1 and solution 4 are similar that the system/network have to reserve additional spectrum for doppler shift/pre-compensation in case edge RB falls into the guard band without meeting current requirements.

Observation 6: From network perspective, both solution 2 and solution 3 can help improve the spectrum utilization comparing to the solution 1 and 4.

Proposal 1: Solution 2 is selected to address this issue. It’s allowed for some UEs to report one capability whether UE is capable of supporting the deployment in NGSO scenario when the edge RB fall into the guard band.

Proposal 2: Send LS to RAN2 and request RAN2 to design the corresponding UE capability.

- References

[1] R4-2401116, Big CR on TS38.101-5 for UE RF Requirements, Samsung Electronics France SA

[2] R4-2405340, Doppler shift issues for guard band and transmission bandwidth configuration, Huawei, HiSilicon

[3] R4-2405995, Way Forward for [110bis][305] NR_NTN_enh_Part1 Doppler pre-compensation into the guard band, Huawei, Thales

[4] R4-2412949, (NR_NTN_enh-Core) Discussion on potential solution on Doppler shift issues for guard band and transmission bandwidth configuration, Huawei, HiSilicon

[5] R4-2413524, Way Forward for Doppler pre-compensation issue, Huawei, HiSilicon

[6] R5-233672, LS on clarifications for Non-Terrestrial Networks, RAN5#99, May. 2023

2 	The terms “out-of-band domain” and “spurious domain” have been introduced in order to remove some inconsistency now existing between, on one hand, the definitions of the terms “out-of-band emission” and “spurious emission” in RR Article 1 and, on the other hand, the actual use of these terms in RR Appendix 3, as revised by the World Radiocommunication Conference (Istanbul, 2000) (WRC-2000). Out-of-band domain emission and spurious domain emission limits apply, respectively, to all unwanted emissions in the out-of-band and spurious domains.