# 3GPP TSG-RAN WG4 Meeting # 113	R4-2419073

# Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rx requirements for NR NTN HPUE

Agenda Item:	7.8.2.2.2

Document for:	Approval

Introduction

In the RAN4#112 meeting, RAN4 reached the following consensus on the Rx requirements for NR NTN HPUE in the Way forward [1].

Issue 4-1: General considerations - Agreement - Rx RF requirements including Max input level, ACS, blocking characteristics, spurious response, intermodulation characteristics, Rx spurious emissions in existing spec can be applied to other power class. Issue 4-2: RSD for NR-NTN - Agreement - Reference sensitivity degradation (RSD) requirements is needed for NTN HPUE. - Further discuss the following aspects: - Use similar way in TN spec by introducing a new table of RSD compared to the PC3 - Whether default Tx-Rx frequency separation is used for test point

In the RAN4#112bis meeting, RAN4 reached the following consensus on the Rx requirements for NR NTN HPUE in the Way forward [2].

Issue 3-1: RSD form in spec Agreement: - Use similar way in TN spec to derive RSD, if needed. Issue 3-2: RSD requirements for PC2 Agreement: - For n256: [0dB] RSD for PC2 NR-NTN for 1Tx and 2Tx - FFS on the duplexer isolation and PA interference Issue 3-4: REFSENS/RSD testing Agreement: - For PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration with PC3 NR NTN UE as starting point.

In this paper, we’d like to share our views on how to derive the REFSENS degradation for NR NTN HPUE.

Discussion

For FDD PC2 NTN HPUE, since the higher output power is implemented, the PA leakage interference will have more interferences on its own receiver than PC3. In addition, since the duplexer has to tolerate higher voltage to support higher output power, the Tx-Rx isolation need more investigation before evaluating the REFSENS degradation. We’d like to investigate the REFSENS degradation for two exemplary bands.

1) For the band n256 (UL 1980~2010MHz, DL 2170~2200MHz), the band n256 duplexer can be combined together with band n1 (UL 1920~1980MHz, DL 2110~2170MHz) from implementation perspective. As there are 100MHz frequency gap for the worst configuration case (UL 20MHz located in 1990~2010MHz and DL 20MHz located in 2110~2130MHz.), referring to the contribution [3], the PA leakage noise can be assumed as -117.8dBm/Hz. Since the pass bandwidth for the FDD duplexer (n1+n256) is about 90*90MHz, the duplexer isolation for PC2 HPUE need some compromise to balance the performance of insertion loss. Thus, 50~52dB isolation can be considered for band n256.

2) For the band n255 (UL 1626.5~1660.5MHz, DL 1525~1559MHz), the band n255 duplexer may be combined together with other new L-bands, so the pass bandwidth of UL filter can achieve 65MHz, i.e. 1610~1675MHz. As there are only 67.5MHz frequency gap for the worst configuration (UL 20MHz located in 1626.5~1646.5MHz and DL 20MHz located in 1539~1559MHz.), the PA leakage noise can be assumed as -107dBm/Hz referring to the figure 7.4.2.2.10-1 of TR 38.863 [4]. Similar to the FDD duplexer (n1+n256), the 50~52dB isolation can be considered for band n256.

Proposal 1: To assume -117.8dBm/Hz PA leakage interference and 50~52dB duplexer isolation for band n256 PC2 RSD calculation.

Proposal 2: To assume -107dBm/Hz PA leakage interference and 50~52dB duplexer isolation for band n255 PC2 RSD calculation.

The 20MHz MSD calculations for 1Tx and 2Tx in band n256

Table 1 20MHz MSD calculation for 1Tx in band n256

|  | parameters | n256 main path | n256 diversity path |
| --- | --- | --- | --- |
| transmit power for n256, dBm |  | 26 |  |
| Tx band n256 BW(MHz, Lcrb) | 20 |  |  |
| RFFE loss, dB | 4 |  |  |
| Duplexer isolation at n256 uplink freq，dB | 52 |  |  |
| antenna isolation, dB | 10 |  |  |
| noise figure dB | 9.5 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -164.5 |  |  |
| Rx band n256 BW(MHz, NRB) | 18 |  |  |
| Thermal noise, dBm |  | -91.95 | -91.95 |
| n256 PA noise PSD at PA output port, dBm/Hz | -117.8 |  |  |
| n256 PA noise PSD at PA output port, dBm/RxBW |  | -45.25 |  |
| n256 Tx filter rejection at 2170~2200MHz, dB | 50 |  |  |
| n256 PA noise power at Rx n256 ant port at 2170~2200MHz, dBm |  | -95.25 | -105.25 |
| Total noise level at ANT port |  | -90.28 | -91.75 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(20MHz BW) |  | -91.28 | -92.75 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(20MHz BW), dBm | -93.09 |  |  |
| MSD | 0.21 |  |  |

Table 2 20MHz MSD calculation for 2Tx in band n256

|  | parameters | n256 main path | n256 diversity path |
| --- | --- | --- | --- |
| transmit power for n256, dBm |  | 23 | 23 |
| Tx band n256 BW(MHz, Lcrb) | 20 |  |  |
| RFFE loss, dB | 4 |  |  |
| Duplexer isolation at n256 uplink freq，dB | 52 |  |  |
| antenna isolation, dB | 10 |  |  |
| noise figure dB | 9.5 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -164.5 |  |  |
| Rx band n256 BW(MHz, NRB) | 18 |  |  |
| Thermal noise, dBm |  | -91.95 | -91.95 |
| n256 PA noise PSD at PA output port, dBm/Hz | -117.8 |  |  |
| n256 PA noise PSD at PA output port, dBm/RxBW |  | -45.25 | -45.25 |
| n256 Tx filter rejection at 2170~2200MHz, dB | 50 |  |  |
| n256 PA noise power at Rx n256 ant port at 2170~2200MHz, dBm |  | -95.25 | -95.25 |
| Total noise level at ANT port |  | -90.28 | -90.28 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(20MHz BW) |  | -91.28 | -91.28 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(20MHz BW), dBm | -92.29 |  |  |
| MSD | 1.01 |  |  |

Proposal 3: based on our calculation, considering the worst case, 1Tx n256 MSD for PC2 is about 0.2dB and TxD n256 MSD for PC2 is about 1dB.

The 20MHz MSD calculations for 1Tx and 2Tx in band n255

Table 3 20MHz MSD calculation for 1Tx in band n255

|  | parameters | n255 main path | n255 diversity path |
| --- | --- | --- | --- |
| transmit power for n255, dBm |  | 26 |  |
| Tx band n255 BW(MHz, Lcrb) | 20 |  |  |
| RFFE loss, dB | 4 |  |  |
| Duplexer isolation at n255 uplink freq，dB | 52 |  |  |
| antenna isolation, dB | 10 |  |  |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n255 BW(MHz, NRB) | 18 |  |  |
| Thermal noise, dBm |  | -92.45 | -92.45 |
| n255 PA noise PSD at PA output port, dBm/Hz | -107 |  |  |
| n255 PA noise PSD at PA output port, dBm/RxBW |  | -34.45 |  |
| n255 Tx filter rejection at 1525~1559MHz, dB | 50 |  |  |
| n255 PA noise power at Rx n256 ant port at 1525~1559MHz, dBm |  | -84.45 | -94.45 |
| Total noise level at ANT port |  | -83.81 | -90.32 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(20MHz BW) |  | -84.81 | -91.32 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(20MHz BW), dBm | -90.20 |  |  |
| MSD | 3.60 |  |  |

Table 4 20MHz MSD calculation for 2Tx in band n255

|  | parameters | n255 main path | n255 diversity path |
| --- | --- | --- | --- |
| transmit power for n255, dBm |  | 23 | 23 |
| Tx band n255 BW(MHz, Lcrb) | 20 |  |  |
| RFFE loss, dB | 4 |  |  |
| Duplexer isolation at n255 uplink freq，dB | 52 |  |  |
| antenna isolation, dB | 10 |  |  |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n255 BW(MHz, NRB) | 18 |  |  |
| Thermal noise, dBm |  | -92.45 | -92.45 |
| n255 PA noise PSD at PA output port, dBm/Hz | -107 |  |  |
| n255 PA noise PSD at PA output port, dBm/RxBW |  | -34.45 | -34.45 |
| n255 Tx filter rejection at 1525~1559MHz, dB | 50 |  |  |
| n255 PA noise power at Rx n256 ant port at 1525~1559MHz, dBm |  | -84.45 | -84.45 |
| Total noise level at ANT port |  | -83.81 | -83.81 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(20MHz BW) |  | -84.81 | -84.81 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(20MHz BW), dBm | -85.82 |  |  |
| MSD | 7.98 |  |  |

Proposal 4: based on our calculation, considering the worst case, 1Tx n256 MSD for PC2 is about 3.6dB and TxD n256 MSD for PC2 is about 8.0dB.

Summary

Proposal 1: To assume -117.8dBm/Hz PA leakage interference and 50~52dB duplexer isolation for band n256 PC2 RSD calculation.

Proposal 2: To assume -107dBm/Hz PA leakage interference and 50~52dB duplexer isolation for band n255 PC2 RSD calculation.

Proposal 3: based on our calculation, considering the worst case, 1Tx n256 MSD for PC2 is about 0.2dB and TxD n256 MSD for PC2 is about 1dB.

Proposal 4: based on our calculation, considering the worst case, 1Tx n256 MSD for PC2 is about 3.6dB and TxD n256 MSD for PC2 is about 8.0dB.

- References

[1] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung

[2] R4-2417122, WF on co-existence study and HPUE for NR-NTN, Samsung

[3] R4-2204223, n3 PC2 FDD 1TX and 2TX REFSENS, Qualcomm Incorporated

[4] TR 38.863 V17.1.0, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence