3GPP TSG RAN WG4 Meeting #113	R4-2419088

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.24.2.3

Source:	vivo

Title:	Further discussion on Conditional Intra-CU LTM

Document for:	Discussion

- Introduction

As per the working arrangement, the corresponding discussion was triggered in RAN2 and RAN4 parallelly in the last meeting. The initial agreements reached in RAN4#112-bis were captured in the WF [1]. In this contribution, we will further provide our thinking on the definition of delay requirements for supporting conditional Intra-CU LTM.

- Discussion

In the last meeting, the first discussion on conditional intra-CU LTM was triggered in RAN4 and it was initially agreed that the cell switch delay requirements for condition LTM (including subsequent LTM) would be defined and FFS on details of delay components. Meanwhile, on the aspects of the supportive mechanisms, there were some significant agreements reached after RAN2 discussion. The corresponding conclusions are shown below. Next, we will analyze the RRM impacts on requirements for conditional LTM delay based on the following latest progress.

RAN2 agreements on C-LTM - Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. - Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. - Source cell and each candidate cell provides its own execution condition for conditional LTM. - It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. - RACH-less Conditional intra-CU LTM is supported. - RACH based conditional intra-CU LTM is supported. - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM. - Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. - For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach. - The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

<conditional LTM>

The legacy conditional handover delay is defined as follows:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

Tinterrupt = Tprocessing + TIU + T∆ + Tmargin

Basically, given the fundamental similarity between the mechanism of conditional LTM and legacy conditional handover, it is straightforward to use the current delay requirements framework for CHO as a baseline for defining requirements for CLTM cell switch delay. In more detail, the timeline of Conditional LTM cell switch delay requirement can be defined as:

Conditional LTM cell switch delay requirement

DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM-interrupt + TCHO_execution

Proposal 1: RAN4 to use CHO delay requirements framework as a baseline when defining conditional LTM delay requirements, the timeline of CLTM cell delay requirement is defined as

DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM-interrupt + TCHO_execution

For the starting point of conditional LTM delay, as per RAN2 design, the conditional LTM configuration is provided via RRC message, which is similar to CHO procedure. For CHO delay, the starting point is defined as the time when the UE receives a RRC message implying conditional handover. Correspondingly, the starting point of CLTM delay can be described similarly.

Proposal 2: The starting point of conditional LTM delay is the time the UE receives a RRC message implying conditional LTM

Based on RAN2 discussion, both RACH-based and RACH-less based conditional intra-CU LTM are supported. The ending point of CLTM delay needs to include both of them. In our understanding, the ending points of the current LTM cell switch delay can be followed, i.e., the time the UE acquires the first available PRACH occasion transmission on the target cell (for RACH-based) and the time the UE transmits the first uplink transmission on the target cell (for RACH-less based).

Proposal 3: The ending point of conditional LTM delay, respectively

- for RACH-based, it is the time the UE acquires the first available PRACH occasion transmission on the target cell

- for RACH-less based, it is the time the UE transmits the first uplink transmission on the target cell

The delay uncertainty Tevent_DU and measurement delay Tmeasure are related to the event/triggering condition evaluation for CLTM. Based on RAN2 guidance, at least L1-level events are supported as evaluation events. And for the issue of whether to support L3 measurement-based conditional LTM in FR1, it was raised parallelly in RAN4 and the following principle was reached in the last meeting.

Issue 1-1-3: Measurement requirements on conditional LTM < Agreement > - On whether L3 measurement based conditional LTM in FR1 can be supported - RAN4 can further discuss this issue in FR1 and make recommendation to RAN2 if RAN4 consensus can be reached. - As generic procedure for the objective led by RAN2, if any conclusion will be made in RAN2, RAN4 will follow the RAN2 conclusion unless any technical issue is identified.

In our understanding, since there’s been some initial discussion in RAN2 and the discussion will proceed for certain, RAN4 can wait for the final decision from RAN2. Accordingly, the related delay components, i.e., TEvent_DU and Tmeasure also need more RAN2 input. On the design of the evaluation event, RAN2 only initially agreed that Event LTM3-like and LTM5-like are used. More details need to be further determined, e.g., the evaluation of beam-level event is based on a single beam or multiple chosen beams, whether the cell-level (if supported) event and beam-level event can be configured simultaneously. The RAN4 discussion can be initiated after more RAN2 inputs are obtained.

Proposal 4: RAN4 to wait for RAN2 conclusion on whether to support L3 based conditional LTM in FR1

Proposal 5: The definitions of TEvent_DU and Tmeasure for CLTM need further RAN2 inputs

For the interruption part TCLTM-interrupt, it highly depends on further discussion on the following mechanisms.

- UE based TA measurement mechanism

- PDCCH ordered early TA acquisition

- Early candidate TCI State activation/deactivation

After the RAN2 discussion, the above mechanisms introduced in Rel-18 LTM are supported in Rel-19 CLTM as well. The details on how to implement these procedures in CLTM are still under discussion in RAN2.

< PDCCH ordered early TA acquisition >

Unlike the Rel-18 LTM, it is based on UE decision on when to trigger LTM and which target cell in case of conditional LTM and there is no MAC CE command on cell switch. Hence, the UL synchronization procedure supported in Rel-18 cannot be reused. The new alternatives need to be introduced with consideration of the issues which potentially include:

- How does NW determine target cells on which to perform early TA acquisition

- How to deliver TA value to the UE.

These issues need to wait for further discussion of RAN2. In particular, additional delays may be introduced based on new design.

< Early candidate TCI State activation/deactivation >

For DL sync procedure, a similar issue may arise. For conditional LTM, L1 measurement results may not be reported to NW. Therefore, it is not very clear how to determine candidate cells on which to perform TCI state activation/deactivation from NW side. As the alternative, there may be UE involvement to decide when and which to perform TCI state activation/deactivation, e.g., totally decided by UE itself based on L1 measurement. The details will be discussed in RAN2 design.

Proposal 6: For the interruption part of CLTM (TCLTM-interrupt), it highly depends on further discussion on procedure design in RAN2 group

<subsequent conditional LTM>

Issue 1-1-1: RAN4 scope of Conditional LTM < Agreement> - RAN4 to define cell switch delay requirements for conditional LTM - RAN4 to define subsequent conditional LTM requirement unless there is any technical issue identified. - Wait for other WGs input to further discuss detailed RRM requirements.

According to the above conclusion, it was agreed that the delay requirements on subsequent conditional LTM would be defined. As per the RAN2 agreement, for conditional LTM, the execution condition for source cell and candidate cells is derived by itself. From this perspective, it can be observed that conditional CLTM implicitly supports ‘subsequent’ evaluation and execution on candidate cells without reconfiguration and re-initialization of the CLTM preparation from the network.

From delay requirement point of view, compared with conditional LTM, the only difference for subsequent conditional LTM is the starting point. The subsequent conditional LTM starts from the completion of LTM procedure for the previous PCell. Apart from this, the delay requirement on conditional LTM can be reused to define the remaining components.

Observation 1: Conditional LTM implicitly supports ‘subsequent’ evaluation and execution on candidate cells without reconfiguration and re-initialization on the CLTM preparation from the network

Proposal 7: For subsequent conditional LTM, the starting point of the delay (for both RACH-based and RACH-less based) is when UE transmits RRCReconfigurationcomplete message for the previous PCell.

- Summary

Proposal 1: RAN4 to use CHO delay requirements framework as a baseline when defining conditional LTM delay requirements, the timeline of CLTM cell delay requirement is defined as

DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM-interrupt + TCHO_execution

Proposal 2: The starting point of conditional LTM delay is the time the UE receives a RRC message implying conditional LTM

Proposal 3: The ending point of conditional LTM delay, respectively

- for RACH-based, it is the time the UE acquires the first available PRACH occasion transmission on the target cell

- for RACH-less based, it is the time the UE transmits the first uplink transmission on the target cell

Proposal 4: RAN4 to wait for RAN2 conclusion on whether to support L3 based conditional LTM in FR1

Proposal 5: The definitions of TEvent_DU and Tmeasure for CLTM need further RAN2 inputs

Proposal 6: For the interruption part of CLTM (TCLTM-interrupt), it highly depends on further discussion on procedure design in RAN2 group

Observation 1: Conditional LTM implicitly supports ‘subsequent’ evaluation and execution on candidate cells without reconfiguration and re-initialization on the CLTM preparation from the network

Proposal 7: For subsequent conditional LTM, the starting point of the delay (for both RACH-based and RACH-less based) is when UE transmits RRCReconfigurationcomplete message for the previous PCell.

- References

- R4-2416863, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom