# 3GPP TSG-RAN WG4 Meeting #113  		R4-2419094

# Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on maintenance of R18 FR2 SCell activation delay reduction

Agenda Item:	5.11.1

Document for:	Discussion

Introduction

In RAN4 #112, the WF [1] is approved. The following remaining issue is left.

Issue 1-1-3: Uncertainty part for SCell activation requirement with L3 reporting FFS: - If UE sends L1 reports earlier than L3 reports and L3 report is later than TCI activation command, the SCell activation delay requirements in TS38.133 clause 8.3.2 apply for the to-be-activated SCell.

Based on all above information, we provide our views on how to solve this issue.

Discussion

In existing TS 38.133, the following is captured in clause 8.3.17.

8.3.17 SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation The requirements in this clause shall apply for UE supporting l3-MeasUnknownSCellActivation-r18 and reporting valid L3 measurement results after receiving the SCell activation command for unknown SCell. The requirements in this clause shall apply for the UE configured with one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated. Clause 8.3.2 is applied for UE who does not report L3 measurement results after receiving SCell activation command for unknown SCell. The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions. Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where: THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3] Tactivation_time is the SCell activation delay in millisecond. - Tactivation_time is: - 10ms + TL3,report+ THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 7ms + TL3,report+ max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting, Editor’s Note: FFS How to define TL3,report and Tuncertainty_MAC for the case UE reports both L1-RSRP and L3-RSRP, and L1-RSRP is reported before L3-RSRP, and TCI activation command is received before L3-RSRP is reported. If the following conditions are met: … where, TL3, report is delay of acquiring the first available UL resource for L3 reporting from 7ms +THARQ after receiving SCell activation command. - The L3 measurement reporting requirement is defined in clause 9.2.4.4 - UE is ready to report the L3 measurement result no later than 7ms + THARQ ms from receiving the SCell activation command. UE is not required to report the L3 results after 3ms + THARQ+ M ms from receiving the SCell activation command where - For FR1, - M= 2*TSSB + TL1-RSRP,report for UE supporting shortMeasInterval-r18 capability, - Otherwise, M = TSMTC +TSSB + TL1-RSRP,report - For FR2-1, - M= (X1+X2)*TSSB + TL1-RSRP,report for UE supporting reduceForCellDetection and/or reduceForSSB-L1-RSRP-Meas and shortMeasInterval-r18 capability, - M = X1*TSMTC +X2*TSSB + TL1-RSRP,report for UE supporting reduceForCellDetection and/or reduceForSSB-L1-RSRP-Meas without supporting shortMeasInterval-r18 capability, - M = 16*TSSB + TL1-RSRP,report for UE supporting shortMeasInterval-r18 without supporting reduceForCellDetection and reduceForSSB-L1-RSRP-Meas capability, - Otherwise, M = 8*TSMTC +8*TSSB + TL1-RSRP,report Where, X1 and X2 are UE capabilities as reported in reduceForCellDetection and reduceForSSB-L1-RSRP-Meas respectively. TFineTiming is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete available SSB corresponding to the TCI state. Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to - First valid L3-RSRP reporting for unknown case, when UE reports valid L3-RSRP - First valid L1-RSRP reporting for unknown case, when UE does not report L3-RSRP results Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to - First valid L3-RSRP reporting for unknown case, when UE reports valid L3-RSRP - First valid L1-RSRP reporting for unknown case, when UE does not report L3-RSRP results Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to - First valid L3-RSRP reporting for unknown case, when UE reports valid L3-RSRP - First valid L1-RSRP reporting for unknown case, when UE does not report L3-RSRP results TRRC_delay is the RRC procedure delay as specified in TS38.331 [2].

The issue is further illustrated in Fig. 1.

Figure 1  Timeline of R18 unknown SCell activation delay reduction with L3 MR reporting

Firstly, our understanding is that, taking single SCell activation scenario as example, if UE reports L3 measurement results during SCell activation procedure, UE shall follow requirements in clause 8.3.17. The TCI activation received before UE reports L3 results is possible, and in such case L3 report would be useless for SCell activation, but using 8.3.17 as baseline would be better since anyway UE can fulfil requirements for this case.

Secondly, existing TS 38.133 counts TL3, report as the delay of acquiring the first available UL resource for L3 reporting from 7ms +THARQ after receiving SCell activation command. L1 reporting is not considered in this term. Anyway, if UE reports L1-RSRP earlier, UE may fulfil such requirements. Therefore, for simplicity, it is OK to add a note without introducing additional requirements.

Thirdly, the applicability conditions for the 3 uncertainty parts are not clear. If UE reports L3 before the reference point of each signalling, there is no issue for current uncertainty definition. There is issue if UE reports the L3 after referece point of each signalling. Therefore, ‘when UE reports valid L3-RSRP’ or ‘when UE does not report L3-RSRP results’ are not the correct condition, .

Based on above discussion, we have the following proposal:

Proposal 1  If UE reports L1-RSRP earlier than L3 reports, and TCI activation is received before L3 reports, UE follows requirements defined in 8.3.17 or 8.3.18, while the requirements still counts the following:

- TL3,report, which is the delay of acquiring the first available UL resource for L3 reporting from 7ms +THARQ after receiving SCell activation command.

  - Add a note under TL3, report: UE may report L1-RSRP earlier than L3 reports, but not additional requirements are defined’.

- uncertainty between each RRC or MAC CE relative to the L1-RSRP reporting

That means, if UE sends L1 reports earlier than L3 reports, and L3 report is later than TCI activation command, RAN4 define a slightly relaxed requirements, so as not to revise TL3,report, which is quite controversial in previous discussion. According to above, the following revision shall be made to the TS 38.133:

- Add a note under TL3, report, replacing existing editor’s note, saying ‘UE may report L1-RSRP earlier than L3 reports, but not additional requirements are defined’.

- Revise ‘when UE reports valid L3-RSRP’ to ‘if UE reports valid L3-RSRP before receiving TCI activation command’

- Revise ‘when UE does not report valid L3-RSRP’ to ‘if UE reports valid L3-RSRP after receiving TCI activation command’.

The details are captured in our companion CR [2].

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  If UE reports L1-RSRP earlier than L3 reports, and TCI activation is received before L3 reports, UE follows requirements defined in 8.3.17 or 8.3.18, while the requirements still counts the following:

- TL3,report, which is the delay of acquiring the first available UL resource for L3 reporting from 7ms +THARQ after receiving SCell activation command.

  - Add a note under TL3, report: UE may report L1-RSRP earlier than L3 reports, but not additional requirements are defined’.

- uncertainty between each RRC or MAC CE relative to the L1-RSRP reporting

- References

[1]  R4-2414020  WF for NR_RRM_enh3, Apple.  RAN4 #112

[2]  R4-2419095  CR on R18 FR2 SCell activation delay reduction, vivo.  RAN4 #113