3GPP TSG-RAN WG4 Meeting #113															R4-2419111
Orlando, US, 18th – 22nd November, 2024

Source:	Ericsson

Title:	On timing requirements for Ku band system parameters

Agenda item:	7.9.1

Document for:	Approval

# 1	Introduction

One issue in selecting FR1 or FR2 for Ku-band NTN is the fact that mobile VSAT Te_NTN Timing Error Limit for mobile VSAT is served by GSO (TS 38.133 Table 7.1C.2-3) is partly in brackets. Furthermore, the value in brackets breaks the UL uncertainty time budget since we allocate more than half a CP. This was pointed out in Eutelsat Group contribution presented at RAN4#112bis [1].

In this contribution we comment on the Te_NTN value for FR2 in 3GPP rel-18.

# 2	Discussion

The existing  Te_NTN Timing Error Limit table looks like this (TS 38.133 Tables 7.1C.2-2 and 7.1C.2-2):

Table 7.1C.2-2: Te_NTN Timing Error Limit for fixed VSAT is served by GSO and fixed VSAT is served by NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | [10]*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | [10]*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

The issue is that Te_NTN = 10 Ts is a ± requirement. At Satellite (BS) RX one UE can be 10 Ts early and another UE 10 Ts late, but the CP is only 18 Ts for SCS = 120 kHz.

However, a budget, albeit pessimistic, was agreed in principle [2]:

### ... Issue 1-6: Te_NTN for 60kHz and 120kHz Agreement: Companies should provide ‘the exact value of Te_NTN and values assumed for X and Y’ and ‘the analysis result based on the following criterion.’ Otherwise, the values/proposals won’t be captured in the list of options. Tg = 0.5*Tcp – (Td + Tp + Tr + Ta + Tf + Tm): an effective guard period in CP - Tcp: a length of CP for the given SCS of UL channel/signal - Td: UE downlink synchronization error for the given SCS of SSB (BW of PBCH DMRS, i.e. 20 PRBs) - Tp = Tp,ue + Tp,sat: a round trip propagation delay estimation error due to UE position and satellite position estimation errors - Tp,ue: a round trip propagation delay estimation error due to [X]m of UE position error - Tp,sat: a round trip propagation delay estimation error due to [Y]m of satellite position estimation error - Tr: TAC resolution error (from TS38.213) - Ta: TA adjustment accuracy error (from Table 7.3.2.2-1 of TS38.133) - Tf: an accumulated timing drift over 160ms due to a frequency offset of 0.1ppm - Tm: a margin needed at gNB receiver to accommodate any additional impairments if needed. - If a non-zero value is assumed in the proposal for Tm, the source of the impairments shall be provided too. Technical analysis is required if any number will be provided for each of the components in the next meeting. Whether the same or different values for different channels is contribution driven. ...

In our contribution to rel-18 NTN WI we analyse the agreed budget [3].

We want Tg > 0. This means that the budget will fit at UL timing reference point as long as:

0.5*Tcp – (Td + Tp + Tr + Ta + Tf + Tm) > 0

To start the analysis, we allocate all the UL time uncertainty budget to the UE side to maximize the allowed UE position uncertainties, Tp. We get:

Tp < 0.5*Tcp – (Td + Tr + Ta + Tf + Tm)

For SSB SCS ∈ 120, 240 kHz and UL SCS  ∈ 60, 120 kHz we get:

| 120/60 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tcp | Td | Tr | Ta | Tf | Tm | Tp |  | Te |
| 36,00 | 0,53 | 2,00 | 2,00 | 0,49 | 0,00 | 12,98 |  | 14,00 |

| 120/120 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tcp | Td | Tr | Ta | Tf | Tm | Tp |  | Te |
| 18,00 | 0,53 | 1,00 | 0,50 | 0,49 | 0,00 | 6,48 |  | 7,50 |
| 240/60 |  |  |  |  |  |  |  |  |
| Tcp | Td | Tr | Ta | Tf | Tm | Tp |  | Te |
| 36,00 | 0,27 | 2,00 | 2,00 | 0,49 | 0,00 | 13,24 |  | 14,00 |
| 240/120 |  |  |  |  |  |  |  |  |
| Tcp | Td | Tr | Ta | Tf | Tm | Tp |  | Te |
| 18,00 | 0,27 | 1,00 | 0,50 | 0,49 | 0,00 | 6,74 |  | 7,50 |

where we put:

Te = Td + Tf + Tp

that is we include in Te the UE downlink synchronization error for the given SCS of SSB (Td) + an accumulated timing drift over 160ms due to a frequency offset of 0.1ppm (Tf) + a round trip propagation delay estimation error due to UE position and satellite position estimation errors (Tp).

Historically, TA adjustment accuracy error (Ta) has been a separate factor, not included in Te. It is also woth pointing out that Tr, the TAC quantization noise is not part of Te. In TS 38.133 a UE has two inputs for calculating UL timing transmission point, it has the DL time reference (SSB blocks), at least every 160 ms and it has N_TA, a TA value. The UL TX time is a function of the DL time reference (time) minus NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc. The UE ability to track the DL time reference and therefore the Te uncertainty, is not a function of the quantization of the TA control loop. It is a function of DL BW, DL SNIR and drift of UE internal oscillators between SSB blocks.

In [3] Ericsson concluded:

- For UL SCS = 120 kHz, a UE with zero implementation margin when it comes to Te requirement has to have a total round trip error allowance of 6.5 to 6.7 Ts, which corresponds to a total position error of 32 to 33 m to distribute among satellite and UE. The Te value = 7.5 Ts.

- For UL SCS = 60 kHz, a UE with zero implementation margin when it comes to Te requirement has to have a total round trip error allowance of 13 Ts, which corresponds to a total position error of 63 m to distribute among satellite and UE. The Te value = 14 Ts.

In the end this became Te = 7.5 Ts and Te = 13 Ts  for fixed VSAT is served by GSO and fixed VSAT is served by NGSO. For UL SCS = 120 kHz and UL SCS = 60 kHz respectively. For mobile VSAT is served by GSO 3GPP RAN4 agreed on [10] Ts and 13 Ts in NTN release-18 WI.

Observation 1:  Ericsson’s original proposal based on agreed budget in R4-2317374, RRM WF and CR work split for NR_NTN_enh, was Te = 7.5 Ts for UL SCS = 120 kHz.

It is certainly so that the agreed budget in [2] is a pessimistic one insofar that the Tp,sat, round trip propagation delay estimation error due to satellite position estimation error is … <Editor’s note: fix! Need better argument>

Proposal 1: Replace Te = [10] Ts for UL SCS = 120 kHz with Te = 7.5 Ts for mobile VSAT is served by GSO 3GPP in table Table 7.1C.2-3:

Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | [10]7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | [10]7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

# 3 	Summary

Observation 1:  Ericsson’s original proposal based on agreed budget in R4-2317374, RRM WF and CR work split for NR_NTN_enh, was Te = 7.5 Ts for UL SCS = 120 kHz.

Proposal 1: Replace Te = [10] Ts for UL SCS = 120 kHz with Te = 7.5 Ts for mobile VSAT is served by GSO 3GPP in table Table 7.1C.2-3:

Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | [10]7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | [10]7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

# References

[1] 		R4-2415284, Feasibility analysis of FR1 and FR2 Numerology, Eutelsat Group

[2]		R4-2317374, RRM WF and CR work split for NR_NTN_enh, Qualcomm Incorporated

[3]		R4-2318821, RRM requirements in above 10 GHz bands, Ericsson