3GPP TSG-RAN WG4 Meeting # 113	R4-2419112

# Orlando, US, 18th – 22nd November, 2024

Source: 	China Telecom

Title: 	Views on SAR Solution

Agenda Item:	7.1.1.1.1

Document for:	Discussion

Introduction

In meeting RAN4#112-bis [1], we made some progress on duty cycle solution for SAR compliance and agreement can be captured as follows.

- 4.2 Duty cycle solution for SAR compliance Agreement: - For intra-band CA and inter-band CA PC1.5, duty cycle solution for SAR will be discussed. - For EN-DC PC1.5 and PC2, duty cycle solution for SAR will discussed for the scenario without new IE introduction.

So in this paper we reiterate our views and further clarify it.

Discussion

- 2.1 PC1.5 HPUE for intra/inter-band CA

For PC1.5 intra-band CA duty cycle solution, the general framework should refer to PC1.5 single carrier UE. Besides, the UE capability of PC2 single carrier, maxUplinkDutyCycle-PC2-FR1, should be reused for threshold. And the value is 0.5* maxUplinkDutyCycle-PC2-FR1 and 25% for default.

Proposal1: For PC1.5 intra-band CA duty cycle solution, the general framework should refer to PC1.5 single CC and reused the PC2 capability, and the value is 0.5* maxUplinkDutyCycle-PC2-FR1 and 25% for default.

For PC1.5 inter-band CA, our standpoint is unchanged that reusing the PC2 methods and capability maxUplinkDutyCycle-interBandCA-PC2 for threshold and maxUplinkDutyCycle-PC2-FR1 for maxDutyNR,x/y.

The average percentage of uplink symbols should also be defined as 50%×( DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y) since the power class is per band rather than per Tx. And the default value of maxDutyNR,x/y should be 25% according to the baseline of SAR requirements.

Proposal2: For PC1.5 inter-band CA, reuse maxUplinkDutyCycle-interBandCA-PC2 for threshold and maxUplinkDutyCycle-PC2-FR1 for maxDutyNR,x/y. The default value of maxDutyNR,x/y should be 25%.

- 2.1 PC1.5 HPUE for inter-band EN-DC

Since there is no duty cycle solution capability for FDD+FDD scenario in legacy specification and the agreement is not introducing new IE, P-MPR should be the default SAR solution and further study the compromise duty cycle solution in a reasonable and simple way by using TDD+TDD or FDD+TDD signaling.

For example, for PC1.5 FDD+FDD inter-band EN-DC, set two thresholds on the LTE side: t1, t2, and two thresholds on the NR side: x1*maxUplinkDutyCycle-FDD-TDD-EN-DC1 and x2*maxUplinkDutyCycle-FDD-TDD-EN-DC2, which are both refer to FDD+TDD scenario. The values of t1, t2, x1, x2 can be further discussed.

Proposal3: For FDD+FDD inter-band EN-DC, P-MPR should be the default SAR solution and further study the compromise duty cycle solution.

For PC1.5 FDD+TDD and TDD+TDD inter-band EN-DC, the SAR global framework should be both in accordance with PC2 operation and only change the threshold value.

For TDD+TDD scenario, the threshold should be 0.5* maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 and 15% for default.

Proposal4: For TDD+TDD scenario, the threshold should be 0.5* maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 and 15% for default.

For FDD+TDD scenario, the thresholds should be 0.5* maxUplinkDutyCycle-FDD-TDD-EN-DC1 and 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC2.

Proposal5: For FDD+TDD scenario, the thresholds should be 0.5* maxUplinkDutyCycle-FDD-TDD-EN-DC1 and 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC2.

Conclusion

Proposal1: For PC1.5 intra-band CA duty cycle solution, the general framework should refer to PC1.5 single CC and reused the PC2 capability, and the value is 0.5* maxUplinkDutyCycle-PC2-FR1 and 25% for default.

Proposal2: For PC1.5 inter-band CA, reuse maxUplinkDutyCycle-interBandCA-PC2 for threshold and maxUplinkDutyCycle-PC2-FR1 for maxDutyNR,x/y. The default value of maxDutyNR,x/y should be 25%.

Proposal3: For FDD+FDD inter-band EN-DC, P-MPR should be the default SAR solution and further study the compromise duty cycle solution.

Proposal4: For TDD+TDD scenario, the threshold should be 0.5* maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 and 15% for default.

Proposal5: For FDD+TDD scenario, the thresholds should be 0.5* maxUplinkDutyCycle-FDD-TDD-EN-DC1 and 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC2.

References

- R4-2417098, WF on HPUE RF requirements, RAN4#112-bis