3GPP TSG-RAN4 Meeting #113	R4-2419125

# Orlando, US, 18 – 22 November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5165 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on test configurations for less than 5MHz BW |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW-Perf |  |  |  |  |  | Date: |  |  | 2024-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some issues in the CORESET RMC for less than 5MHz. - For CORESET0 RMC, the number of OFDM symbols should be 3 for 15RB and 20RB allocation (CR.1.3 FDD and CR.1.4 FDD) according to Table 8.1.2.1-3. - For dedicated RMC, the aggregation level should be 4 for 12RB allocation (CCR.1.7 FDD) because 8 CCEs cannot be accomodated in 12RB even with 3 symbols, and the number of OFDM symbols should be 3 for 18RB allocation (CCR.1.8 FDD) to support 8 CCEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the CORESET RMC for less than 5MHz. - For CORESET0 RMC, change the number of OFDM symbols to 3 for CR.1.3 FDD and CR.1.4 FDD. - For dedicated RMC, change the aggregation level to 4 for CCR.1.7 FDD, and change the number of OFDM symbols to 3 for CCR.1.8 FDD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CORESET RMC for less than 5MHz are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1.2.1, A.3.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### A.3.1.2.1	FDD

Table A.3.1.2.1-1: RMSI CORESET Reference Channel for FDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.1.1 FDD | CR.1.2 FDD | CR.1.3 FDD | CR.1.4 FDD |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case |  |  |  |
| Subcarrier spacing for RMSI CORESET | kHz | 15 | 15 | 15 | 15 |  |  |  |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 | 12 | 15 | 20 |  |  |  |
| Subcarrier spacing for SSB | kHz | 15 | 15 | 15 | 15 |  |  |  |
| SSB and RMSI CORESET multiplexing configuration Note 7 |  | Pattern 1 | Pattern 1 | Pattern 1 | Pattern 1 |  |  |  |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note8) | 0 (Note8) | 0 (Note8) | 0 (Note8) |  |  |  |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 | Index 4 | Index 4 | Index 4 |  |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 |  |  |  |
| Duration of RMSI CORESET Note 7 | symbols | 2 | 2 | 3 | 3 |  |  |  |
| DCI Format Note 1 |  | Note 2 | Note 2 | Note 2 | Note 2 |  |  |  |
| Aggregation level | CCE | 8 | 4 | 8 | 8 |  |  |  |
| DMRS precoder granularity |  | 6 | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |  |
| REG bundle size |  | 6 | 6 | 6 | 6 |  |  |  |
| Mapping from REG to CCE |  | Distributed | Distributed | Non-Distributed | Distributed |  |  |  |
| Cell ID |  | Note 5 | Note 5 | Note 5 | Note 5 |  |  |  |
| Payload (without CRC) | bits | Note 6 | Note 6 | Note 6 | Note 6 |  |  |  |
| Note 1: DCI formats are defined in TS 38.212. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [3]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-1 in TS 38.213 [3] Note 8: Other values can be used to align with GSCN [13] as long as SSB does not overlap the RMC. |  |  |  |  |  |  |  |  |

<End of Change 1>

<Start of Change 2>

#### A.3.1.3.1	FDD

Table A.3.1.3.1-1: Control Channel RMC for FDD with SCS=15KHz

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 FDD | CCR.1.2 FDD | CCR.1.3 FDD | CCR.1.4 FDD | CCR.1.5 FDD | CCR.1.6 FDD | CCR.1.7 FDD | CCR.1.8 FDD |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case | 10 | 3MHz | 3MHz | 5MHz |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 18 | 24 | 18 | 24 | 12 | 12 | 18 |  |  |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |  |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 |  |  |
| monitoringSymbolsWithinSlot |  | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 | 0010000 0000000 | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 |  |  |
| REG bundle size |  | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Non-Interleaved | Interleaved |  |  |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 | 2 | N/A | 2 |  |  |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |  |  |
| Aggregation level | CCE | 4 | 2 | 8 | 4 | 4 | 4 | 4 | 8 |  |  |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | 1-0 | 1-0 | 1-0 |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.1.3.1-2: Control Channel RMC for FDD with SCS=120KHz

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.2.1 FDD |  |  |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 24 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Duration of CORESET | symbols | 1 |  |  |  |  |  |  |
| monitoringSlotPeriodicityAndOffset Note 4 |  | sl160 0 |  |  |  |  |  |  |
| monitoringSymbolsWithinSlot |  | 1100000 0000000 |  |  |  |  |  |  |
| REG bundle size |  | 6 |  |  |  |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size |  |  |  |  |  |  |
| CCE to REG mapping |  | Interleaved |  |  |  |  |  |  |
| Interleave n_shift |  | 0 |  |  |  |  |  |  |
| Interleave size |  | 2 |  |  |  |  |  |  |
| Beamforming Pre-Coder |  | N/A |  |  |  |  |  |  |
| Aggregation level | CCE | 4 |  |  |  |  |  |  |
| DCI formats |  | Note 1 |  |  |  |  |  |  |
| Payload size (without CRC) | bits | Note 2 |  |  |  |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the resource blocks where the associated RMC is scheduled. Note 4: monitoringSlotPeriodicityAndOffet is set to “sl1 0” if it is specifically stated that cell(s) configured with one of the control channel RMCs above shall transmit PDCCHs continuously. |  |  |  |  |  |  |  |  |

<End of Change 2>