3GPP TSG-RAN WG4 Meeting #113	R4-2419144

Orlando, US, 18 – 22 November, 2024

Title: 	Discussion on RRM requirements for less than 5MHz for NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.8.3.4

Document for:	Discussion

Introduction

RRM requirements for less than 5MHz in NTN are discussed in RAN4#112-bis, and the outcomes are captured in WF [1]. Based on the discussion, most issues were resolved, and a few needs to be further discussed.

- Intra-band CA

- PSCell related requirements

- Requirement applicability

In this paper, we will provide our views on RRM requirements for less than 5MHz in NTN.

Discussion

#### Issue 1-2-1-2 CHO delay Agreements/ Way-forward: - for CHO without L3 measurement: - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs] - FFS on for CHO with L3 measurement: - Tmeasure which is same as the definition for Tidentify_intra_with_index and Tidentify_inter_with_index in issue 1-2-4-2 will be extended - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs]

For CHO with L3 measurement, we support to extend T∆ from 1 Trs to 3 Trs. Even UE has measured the target cell and may have also read the SSB index of the target cell, during HO execution, UE may perform SSB index verification as part of time tracking. Therefore, T∆, which is accounting for the delay for time tracking, should be extended to 3 Trs due to PBCH puncturing. This is also aligned with TN requirements.

Proposal 1: For CHO with L3 measurement, T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to 3 Trs.

#### Issue 1-2-4-2: How to define the new requirements for SSB time index detection Agreements/Wayforward - The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) will be extended by X and Y more samples for intra-frequency and inter-frequency cases respectively. - For X and Y, the values from TN less than 5MHz as starting point.

We support to adopt X=4 and Y=3, which is same as TN. In our view, the performance impact of PBCH puncturing is same for TN and NTN. With same SINR condition, the extension should be same as TN.

Proposal 2: For SSB index reading, extend the delay requirements by X=4 and Y=3 SSB samples.

Conclusions

In this paper we provided provide our views on RRM requirements for less than 5MHz in NTN.

Proposal 1: For CHO with L3 measurement, T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to 3 Trs.

Proposal 2: For SSB index reading, extend the delay requirements by X=4 and Y=3 SSB samples.

Reference

- R4-2416851, WF on RRM requirements for [112bis][207] NR_IoT_NTN_req_test_enh, Moderator (Xiaomi)