3GPP TSG-RAN WG4 Meeting #113	R4-2419147

Orlando, US, 18 – 22 November, 2024

Title: 	Discussion on UE CLI measurement for SBFD

Source: 	Huawei, HiSilicon

Agenda item:	7.20.3.1

Document for:	Discussion

Introduction

RRM requirements for SBFD are discussed in RAN4#112-bis, and the outcomes are captured in WF [1]. Based on the discussion, following issues needs to be further discussed for UE CLI measurement.

- Measurement methods

- Rx beam

- Measurement resources

- Measurement period

- Measurement reporting

- Scheduling and measurement restriction

- Requirement applicability

In this paper, we will provide our views on RRM requirements for UE CLI measurement in SBFD.

Discussion

Measurement methods

In RAN1#118, the following was agreed regarding measurement method.

Agreement For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband

In RAN1#118-bis, there was no further agreement on whether to support method#3 or not, but RAN1 agreed the following on the measurement timing for method #1.

Agreement - For L1 CLI-RSSI measurement, DL timing is used for Method #1 (UE measures RSSI within DL subband).

On the other hand, RAN4#112 agreed on the measurement timing for method #2.

Issue 1-1-14: Measurement timing for L1-SRS-RSRP - Agreements (from online session) - As baseline, for L1 SRS-RSRP measurement, UE will apply a constant offset relative to the downlink reference timing in the serving cell, and the constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset. - This can be revisited if it proves to be conflicting with further RAN1 agreements.

Now that the measurement behaviour for method #1 and #2 are clear, we suggest RAN4 to focus on the two agreed method first, and method #3 can be further considered if it is supported by RAN1.

Proposal 1: RAN4 to define L1 UE-to-UE CLI measurement requirements at least for

- Method#1: UE measures RSSI within DL subband, and

- Method#2: UE measures RSRP of aggressor UE within UL subband

Other methods can be considered based on further RAN1 agreements.

Rx beam

In RAN1#118-bis, the following was agreed regarding the TCI state configuration for CLI measurement.

Agreement For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH, support the following in the IE CSI-AperiodicTriggerStateList: - In CSI-AssociatedReportConfigInfo a list of TCI state(s) with qcl-Type set to 'typeD' is optionally configured where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo - The list of TCI states can be configured (present) only if unifiedTCI-StateType is configured and the CLI measurement resource(s) are aperiodic - If the list of TCI states is not configured (absent), the UE assumes the following for each CLI measurement resource: - If unifiedTCI-StateType is not configured - Same assumption as for L3-based CLI measurement as in 38.133: “For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET” - If unifiedTCI-StateType is configured - “Indicated” DL only/joint TCI state as specified in TS 38.214 - Note: Above default rules apply to aperiodic reporting based on aperiodic, semi-persistent, and periodic CLI measurement resources FFS: details of configuration of TCI state(s) for aperiodic reporting based on periodic and semi-persistent CLI measurement resources

It can be seen that TCI can be optionally configured for the CLI measurement resource in the measurement configuration (it was already agreed how to configure it for aperiodic resource, and how to configure it for periodic and semi-persistent resource is FFS). If TCI is not configured for a resource, then a default TCI can be found (the exact approach depends on whether unified TCI is configured or not).

We understand in both cases, Rx beam sweeping is not needed for the measurement in FR2, as UE already knows which Rx beam to use for the measurement thus no need to try different Rx beams. We suggest RAN4 to confirm that no Rx beam sweeping is assumed in measurement period requirements.

Proposal 2: RAN4 to confirm no Rx beam sweeping is assumed in measurement period requirements for L1 based UE-to-UE CLI measurement.

Measurement resources

In RAN1#118 and #118-bis, following were agreed regarding resource and report of UE CLI measurement.

Agreement For L1 UE-to-UE CLI measurement and reporting, the following are supported - Wideband CLI-RSRP reporting - Wideband CLI-RSSI reporting - FFS: Subband CLI-RSSI reporting Agreement For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands - FFS: Number of resources that can be configured - FFS: UE behavior for measurement Agreement For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs. - A single wideband RSSI measurement report is supported - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband.

We do not see any issue to be discussed for L1-SRSR-RSRP since there is only a single option for resource configuration (there is a single UL subband) and reporting configuration (only wideband reporting for L1-SRSR-RSRP).

For L1-CLI-RSSI, some companies mentioned that the requirements may be impacted by the resource configuration (there are 2 resource types) or reporting configuration (wideband reporting is supported, and FFS per-DL-subband wideband reporting for resource type #2 and FFS subband reporting).

In our view, RSSI measurement simply reflects the total received power on the indicated time and frequency resources, and the measurement performance (delay or accuracy) does not depend on the RB of measurement resource or measurement reporting. Therefore, we suggest RAN4 to confirm that measurement delay and accuracy requirements for L1-CLI-RSSI are agnostic to measurement resource or reporting configuration.

Proposal 3: RAN4 to confirm that measurement delay and accuracy requirements for L1-CLI-RSSI are agnostic to measurement resource or reporting configuration.

Measurement period

For L1-SRS-RSRP, RAN4 has sent LS to RAN1 to clarify the scenarios to be considered. Also, RAN4 is evaluating the measurement performance under different side conditions (timing error and Es/Iot) with different sample numbers. The measurement period of L1-SRS-RSRP can be discussed after RAN4 gets feedback from RAN1 and based on simulation results.

For L1-CLI-RSSI, the measurement period should be based on single shot. The measurement does not require filtering among multiple shots, and so far, all the RSSI related measurement, including L3 CLI-RSSI (clause 9.7.3.5), RSSI measurement in NR-U (clause 9.2A.7.1 and 9.3A.8) and SL-RSSI (clause 12.6) are all based on single shot. We do not see technical difference in L1-CLI-RSSI.

Proposal 4: Measurement period for L1-CLI-RSSI is defined based on single shot.

Measurement reporting

In WID it is clarified that aperiodic reporting will be supported for L1 CLI measurement, so RAN4 should define corresponding measurement reporting requirements. In our view, the reporting requirements for L1-RSRP in clause 9.5.3.3 can be used as baseline since both are L1 measurements.

#### 9.5.3.3 Aperiodic Reporting Reported L1-RSRP measurements contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively. The UE shall only send aperiodic L1-RSRP measurement reports, if a DCI trigger has been received. After the UE receives CSI request in DCI, the UE shall transmit the aperiodic L1-RSRP reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

RAN4 can further discuss the reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement.

Proposal 5: On measurement reporting requirements,

- RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting. L1-RSRP measurement reporting requirement can be re-used.

- FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement.

Scheduling and measurement restriction

In R16, scheduling restriction are defined for both SRS-RSRP and CLI-RSSI assuming UE may use a different (earlier) timing than DL timing.

- For L1-SRS-RSRP, RAN4 agreed to re-use the measurement timing as R16, so similar requirements as for SRS-RSRP may be defined. As the timing error is under discussion, RAN4 may discuss the exact number of restricted symbols later.

- For L1-CLI-RSSI, RAN1 agreed to use DL timing for measurement. In this case, scheduling restriction on DL would be needed only for Rx beam, i.e. only when the measurement resource is not type-D QCL-ed with the PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI.

Proposal 6: RAN4 to define scheduling restriction for L1 based UE-to-UE CLI measurement

- For L1-SRS-RSRP, R16 requirements are re-used as baseline, and exact number of restricted symbols is FFS.

- For L1-CLI-RSSI, scheduling restriction applies in OFDM symbols on which the UE performs the measurements. For DL, scheduling restriction applies when the measurement resource is not type-D QCL-ed with the DL reception in FR2.

On measurement restriction, in R16 SRS-RSRP and CLI-RSSI measurements are assumed to not overlap with other L3 or L1 measurement, otherwise no requirements for SRS-RSRP and CLI-RSSI apply.

#### 9.7.2.5 SRS-RSRP measurement period If the SRS resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TSRS_RSRP_measurement_period. #### 9.7.3.5 CLI-RSSI measurement period If the CLI-RSSI measurement resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TCLI_RSSI_measurement_period.

In our view, the R16 principle can be re-used, but some adaptations may be needed. As discussed above, L1-CLI-RSSI is based on DL timing, so it does not necessarily need to be separated from other measurement if they can be done at same time.

Proposal 7: RAN4 to define measurement restriction for L1 based UE-to-UE CLI measurement

- For L1-SRS-RSRP, R16 requirements are re-used as baseline.

- For L1-CLI-RSSI, no measurement requirements apply if the L1-CLI-RSSI measurement resources are overlapping with

  - Measurement gaps in FR1 or FR2, or

  - SMTC window or SSB in FR2, or

  - CSI-RS in FR2 when the measurement resource is not type-D QCL-ed with the DL reception.

Requirement applicability

In R16, following applicability is defined for SRS-RSRP and CLI-RSSI.

The UE capable of performing CLI measurements shall be able to measure SRS-RSRP and CLI-RSSI which are defined in TS38.215 [4] within active DL BWP. The measurements requirements in this clause apply for TDD mode only. CLI measurements are only applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resouce is configured within active BWP

For L1-SRS-RSRP and L1-CLI-RSSI, RAN1#118 agreed following.

Agreement For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband FFS: Method#3: UE measures RSSI within UL subband

Given the RAN1 agreement, R16 applicability can be directly re-used.

Proposal 8: L1 based UE-to-UE CLI measurement requirements apply provided that

- when L1-SRS-RSRP measurement resource is fully confined within BW of DL active BWP

- when L1-CLI-RSSI measurement resource is configured within BW of DL active BWP

Conclusions

In this paper we provided our views on RRM requirements for UE CLI measurement in SBFD.

Proposal 1: RAN4 to define L1 UE-to-UE CLI measurement requirements at least for

- Method#1: UE measures RSSI within DL subband, and

- Method#2: UE measures RSRP of aggressor UE within UL subband

Other methods can be considered based on further RAN1 agreements.

Proposal 2: RAN4 to confirm no Rx beam sweeping is assumed in measurement period requirements for L1 based UE-to-UE CLI measurement.

Proposal 3: RAN4 to confirm that measurement delay and accuracy requirements for L1-CLI-RSSI are agnostic to measurement resource or reporting configuration.

Proposal 4: Measurement period for L1-CLI-RSSI is defined based on single shot.

Proposal 5: On measurement reporting requirements,

- RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting. L1-RSRP measurement reporting requirement can be re-used.

- FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement.

Proposal 6: RAN4 to define scheduling restriction for L1 based UE-to-UE CLI measurement

- For L1-SRS-RSRP, R16 requirements are re-used as baseline, and exact number of restricted symbols is FFS.

- For L1-CLI-RSSI, scheduling restriction applies in OFDM symbols on which the UE performs the measurements. For DL, scheduling restriction applies when the measurement resource is not type-D QCL-ed with the DL reception in FR2.

Proposal 7: RAN4 to define measurement restriction for L1 based UE-to-UE CLI measurement

- For L1-SRS-RSRP, R16 requirements are re-used as baseline.

- For L1-CLI-RSSI, no measurement requirements apply if the L1-CLI-RSSI measurement resources are overlapping with

  - Measurement gaps in FR1 or FR2, or

  - SMTC window or SSB in FR2, or

  - CSI-RS in FR2 when the measurement resource is not type-D QCL-ed with the DL reception.

Proposal 8: L1 based UE-to-UE CLI measurement requirements apply provided that

- when L1-SRS-RSRP measurement resource is fully confined within BW of DL active BWP

- when L1-CLI-RSSI measurement resource is configured within BW of DL active BWP

Reference

- R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon