3GPP TSG-RAN WG4 Meeting #113	R4-2419148

Orlando, US, 18 – 22 November, 2024

Title: 	Initial simulation results for UE CLI measurement

Source: 	Huawei, HiSilicon

Agenda item:	7.20.3.1

Document for:	Discussion

Introduction

RRM requirements for SBFD are discussed in RAN4#112-bis, and the outcomes are captured in WF [1]. Based on the discussion, RAN4 needs to evaluate the measurement performance of L1-SRS-RSRP, with different sample number, different timing error condition and different Es/Iot. For this purpose, link level simulation assumption is approved in [2].

In this paper, we will provide our initial simulation results for L1-SRS-RSRP.

Discussion

In this section, we will provide simulation results for the following configurations in [2].

- Number of SRS symbols: 1

- Number of SRS RBs: 48

- SRS SNR: 1dB

Case 1

The measurement performance for Case 1 with 1 and 3 samples are shown in Table 1-3 for different channel conditions. For AWGN channel, the R16 requirements, assuming 2.5dB RF margin, are listed for reference.

Table 1: L1-SRS-RSRP performance for Case 1 in AWGN

| Sample number | SCS (kHz) | L1-SRS-RSRP error (dB) |  |  | Required accuracy (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% |  |
| 1 | 15 | -0.98 | -0.46 | 0.07 |  |
|  | 30 | -1.66 | -1.09 | -0.50 |  |
|  | 60 | -3.28 | -2.51 | -1.76 |  |
|  | 60 | -2.59 | -1.86 | -1.22 |  |
|  | 120 | -6.01 | -4.71 | -3.60 |  |
| 3 | 15 | -0.84 | -0.52 | -0.23 | 0.5 |
|  | 30 | -1.54 | -1.18 | -0.84 | 1 |
|  | 60 | -3.05 | -2.59 | -2.18 | 2.5 |
|  | 60 | -2.33 | -1.95 | -1.56 | 2 |
|  | 120 | -5.56 | -4.85 | -4.18 | 4.5 |

Table 2: L1-SRS-RSRP performance for Case 1 in TDL-A

| Sample number | SCS (kHz) | L1-SRS-RSRP error (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% |
| 1 | 15 | -1.40 | -0.67 | 0.02 |
|  | 30 | -2.03 | -1.28 | -0.61 |
|  | 60 | -3.61 | -2.74 | -1.76 |
|  | 60 | -2.84 | -2.03 | -1.25 |
|  | 120 | -6.40 | -4.92 | -3.49 |
| 3 | 15 | -1.00 | -0.65 | -0.31 |
|  | 30 | -1.75 | -1.31 | -0.92 |
|  | 60 | -3.29 | -2.73 | -2.22 |
|  | 60 | -2.47 | -2.04 | -1.55 |
|  | 120 | -5.85 | -4.88 | -4.11 |

Table 3: L1-SRS-RSRP performance for Case 1 in TDL-C

| Sample number | SCS (kHz) | L1-SRS-RSRP error (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% |
| 1 | 15 | -1.28 | -0.64 | -0.02 |
|  | 30 | -2.00 | -1.27 | -0.56 |
|  | 60 | -3.65 | -2.66 | -1.82 |
|  | 60 | -3.00 | -2.10 | -1.34 |
|  | 120 | -6.65 | -4.98 | -3.71 |
| 3 | 15 | -0.97 | -0.64 | -0.31 |
|  | 30 | -1.68 | -1.28 | -0.88 |
|  | 60 | -3.21 | -2.68 | -2.16 |
|  | 60 | -2.66 | -2.12 | -1.65 |
|  | 120 | -6.10 | -5.04 | -4.20 |

Case 2

The measurement performance for Case 1 with 1 and 3 samples are shown in Table 1-3 for different channel conditions.

Table 4: L1-SRS-RSRP performance for Case 2 in AWGN

| Sample number | SCS (kHz) | L1-SRS-RSRP error (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% |
| 1 | 15 | -0.54 | -0.07 | 0.42 |
|  | 30 | -0.78 | -0.28 | 0.19 |
|  | 60 | -1.25 | -0.73 | -0.16 |
|  | 60 | -0.70 | -0.21 | 0.30 |
|  | 120 | -1.05 | -0.55 | -0.05 |
| 3 | 15 | -0.39 | -0.13 | 0.15 |
|  | 30 | -0.64 | -0.36 | -0.08 |
|  | 60 | -1.13 | -0.82 | -0.50 |
|  | 60 | -0.56 | -0.27 | 0.02 |
|  | 120 | -0.92 | -0.64 | -0.30 |

Table 5: L1-SRS-RSRP performance for Case 2 in TDL-A

| Sample number | SCS (kHz) | L1-SRS-RSRP error (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% |
| 1 | 15 | -0.82 | -0.22 | 0.35 |
|  | 30 | -1.12 | -0.47 | 0.16 |
|  | 60 | -1.59 | -0.91 | -0.30 |
|  | 60 | -0.90 | -0.34 | 0.26 |
|  | 120 | -1.31 | -0.67 | -0.03 |
| 3 | 15 | -0.56 | -0.24 | 0.07 |
|  | 30 | -0.81 | -0.47 | -0.16 |
|  | 60 | -1.29 | -0.89 | -0.57 |
|  | 60 | -0.66 | -0.34 | 0.01 |
|  | 120 | -1.02 | -0.67 | -0.30 |

Table 6: L1-SRS-RSRP performance for Case 2 in TDL-C

| Sample number | SCS (kHz) | L1-SRS-RSRP error (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% |
| 1 | 15 | -0.77 | -0.24 | 0.36 |
|  | 30 | -1.05 | -0.46 | 0.13 |
|  | 60 | -1.49 | -0.87 | -0.29 |
|  | 60 | -1.04 | -0.43 | 0.12 |
|  | 120 | -1.50 | -0.80 | -0.19 |
| 3 | 15 | -0.53 | -0.22 | 0.07 |
|  | 30 | -0.80 | -0.46 | -0.10 |
|  | 60 | -1.26 | -0.89 | -0.51 |
|  | 60 | -0.82 | -0.44 | -0.13 |
|  | 120 | -1.35 | -0.83 | -0.42 |

Conclusions

In this paper we provided our initial simulation results for L1-SRS-RSRP. We have made the following observations from the results.

Observation 1: In Case 1, 3-sample can provide 0.1-0.5dB gain in the accuracy over 1-sample. The gain is larger when the timing error has a larger impact (i.e. with larger SCS), and more obvious in fading channel.

Observation 2: In Case 2, the accuracy is clearly better than in Case 1, for 15kHz SCS the gap is 0.5dB while for 120kHz the gap can be up to 5dB. The performance gain of 3-sample over 1-sample is marginal (~0.2dB) for all cases.

Reference

- R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson