# 3GPP TSG-RAN WG4 Meeting #113                                                                R4-2419159

# Orlando, US, 18th – 22nd November, 2024

Title: 	Clarifications Reference architecture for intra-band NC CA

Source: 	Huawei, HiSilicon

Agenda item:	7.5.2.3

Document for:	Discussion

# Introduction

RAN4 112-bis meeting the WF [1] was agreed, in which the reference architectures are yet to be discussed. In [2]we proposed our initial perspective and in the current document, the reference architectures are discussed.

# Discussion

In Rel-11, TR 36.823, a partially shared receiver architecture was considered for intra-band non-contiguous CA, as shown below.

Moreover in the WF of the DL fragmented carrier SI [3], it was confirmed that for the existing DL NCCA requirements partially shared architecture is used, which shows the validity of the reference architecture in TR 36.823. Therefore the type 1 UE architecture that is considered as baseline and a fallback from type 2 UE architecture, in Rel-18, has to follow a partially shared architecture, which is not the current state of the table in the WF R4-2217734 that is mentioned in the WID.

As can be observed above, type 1 has a fully shared architecture, which is susceptible to in-gap interference in an intra-band NC CA configuration, because the low pass filters (LPF) are shared among the CCs. A solution to solve the in-gap interference, in a fully shared architecture, is to reduce the number of DL layers/CC from 4 to 2. So each CC will have its own LPFs. On the other hand, having a type 1 UE architecture with 2 DL layers/CC does not have any performance improvement with respect to type 2, and consequently, a type 1 fully shared architecture cannot be a good fallback from type 2.

In-gap interference is not an issue for partially shared UE architecture as each CC has its own LPF which blocks the interference.

Based on the above discussions, especially the fact that the current intra-band NC CA is based on partially share architecture, one would need to either change drastically the table and make sure that partially shared UE architecture is the baseline or the table can be simplified as follows which removes the details of the receiver architecture:

| UE Type | CC# | antenna / LNA |  | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  |  |
| 2 | 1 | 2 | 4 total | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  |  |  |  |
| 3a | 1 | 4 shared |  | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  |  |  |  |
| 3b | 1 | 4 shared |  | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  |  |  |  |
| 4a | 1 | 4 | 6 total | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  |  |  |  |
| 4b | 1 | 4 | 8 total | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  |  |  |  |

Observation 1: Partially share architecture is the reference architecture for the existing DL NC CA requirements, per WF R4-2417201

Proposal 1: Remove the details of the architecture and simplify the table in WF R4-2217734 as above, which remains in accordance with having partially shared UE architecture as type 1.

# Conclusions

Observation 1: Partially share architecture is the reference architecture for the existing DL NC CA requirements, per WF R4-2417201.

Proposal 1: Remove the details of the architecture and simplify the table in WF R4-2217734 as below, which remains in accordance with having partially shared UE architecture as type 1.

| UE Type | CC# | antenna / LNA |  | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  |  |
| 2 | 1 | 2 | 4 total | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  |  |  |  |
| 3a | 1 | 4 shared |  | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  |  |  |  |
| 3b | 1 | 4 shared |  | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  |  |  |  |
| 4a | 1 | 4 | 6 total | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  |  |  |  |
| 4b | 1 | 4 | 8 total | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  |  |  |  |

# Reference

[1]	“R4-2417115, WF on rest issues for intra-band non-collocated EN-DC/NR-CA, KDDI.”

[2]	“R4-2416260, How to introduce UE types into the specifications and rectifying errors UE architecture types, Huawei HiSilicon.”

[3]	“R4-2417201, WF on NR FR1 DL fragmented carriers study, Mediatek.”