# 3GPP TSG-RAN WG4 Meeting #113                                 R4-2419169

# Orlando, US, 18th – 22nd November, 2024

# Title: 	Discussion on BS demodulation performance requirements MMSE-IRC

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.16.3

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the revised WID of NR demodulation performance requirements for NR BS with MMSE-IRC receiver was approved in [1], wherein the objectives related to NR BS MMSE-IRC receiver are listed as below:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, we give some initial discussion on the NR BS MMSE-IRC receiver.

- Discussion

Based on the conclusion in RAN4 #112bis meeting[2], some issues for BS demodulation performance requirements MMSE-IRC need further discussion.

With regard to the test scenarios, 2 scenarios are agreed to be covered.

#### Sub-topic 1 General Test scenarios - On HomNet and HetNet scenarios: - Cover HomNet and HetNet scenarios. - FFS whether to introduce additional applicability rule. - On scenario with unaligned TDD pattern: - For NR_demod_Ph5 WI, only consider interference model with aligned TDD pattern between cells.

It is pointed out by company[3] that in LTE the HomNet requirements won’t be applied to Local Area BS and Home BS by adding a note in the requirement table. From the perspective of test effort, it is possible to consider the applicability rule between HomNet and Het if a BS supports both. A straightforward way is test HetNet scenario and skip HomNet if a BS supports both. But the decision can be made after the simulation parameters are aligned and the simulation results are aligned between companies.

Proposal 1. To discuss applicability rule after simulation parameters and simulation results are aligned.

With regard to the interference modelling to account for HPUE, 2 proposals are pointed out.

#### Sub-topic 2 Interference modelling Interference profile - On interference modelling: - Reuse the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH - On how to account for HPUE in existing LTE profiles: - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE. - It is not precluded for companies to bring system level simulation on other profiles. - Proposals on how to account for HPUE in existing LTE profiles. - Proposal 1: Perform new system-level simulations with the inclusion of HPUEs - Proposal 2: Consider artificially increasing DIP/INR with additional 3 or 7 dB

If we want to account for HPUE some details, such as the HPUE distribution in system level simulation, need to be discussed first. Increasing DIP/INR with a fixed value may not reflect the interference level when HPUEs are involved, although this option is convenient for reducing the workload of RAN4.

Observation 1. The HPUE distribution is not reached.

Proposal 2. FFS whether account for HPUE, as the HPUE distribution is not reached

With regard to propagation condition, there are still multiple options that need to be further down selected.

Propagation condition - Candidate options: - For serving cell: - Option 1: TDLA30-10 and TDLC300-100 for HetNet and HomNet respectively - Option 2: TDLA30-10 and TDLB100-100 for HetNet and HomNet respectively - Option 3: Cover TDLA30-10, TDLB100-400, TDLC300-100 - Option 4: Use TDLB100-400 - For interference cell: - Option 1: Introduce new propagation channel for testing with desired delay spread 1000ns for the interference PUSCH, i.e., TDLC1000 - Option 2: Same propagation condition for serving and interference PUSCH - Option 3: Use TDLC300-100

For the serving cell, there are 4 options for channel model selection. According to the conclusion of the last meeting, the one or more MCSs will be examined. If multiple MCSs are determined for performance requirement, it is preferred to select different channel model for different MCS. As for channel model of interfering cell, a channel model with larger delay spread such as TDLC300-100 can be considered

Proposal 3. To consider option 3 for serving cell and option 3 for interference cell .

With regard to MCS, 3 options are identified last meeting.

MCS - Candidate options: - Option 1: Cover QPSK/16QAM/64QAM - Option 1A: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024) - Option 1B: MCS index 2, 12, 20 in MCS table 1 - Option 1C: Cover MCS 2, 16, 20 - Option 2: Choose only one MCS configuration - Option 3: Consider MCS 5(QPSK, R=379/1024) and MCS 13(16QAM, R=490/1024) and FFS 64QAM (ZTE)

From TS 36.104[3] we see the MCS used for LTE requirement definition which is summarized in table 2-1.

Table 2-1 LTE MCS and the close MCS in NR table 1

| Index | LTE MCS |  | NR MCS |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Modulation order | Code rate | MCS index | Modulation order | Code rate |
| 1 | QPSK | 0.36 | 5 | QPSK | 0.3700 |
| 2 | 16QAM | 0.51 | 13 14 | 16QAM 16QAM | 0.4785 0.5400 |
| 3 | 16QAM | 3/4 | 20 24 | 64QAM 64QAM | 0.5537 0.7539 |

As agreed in RAN4 112 meeting, QPSK/16QAM/64QAM64QAM need to be covered. For QPSK/16QAM, MCS 5 and MCS 13 can be considered. The alternatives for 64QAM are MCS 20 and MCS 24. MCS 24 and LTE 16QAM 3/4 have similar code rate, but the modulation order has been raised from 16QAM to 64QAM, which will require higher SINR. Therefore, MCS20 is preferred 64QAM performance requirement.

Proposal 4. To consider MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS20(64QAM R=0.5537).

With regard to waveform, it is pointed by company that DFT-s-OFDM is rare in the real deployment. If DFT-s-OFDM is rarely used in real deployment we can only define performance requirements for CP-OFDM.

Waveform - Agreement - Cover CP-OFDM and FFS DFT-s-OFDM

Proposal 5. To cover only CP-OFDM.

With regard to DMRS, the DMRS type and DMRS sequence need further discussion .

DMRS configuration - Candidate options on DMRS type: - Option 1: Cover DMRS configuration type 1 and type 2 - Option 2: DMRS configuration type 1 - Candidate options on DMRS sequence generation: - Option 1: - NID0=0, nSCID =0 for CP-OFDM - NID0=0, group hopping and sequence hopping are disabled for DFT-s-OFDM - Option 2: - nSCID =0 for all cells - Different NID0=0 for serving and interference cells

It is pointed by company that type 2 DMRS needs to be included to ensure the covariance matrix estimation accuracy. From the initial simulation results provided last meeting, MMSE-IRC shows obvious gain compared to MMSE. Maybe type 2 DMRS can provide better accuracy for covariance matrix estimation, but from the perspective of performance requirement, it is preferred to consider only type 1.

Proposal 6. Option 2 is preferred for DMRS type.

For DMRS sequence generation, option 2 can be considered to initialize 2 different sequences for serving cell and interference cell.

Proposal 7. Option 2 is preferred for DMRS sequence generation.

With regard to test requirement definition, two options are recognized.

Test requirement definition - Proposals: - Option 1: Using SINR based requirement definition - Option 2: SNR based requirement definition - Proposals on SINR definition: - Option 1: - For BS type 1-C and BS type 1-H: The SINR used in this clause is specified based on a single carrier and defined as: SINR = S / N’ Where: S is the total signal energy of one intra-cell UE in the slot on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). N’ is the summation of the received energy of the strongest inter-cell interfering signals explicitly defined in a test procedure plus the white noise energy N in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). The respective energy of each inter-cell interfering signal relative to N is defined by its associated INR value.. - For BS type 1-O The SINR used in this clause is specified based on a single carrier and defined as: SINR = S / N’ Where: S is the total signal energy of one intra-cell UE in a slot on a RIB. N’ is the summation of the received energy of the strongest inter-cell interfering signals explicitly defined in a test procedure plus the white noise energy N in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists on a RIB. The respective energy of each inter-cell interfering signal relative to N is defined by its associated INR value..

Given a SNR and DIP set, the SINR can be derived. As for the requirement definition, SNR is preferred because it does not require the step of converting SNR to SINR.

Proposal 7. Option 2 is preferred for test requirement definition.

In last meeting, it is proposed to check the performance gain of 1 or 2 interferers for 2Rx.

Number of interferers - For 4Rx and 8Rx, consider 2 interferers. - Check the performance gain of 1 or 2 interferers for 2Rx and make decision based on simulation result.

From our simulation results in the accompany contribution [4], we can observe that for 2 interfers 2Rx, the gain of MMSE-IRC over MMSE is from 1.1 to 2.2 dB. TDLA30-10 is selected for both serving cell and interfering cell. While the gain for 1 interfer 2Rx is from 2.2 to 5.1 dB. From the simulation results, we think the gain is obvious for some scenario.

Observation 2. In some scenarios, obvious performance gain , such as 2 dB or above, can be observed.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements of MMSE-IRC receiver for NR BS, The conclusions are:

Observation 1. The HPUE distribution is not reached.

Observation 2. In some scenarios, obvious performance gain , such as 2 dB or above, can be observed.

Proposal 1. To discuss applicability rule after simulation parameters and simulation results are aligned.

Proposal 2. FFS whether account for HPUE, as the HPUE distribution is not reached

Proposal 3. To consider option 3 for serving cell and option 3 for interference cell .

Proposal 4. To consider MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS20(64QAM R=0.5537).

Proposal 5. To cover only CP-OFDM.

Proposal 6. Option 2 is preferred for DMRS type.

Proposal 7. Option 2 is preferred for DMRS sequence generation.

# 4	Reference

- RP-241297, Revised WID on NR demodulation performance: Phase 5, .

- R4-2416548, WF on [112bis][319] NR_demod_Ph5_Part1_General_BS, China Telcom

- R4-2416091, Discussion on Rel-19 MMSE-IRC BS demodulation requirements, Ericsson

- R4-2419170 ,Simulation results for BS demodulation performance for MMSE-IRC, ZTE Corporation, Sanechips