3GPP TSG-RAN WG4 Meeting #113	R4-2419184

Orlando, US, 18th – 22nd November 2024

Source:	Nokia

Title:	RRM core requirements for AI/ML Based Beam Management

Agenda item:	7.17.4.1

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#112bis in the scope of Rel-19 WI based. Agreements achieved during this meeting are summarized in the WF [1].

Some of the issues related to AI/ML enabled BM-Case1 and BM-Case2 require further discussion, In this paper, we provide some additional views on issues related to RRM core requirements, as listed below:

- Measurement error impact and the approximated ground truth

- Indicated/activated TCI states requirements

- Impacts on requirements related to candidate beam detection

- KPIs for beam prediction

- LCM related requirements

Generalization, test coverage and testing setup related aspects are discussed in our accompanying paper ‎[2].

Discussion

Measurement error impact and the ground truth

During RAN4#112 and RAN4112bis meetings, RAN4 agreed to study the impact of measurement errors to AI/ML BM functionality on the performance of prediction of AI/ML BM. The agreements are copied below:

Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. - impact on different prediction accuracy metrics should be taken into account - studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

Agreement: ​ RAN4 to perform study to assess the impact of measurement errors on prediction accuracy​ Perform LLS to evaluate the RSRP prediction accuracy and other KPIs​ - UE Model trained using ideal measurements​ Simulation assumptions in R4-2417211 can be used as starting point.

In this section, we evaluate the impact of measurement errors for an AI/ML model in inference operation. We consider spatial domain prediction with 32 beams in Set A, Set B is subset of Set A and contains 8 beams for measurement as a baseline. The AI/ML model’s output is the Top-K predicted beam IDs.

Methodology

We followed the procedure described in Section 5 of ‎[3], where we trained an AI/ML model with a subset of samples from an ideal L1-RSRP dataset generated from the system level simulation (SLS) following the assumptions defined in TR 38.843 Table 6.3.1-1.

Next, we followed the “Approach a” of ‎[3], and we used the LLS simulation assumptions defined in Table 6 of Annex I to generate the Baseband (BB) errors. We also modelled the RF errors, and we added both BB and RF errors to the subset of ideal L1-RSRP data to derive the dataset with measurement errors as follows,

RSRP with measurement errors = Ideal RSRP + Baseband (BB) error + Radio frequency (RF) error

Given the dataset with measurement errors, we evaluate the performance metrics considering measurement errors. The AI/ML model uses measured RSRP as the input. The ground-truth for evaluating the predictions is with ideal RSRP.

The results are compared to the performance metrics without considering measurement errors, for which we used another subset of samples from the ideal L1-RSRP dataset generated from the SLS and without measurement errors.

We evaluated the performance of the AI/ML model with Top-K accuracy for K =1,3, and 5, and we evaluated the 90%-tile of L1-RSRP difference between the maximum RSRP of the Top-K predicted beam(s) and the RSRP of the genie aided strongest beam.

For BB error modeling, we consider following options:

- No error: This is the baseline case where BB errors are not considered in AI/ML model input.

- LLS (Sample = 3, SINR = -4 dB): The BB error distribution obtained by LLS simulations based on the LLS assumption in Table 6 for side conditions where number of samples in L1 averaging=3 and SINR = -4 dB.

- LLS (Sample = 5, SINR = 0 dB): The BB error distribution obtained by LLS simulations based on the LLS assumption in Table 6 for side conditions where number of samples in L1 averaging=5 and SINR = 0 dB.

- Gaussian distribution: 95% of users are distributed within ±2.5 dB error margin with zero mean.

For RF error modeling, we consider following options:

- No error: This is the baseline case where RF errors are not considered as input of the AI/ML model.

- Gaussian (Independent for each TX beam): 95% of users are distributed within ±4.5 dB error margin with mean zero. The RF error is generated independently for each RSRP measurement of TX beam in a single inference instance so that different TX beams are measured with different RF errors.

- Uniform distribution (Independent for each TX beam): All users are distributed within ±4.5 dB error margin with mean zero. The RF error is added independently for each RSRP measurement of TX beam in a single inference instance so that different TX beams are measured with different RF errors.

- Gaussian distribution (Same for all TX beams): 95% of users are distributed within ±4.5 dB error margin with mean zero. The same RF error is added to all the RSRP measurements of the different TX beams for a single inference instance.

Evaluation Results of BB error excluding RF error

In Figure 1, we evaluate the impact of BB error on measurement without considering RF error. The BB error distributions in Figure 1 shows 95% of BB errors in the range of [-0.83, 1.78] with 1st LLS conditions (# samples = 3, SINR = -4 dB) and in the range of [-0.35, 0.83] with the 2st LLS conditions (# samples = 5, SINR = 0 dB). The BB error distribution with a better SINR and longer L1 averaging filter reduce the error margins of the BB error. On the other hand, a Gaussian distribution with ±2.5 dB error margin shows a higher BB error variance compared to both BB errors with distributions derived from LLS.

We summarize the AI/ML model performance for BB error options in Table 1. The Top-1 accuracy of BB error with 2nd LLS conditions (# samples = 5, SINR = 0 dB) was reduced by 0.61% compared to that of the case with no BB error. It further reduced by 1.6% for 1st LLS conditions (# samples = 3, SINR = -4 dB). The highest degradation of Top-1 accuracy is about 4.91% with the Gaussian BB error. Moreover, the Top-3 and Top-5 accuracy follow a similar trend to that of Top-1, and performance reduction becomes insignificant as K increases. Similar observations can be made with the 90%-tile L1-RSRP difference, which it shows ~1 dB degradation for LLS-based BB error models wherein a 2.96 dB degradation is observed with the Gaussian distribution.

Figure 1. CDFs of BB error distribution (left) and L1-RSRP difference (right) without considering RF error.

Table 1. AI/ML model performance with BB error models without considering RF error.

| BB error model | Top-1 (%) | Top-3 (%) | Top-5 (%) | 90%-tile L1-RSRP difference (dB) |
| --- | --- | --- | --- | --- |
| No error | 83.59 | 96.22 | 98.34 | 5.98 |
| 1st LLS conditions (# samples = 3, SINR = -4 dB) | 81.99 | 95.98 | 98.21 | 6.83 (Δ = 0.85) |
| 2nd LLS conditions (# samples = 5, SINR = 0 dB) | 82.98 | 96.22 | 98.34 | 6.44 (Δ = 0.46) |
| Gaussian (95% of errors within ±2.5 dB error margin) | 78.68 | 95.14 | 97.94 | 8.94 (Δ = 2.96) |

Note: 90%-tile L1-RSRP difference = max(abs(95%-tile L1-RSRP), abs(5%-tile L1-RSRP))

In Table 1, we can see that the BB error margin of the Gaussian distribution is larger than the BB error margins provided by LLS distributions.

Given the lower BB error margin with LLS distributions, both prediction accuracy and L1-RSRP difference KPIs are not significantly impacted by the BB errors with distributions derived from LLS. On the other hand, there is a degradation of ~3 dB for the BB error with Gaussian distribution where the error margin is considering current measurement error margin ±2.5 dB for BB errors (considering error margin ±6.5 for both BB and RF errors).

Evaluation Results of BB errors with different RF error models

- In Tables 2, 3 and 4, we summarize the AI/ML model performance for different RF error models added to the BB errors. The RF error models follow Gaussian, and Uniform distribution with ±4.5 dB error margin, assuming the RF error generated as detailed in Section 2.1.1.

Evaluation of Gaussian vs. Uniform: If we consider the Gaussian BB error model, Table 2 shows a Top-1 accuracy reduction of 15.21% with Gaussian RF error (Independent for each TX beam)) compared to the scenario without measurement error.  For the same BB error model, the Uniform distribution (Independent for each TX beam) shows the highest degradation of 17.37% in terms of Top-1 accuracy compared to the scenario with no measurement error. The Top-1 accuracy degradation of Gaussian is lower than the Uniform distribution as most of the RF errors are concentrated near the mean value zero, and few RF errors appear further away from the mean.

Evaluation of independent RF errors vs. the same RF error for different TX beams: For Gaussian RF error independent for each TX beam, Table 3-4 shows that with RF+BB errors for both LLS conditions, the impact on performance KPIs is significant: From Table 3, Top-1 decreases by 13% and 90%-tile L1-RSRP difference decreases by 7 dB compared to the case without errors. On the other hand, when using the same RF error according to Table 2, 3, and 4, the Top-1 accuracy reduction is less than 1% for all BB error models compared to no error case. Similarly, the 90%-tile L1-RSRP difference shows less than 0.5 dB degradation when the same RF error is applied for all TX beams. Therefore, using the same RF error for all TX beams does not significantly impact the beam prediction accuracy. This is similar to apply a RSRP offset for AI/ML model’s input and therefore, since normalization is applied, the AI/ML model is able to predict the strongest beam IDs without impacting the accuracy.

Evaluation of impact of BB error + RF error with LLS conditions: comparing the RF+BB errors with different LLS conditions, there is a marginal degradation of 0.09 dB in the 90%-tile L1-RSRP difference. However, this also happens, as Table 1 shows, without considering the RF error, where the comparative degradation is 0.39 dB. The same trend is visible in Top-1 accuracy as well. Thus, the impact on the accuracy of beam prediction while changing LLS side conditions, is even less when both BB and RF error are considered.

Table 2. AI/ML model performance for different RF error models with BB error distribution fixed to Gaussian (95% of UEs in ±2.5 dB).

| RF error model | Top-1 (%) | Top-3 (%) | Top-5 (%) | 90%-tile L1-RSRP difference (dB) |
| --- | --- | --- | --- | --- |
| No BB+No RF errors | 83.59 | 96.22 | 98.34 | 5.98 |
| Gaussian with ±4.5 dB error margin (Independent for each TX beam) | 68.38 | 90.86 | 95.87 | 14.14 |
| Uniform with ±4.5 dB error margin (Independent for each TX beam) | 66.22 | 89.92 | 95.19 | 14.51 |
| Gaussian with ±4.5 dB error margin (Same for all TX beams) | 83.34 | 96.22 | 98.31 | 6.37 |

Note: 90%-tile L1-RSRP difference = max(abs(95%-tile L1-RSRP), abs(5%-tile L1-RSRP))

Table 3. AI/ML model performance for different RF error models with BB error distribution fixed to LLS (# samples = 3, SINR = -4 dB).

| RF error model | Top-1 (%) | Top-3 (%) | Top-5 (%) | 90%-tile L1-RSRP difference (dB) |
| --- | --- | --- | --- | --- |
| No BB+No RF errors | 83.59 | 96.22 | 98.34 | 5.98 |
| Gaussian with ±4.5 dB error margin (Independent for each TX beam) | 70.55 | 91.70 | 96.24 | 13.32 |
| Uniform with ±4.5 dB error margin (Independent for each TX beam) | 67.65 | 90.48 | 95.83 | 14.06 |
| Gaussian with ±4.5 dB error margin (Same for all TX beams) | 83.41 | 96.20 | 98.19 | 6.46 |

Note: 90%-tile L1-RSRP difference = max(abs(95%-tile L1-RSRP), abs(5%-tile L1-RSRP))

Table 4. AI/ML model performance for different RF error models with BB error distribution fixed to LLS (# samples = 5, SINR = 0 dB).

| RF error model | Top-1 (%) | Top-3 (%) | Top-5 (%) | 90%-tile L1-RSRP difference (dB) |
| --- | --- | --- | --- | --- |
| No BB+No RF errors | 83.59 | 96.22 | 98.34 | 5.98 |
| Gaussian with ±4.5 dB error margin (Independent for each TX beam) | 71.03 | 92.14 | 96.54 | 13.23 |
| Uniform with ±4.5 dB error margin (Independent for each TX beam) | 68.00 | 90.88 | 95.86 | 13.91 |
| Gaussian with ±4.5 dB error margin (Same for all TX beams) | 83.32 | 96.06 | 98.24 | 6.07 |

Note: 90%-tile L1-RSRP difference = max(abs(95%-tile L1-RSRP), abs(5%-tile L1-RSRP))

The performance degradation of AI/ML model in presence of Gaussian RF error is lower than that of Uniform distribution as most of the RF errors are concentrated near the mean value zero, and few RF errors appear further away from the mean.

The performance degradation of AI/ML model is high when RF error is added independently. Contrarily, performance degradation is minimal when RF error is same for all TX beams. Hence, the performance degradation difference between two cases is significant.

RAN4 to consider realistic approach to model RF error as the prediction accuracy difference of AI/ML model with worst case scenario of independent RF error and best-case scenario of same RF error for all TX beams is very high.

Given the significant degradation in prediction performance observed with independent RF error, RAN4 shall discuss: 
- Whether such degradation (Top-1 decreases by 13% and 90%-tile L1-RSRP difference decreases by 7 dB) with respect to the case without errors is acceptable or not.

Indicated/activated TCI state requirements

The issue of indicated/activated TCI states requirements was discussed during RAN4#112 meeting. Although, no agreement was reached but initial discussions were held on the options listed below:

- Option 1: Tx beam prediction does not imply that UE knows the best UE Rx beam to receive the predicted beam - TCI state for the predicted beam is unknown unless it has been explicitly measured within 1280ms - Option 2: UE knows the Rx beam paired with the predicted Tx beam - FFS whether the TCI state associated with a predicted Tx beam is known or unknown - Option 3: UE knows the Rx beam paired with the predicted Tx beam - TCI state should be considered known , FFS whether side conditions for known TCI state should be modified (relaxed) for this case - Option 4: others

In this section, we further provide our view on above listed options. In Rel-19 AI/ML BM, the measurements of the Set B beams are used as input to the ML model, which outputs the best K beams (beam indices and/or L1-RSRP values) from Set A beams. For a UE-sided model, Set A and B can be provided with the CSI reporting configurations.

When the UE predicts the best Tx beams out of Set A, while using only Set B Tx beams as input to the AI/ML functionality, and since UE has the information about complete set of beams (i.e. Set A), hence it is reasonable to assume that UE may know already the Rx beam to be used for receiving the predicted Tx beam. Furthermore, without corresponding Rx beams, the prediction for Tx beams may not be correct and may change with a change in corresponding Rx beams. Therefore, among Option 1 and Option 2 above, in our view, Option 2 is the right option.

It is reasonable to assume that UE may know the Rx beam to be used for receiving the predicted Tx beam, therefore Option 2 should be selected.

However, Option 2 only helps in identifying a problem, but it does not provide a solution. Therefore, it is important to consider Option 3 as well. In the following, we highlight the problem where we may have much more unknown TCI states due to the existence of predicted beams in case of AI/ML BM use case in comparison of legacy.

The NW can determine whether any changes are required in the set of active TCI states, and if yes, the NW can adapt the set of active TCI states based on the prediction report received from the UE. Here, the NW needs to ensure that the RS resource corresponding to the activated/indicated beams which can be measured by the UE prior any beam indication takes place corresponding to the predicted RS resource.

In the case of UE uses subset of beam measurements (Set B) to predict beam(s) from Set A, the UE may not have measured the RS resource included in the indicated/activated TCI state. Since the predicted beam(s) is not measured by the UE for longer period, the predicted beam(s) will be corresponding to unknown TCI state. The delay for switching the beam to an unknown TCI state is higher than switching to a known TCI state. Therefore, in our view, for a UE that supports beam prediction, there is a need to relax the known conditions for TCI state, specified in Clause 8.10.2 of TS 38.133:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown.

If this condition is relaxed, then the predicated beam (which may not be part of Set B beams) corresponding to a Set A/subset of set A, may be measured with a longer delay than 1280 ms and still fulfil TCI known state condition. In other words, a higher value of the delay requirements should be used for known TCI state conditions in Clause 8.10.2 of TS 38.133 for predicted beams.

In order to relax the known conditions of TCI state such that the predicted beam could be measured, e.g.,with a delay longer than 1280 ms, we need to further specify on how long the delay could be possible. Further analysis of this delay requirements or simulation results might be needed to define the exact delay requirements of known TCI states.

RAN4 should consider relaxing known conditions for TCI states (Clause 8.10.2 of TS 38.133) for BM-Case1 and BM-Case2. TCI state switch command can be received within a delay of X+1280 ms.  Further analysis is required to define the exact value of X.

Impacts on requirements related to candidate beam detection

In current RAN4 requirements, as mentioned in Clause 8.5.6.2 of TS 38.133, the UE is supposed to evaluate during a certain time period the L1-RSRP measured on the configured CSI-RS resource in the candidate beam set (called q1) containing CSI-RS resources configured for a serving cell. The objective is to verify if the L1-RSRP measured on the configured CSI-RS resource has not been degraded too much and is still better than a certain threshold (Qin_LR). The certain period used to evaluate L1-RSRP is known as TEvaluate_CBD_CSI-RS and is specified in Table 8.5.6.2-2 of clause 8.5.6.2 in TS 38.133.

Table 8.5.6.2-2: Evaluation period TEvaluate_CBD_CSI-RS for FR2 Configuration; TEvaluate_CBD_CSI-RS (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(MCBD ´ P ´ N ´ PCBD) ´ TCSI-RS). DRX cycle > 320ms; Ceil(MCBD ´ P ´ N ´ PCBD) ´ TDRX. Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length..

- However, with the addition of AI/ML BM feature, the UE is supposed to predict best beams from the Set A beams and the UE is configured to measure only a few beams (i.e. Set B). Therefore, the network will often configure only CSI-RS of the Set B beams, therefore the CSI-RS periodicity of Set A beams may be increased. If the NW uses same CSI-RS periodicity also for candidate RS resources, it may lead to a higher value of TCSI-RS used for the calculation of TEvaluate_CBD_CSI-RS. This will eventually lead to a higher value of  TEvaluate_CBD_CSI-RS, which consequently may cause a delayed beam recovery operation and may cause frequent beam failures A higher value of TCSI-RS in Table 8.5.6.2-2 may lead to a higher value of TEvaluate_CBD_CSI-RS, which consequently may be the cause of frequent beam failures.

RAN4 should consider the impact in RAN4 requirements for the Evaluation period TEvaluate_CBD_CSI-RS due to the presence of predicted beams.

KPIs for beam prediction

RAN4 identified different test metrics to assess and establish performance requirements of AI/ML Beam Prediction. The metrics were categorized into different options, see below, in 3GPP TR 38.843 V18.0.0. And the targeted use cases for the metrics include beam prediction for BM-Case1 and BM-Case2. Here, we are discussing the merits of these options and providing our view on which of these option(s) might be relevant.

The following test metrics/KPIs for Beam Prediction are considered: - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams", where K >1 and values can be reported. - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

During RAN4#112bis, the following definition of absolute RSRP accuracy as a test metric in option 1 has been agreed.

Agreement​ If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – ground truth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on ground truth L1-RSRP].

In the following, we provide our view on Options 2, 3 and 5.

Option2 (Beam IDs prediction accuracy):

In this option, when AI/ML functionality outputs Top-K beam IDs prediction, RAN4 discussed following metrics:

Top-1(%): This is the success rate, based on a number of occurrences, when Top-1 predicted beam is also the actual Top-1 strongest beam. The number of occurrences required for this metric needs yet to be discussed in RAN4. In our view, this is the strictest requirement, and the test may be failed due to even smallest prediction errors. Therefore, either RAN4 should have a relaxed performance target for this metric, or this metric should not be considered in RAN4.

Top-1(%) metric of Option2 is the strictest requirement and RAN4 should not consider it as a performance metric.

Top-K/1(%): This is the success rate, based on a number of occurrences, when Top-1 actual strongest beam is one of the Top-K predicted beams, where K>1. The number of occurrences required for this metric needs yet to be discussed in RAN4. This metric only guarantees that Top-1 actual strongest beam is one of the predicted beams. It doesn’t verify the accuracy of prediction by the AI/ML functionality for other beams. Therefore, this metric should not be considered alone. Although, one solution may be to use this metric in combination with the Top-1/K(%) metric.

Top-K/1(%) metric of Option2 only verifies if actual strongest beam is among Top-K predicted beams and it doesn’t verify the accuracy of prediction for other beams.

Top-K/1(%) metric of Option 2 should be considered in the combination of Top-1/K(%).

Top-1/K (%) : This is the success rate, based on a number of occurrences, when Top-1 predicted beam is one of the Top-K actual strongest beams, where K>1. The number of occurrences required for this metric needs yet to be discussed in RAN4. This metric only guarantees if the prediction of Top-1 predicted beam has a reasonable accuracy. It doesn’t verify the accuracy of other predictions. Since, network may indicate to the UE to use any of the predicted beams in the field, and if the prediction accuracy of other beams is not verified then it may cause a degraded performance in the field. Therefore, this metric should not be considered alone. Although, one solution may be to use this metric in combination with the Top-K/1(%) metric.

Top-1/K(%) metric of Option2 only verifies if Top-1 predicted beam is among Top-K actual strongest beams and it doesn’t verify the accuracy of other predictions.

Option 3:

Option 3 is defined as, “The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam x dB”.

This metric compares the measured L1-RSRP of the strongest predicted beam with the measured L1-RSRP of the actual strongest beam. In our view, this is a reasonable metric and should be considered by RAN4. However, this metric doesn’t verify the quality of prediction for other predicted beams. Since, network may indicate to the UE to use any of the predicted beams in the field, and if the prediction accuracy of other beams is not verified then it may cause a degraded performance in the field. Therefore, in our view, only Option 3 will not be sufficient, and it must be considered in combination with other Options (such as Option 1 and Option 2).

Option 3 only compares the measured L1-RSRP of the strongest predicted beam with the measured L1-RSRP of the actual strongest beam. It does not verify the quality of prediction for other predicted beams.

RAN4 should consider Option 3 but in combination of other Options (such as Option 1 and Option 2).

Option 5:

We introduced Option 5 as another option, which is to define the requirements not based on UE measurement as a reference, but to verify the expected behaviour of AI/ML-enabled functionality based on the change in the configurations/inputs of the functionality. For example, the best predicted beam should not change, if the power of all measured beams is scaled in the same way or when one of the measured beams cannot be received anymore, etc. In other words, the AI/ML functionality can be tested without explicit definition of the ground truth.

The behaviour of beam prediction functionalities can be tested without explicit definition of the ground truth by comparing the predictions to each other.

RAN4 to introduce additional requirements based on relative difference in between the predictions due to change in the conditions/inputs, i.e., without dependency on the measurement accuracy errors and uncertainties.

Option 5 should be considered in the combination of other approximated ground truth-based options (such as Option 1).

Furthermore, in RAN4#112bis, in general aspects agenda item, following agreement was achieved:

Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

Regarding, FFS on target performance of beam prediction and RSRP prediction for BM use case, based on the output of ongoing study on the measurement error impacts on RSRP prediction, RAN4 will be in better position to decide if at the advantage of measurement reduction, worse than legacy performance for RSRP prediction can be acceptable or not.

RAN4 should consider, based on the study on the measurement error impacts on RSRP prediction, if worse than legacy performance for RSRP prediction can be acceptable if there is significant reduction in the measurements.

LCM related requirements

LCM related core requirements are further discussed in this section.  In RAN4#111, RAN4 reached the following agreement:

Agreement: - RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are: - Model identification - Selection - Activation - Deactivation - Switching - fallback to non-AI operation - performance monitoring - Others are not precluded

The LCM operations for AI/ML BM, e.g., activation, deactivation, and fall-back (e.g., fall-back from AI/ML beam prediction to legacy mode) could be supported by CSI report configuration. These operations can be triggered, for example, based on the performance monitoring output. Reusing the existing capability framework, NW can configure CSI report configurations to enable BM-Case1 or BM-Case2 beam prediction. The CSI reports for both BM-Case1 and BM-Case2 can be based on periodic, semi-persistent, or aperiodic CSI-reporting. The latency of these operations will depend on multiple factors such as,

- Whether CSI configuration (ResourceSet) to be used is already available in the device or not,

- The type of the CSI reporting (periodic, semi-persistent, aperiodic),

- The availability of (previous) measurements that shall be used as an input to the model/functionality inference/prediction,

- Availability of the necessary model(s) in the device,

- The time it takes to substitute one model/functionality with another (the UE may have specialized AI-hardware accelerators to run the inference, which may require loading of the model),

However, it should be clearly specified how much time it takes for the UE to report the new predictions/inference results following the changes indicated by the LCM operation. Without such information NW will not be able to decide when to schedule grants for the reporting of measurements/predictions or data and taking too much time in these operations may also degrade the performance.

In LCM operation, the functionality activation operation is the first step when the UE receives from NW the necessary inference configuration needed for running the inference with the AI/ML functionality available at the UE. Then, other operations, e.g., deactivation, switching and fallback, come after functionality activation operation.

For AI/ML beam prediction reporting enabled by periodic CSI-report, the LCM-related latency requirements can be defined as below:

- For activating or switching inference operation of AI/ML-enabled beam prediction via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

- delay related to RRC Reconfiguration,

- delay from UE processing time: This may be related to the size of the inference report, model-related pre-processing time or memory limitations on storing models (associated with beam prediction),

- delay related to associated ID: This delay will depend on if the associated ID is used or not used in the past, there might be delay corresponding to configure/indicate the associate ID to UE,

- delay due to CSI-RS measurement period.

- For deactivating or switch to fallback to non-ML operation (beam measurement) via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay related to RRC Reconfiguration,

  - delay from UE processing time

  - delay due to CSI-RS measurement period.

For AI/ML beam prediction reporting enabled by semi-persistent CSI-report, the LCM-related latency requirements can be defined as below:

- For activating or switching inference operation of AI/ML-enabled beam prediction via semi-persistent CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay from activating semi-persistent CSI resource set and delay for selecting of semipersistent CSI resource set.

  - delay from UE processing time

  - delay related to associated ID

  - delay due to CSI-RS measurement period.

- For deactivating or switch to fallback to non-ML operation (beam measurement) via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay from NW selecting of semi-persistent CSI resource set.

  - delay from UE processing time related

  - delay due to CSI-RS measurement period.

For AI/ML beam prediction reporting enabled by aperiodic CSI-report, the LCM-related latency requirements can be defined as below:

- For activating or switching inference operation of AI/ML-enabled beam prediction via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay related activating aperiodic CSI resource set,

  - delay from UE processing time

  - delay related to associated ID,

  - delay due to CSI-RS measurement period.

- For deactivating or switch to fallback to non-ML operation (beam measurement) via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay related NW selecting of aperiodic CSI resource set,

  - delay from UE processing time related

  - delay due to CSI-RS measurement.

In summary, the delay components related to LCM operations are provided in Table 5.

Table 5: Delay components related LCM operations for different CSI-report configurations

| CSI-Report Configuration | Activation | Switching | Fall-back (From AI/ML BM to legacy) |
| --- | --- | --- | --- |
| Periodic CSI-reporting | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay due to CSI-RS measurement period |
| Semi-persistent CSI-reporting | Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay due to CSI-RS measurement period |
| Aperiodic CSI reporting | Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay from UE processing related to capability report Delay due to CSI-RS measurement period |

The delay components of LCM operations should be considered for different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2.

RAN4 should consider Table 5 as a baseline to start discussions for LCM operations related RRM core requirements.

Conclusion

In this paper we share our views on potential RAN4 impacts from issues related to RRM core requirements for AI/ML enabled BM-Case1 and BM-Case2. Specifically, we cover following aspects:

- Measurement error impact and the approximated ground truth

- Indicated/activated TCI states requirements

- Impacts on requirements related to candidate beam detection

- KPIs for beam prediction

- LCM related requirements

In the paper, the following Observations and Proposals were made:

Observation 1: In Table below, we can see that the BB error margin of the Gaussian distribution is larger than the BB error margins provided by LLS distributions.

Table. AI/ML model performance with BB error models without considering RF error.

| BB error model | Top-1 (%) | Top-3 (%) | Top-5 (%) | 90%-tile L1-RSRP difference (dB) |
| --- | --- | --- | --- | --- |
| No error | 83.59 | 96.22 | 98.34 | 5.98 |
| 1st LLS conditions (# samples = 3, SINR = -4 dB) | 81.99 | 95.98 | 98.21 | 6.83 (Δ = 0.85) |
| 2nd LLS conditions (# samples = 5, SINR = 0 dB) | 82.98 | 96.22 | 98.34 | 6.44 (Δ = 0.46) |
| Gaussian (95% of errors within ±2.5 dB error margin) | 78.68 | 95.14 | 97.94 | 8.94 (Δ = 2.96) |

Note: 90%-tile L1-RSRP difference = max(abs(95%-tile L1-RSRP), abs(5%-tile L1-RSRP))

Observation 2: Given the lower BB error margin with LLS distributions, both prediction accuracy and L1-RSRP difference KPIs are not significantly impacted by the BB errors with distributions derived from LLS. On the other hand, there is a degradation of ~3 dB for the BB error with Gaussian distribution where the error margin is considering current measurement error margin ±2.5 dB for BB errors (considering error margin ±6.5 for both BB and RF errors).

Observation 3: The performance degradation of AI/ML model in presence of Gaussian RF error is lower than that of Uniform distribution as most of the RF errors are concentrated near the mean value zero, and few RF errors appear further away from the mean.

Observation 4: The performance degradation of AI/ML model is high when RF error is added independently. Contrarily, performance degradation is minimal when RF error is same for all TX beams. Hence, the performance degradation difference between two cases is significant.

Proposal 1: RAN4 to consider realistic approach to model RF error as the prediction accuracy difference of AI/ML model with worst case scenario of independent RF error and best-case scenario of same RF error for all TX beams is very high.

Proposal 2: Given the significant degradation in prediction performance observed with independent RF error, RAN4 shall discuss:

- Whether such degradation (Top-1 decreases by 13% and 90%-tile L1-RSRP difference decreases by 7 dB) with respect to the case without errors is acceptable or not.

Proposal 3: It is reasonable to assume that UE may know the Rx beam to be used for receiving the predicted Tx beam, therefore Option 2 should be selected.

Proposal 4: RAN4 should consider relaxing known conditions for TCI states (Clause 8.10.2 of TS 38.133) for BM-Case1 and BM-Case2. TCI state switch command can be received within a delay of X+1280 ms.  Further analysis is required to define the exact value of X.

Proposal 5: RAN4 should consider the impact in RAN4 requirements for the Evaluation period TEvaluate_CBD_CSI-RS due to the presence of predicted beams.

Observation 5: Top-1(%) metric of Option2 is the strictest requirement and RAN4 should not consider it as a performance metric.

Observation 6: Top-K/1(%) metric of Option2 only verifies if actual strongest beam is among Top-K predicted beams and it doesn’t verify the accuracy of prediction for other beams.

Proposal 6: Top-K/1(%) metric of Option 2 should be considered in the combination of Top-1/K(%).

Observation 7: Top-1/K(%) metric of Option2 only verifies if Top-1 predicted beam is among Top-K actual strongest beams and it doesn’t verify the accuracy of other predictions.

Observation 8: Option 3 only compares the measured L1-RSRP of the strongest predicted beam with the measured L1-RSRP of the actual strongest beam. It does not verify the quality of prediction for other predicted beams.

Proposal 7: RAN4 should consider Option 3 but in combination of other Options (such as Option 1 and Option 2).

Observation 9: The behaviour of beam prediction functionalities can be tested without explicit definition of the ground truth by comparing the predictions to each other.

Proposal 8: RAN4 to introduce additional requirements based on relative difference in between the predictions due to change in the conditions/inputs, i.e., without dependency on the measurement accuracy errors and uncertainties.

Proposal 9: Option 5 should be considered in the combination of other approximated ground truth-based options (such as Option 1).

Proposal 10: RAN4 should consider, based on the study on the measurement error impacts on RSRP prediction, if worse than legacy performance for RSRP prediction can be acceptable if there is significant reduction in the measurements.

Proposal 11: The delay components of LCM operations should be considered for different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2.

Proposal 12: RAN4 should consider Table below as a baseline to start discussions for LCM operations related RRM core requirements.

Table: Delay components related LCM operations for different CSI-report configurations

| CSI-Report Configuration | Activation | Switching | Fall-back (From AI/ML BM to legacy) |
| --- | --- | --- | --- |
| Periodic CSI-reporting | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay due to CSI-RS measurement period |
| Semi-persistent CSI-reporting | Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay due to CSI-RS measurement period |
| Aperiodic CSI reporting | Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay from UE processing related to capability report Delay related to associated ID Delay due to CSI-RS measurement period | Delay from UE processing related to capability report Delay due to CSI-RS measurement period |

- References

- R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm Incorporated, RAN4#112bis meeting, 14th – 18th October 2024, China.

- R4-2419185, Testability and Interoperability aspects for AI/ML Based Beam Management, Nokia, RAN4#113, Orlando, US, 18 – 22 November 2024.

- R4-2417211, Simulation assumptions for beam prediction, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT, RAN4#112-bis, Hefei, China, 14th –18th October 2024.

- Annex I (LLS assumptions for BB error modeling)

Table 6. Baseline LLS assumptions for AI/ML in beam management evaluations.

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30GHz. |
| Subcarrier spacing | 120kHz |
| Channel model | AWGN channel |
| BS antenna configurations | Option 2: 1 Tx antenna |
| BS antenna element radiation pattern | Option 2: N/A |
| BS antenna height and antenna array down-tilt angle. | Option 2: N/A |
| Beamforming characteristic of the BS pattern | Option 2: N/A |
| UE antenna configurations | Option 2: 2 Rx |
| UE antenna element radiation pattern | Option 2: N/A |
| UE moving speed | 3km/h |
| Reference signal | CSI-RS |
| DRX | No |
| Number of samples in L1 averaging | [3, 5] |