3GPP TSG-RAN WG4 Meeting #113                                        R4-2419191

Orlando, US, 18th – 22nd November, 2024

Source:	Samsung

Title:              Discussion on NES common signal adaptation

Agenda item:			7.22.2.2

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some discussions on NES common signal adaptation. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 RAN1 agreements update

The latest progress for NES common signal/channel adaptation is duplicated as below

<SSB adaptation> Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19. Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior). - No spec impact <PRACH adaptation> Agreement For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources. Agreement For adaptation of PRACH in time-domain, support both of the following - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources - FFS: Additional details Possible Agreement For DCI-based adaptation for additional PRACH resources the following is supported - Alt1A: At least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P- RNTI is used - Alt1B: At least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - New RNTI is used - Alt2: - At least DCI format 2_7 can carry the adaptation indication for UEs in idle/inactive mode. - At least DCI format 2_9 can carry the adaptation indication for UEs in connected mode. - Alt3 - Alt1 + first sub-bullet of Alt2 Working Assumption For DCI-based adaptation for additional PRACH resources, - Alt1A2: At least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used Agreement For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different - FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement). - Discuss further following options for signaling - Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources. - msg1-FrequencyStart at least for 4-step RACH - FFS: other applicable legacy frequency domain parameter(s) - Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources - Note: Offset to legacy frequency domain parameter(s) is a new parameter - FFS: applicable legacy frequency domain parameter(s) Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211) - For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

Based on the agreements achieved in RAN1, we have the following observations and proposals:

Observation 1: Based on RAN1 agreements on SSB adaptation:

- Rel-19 NES-capable UE in idle mode is not supported

- For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, no RAN1 spec. impact

Proposal 1: There is no need to discuss the RRM requirements for the scenarios of Rel-19 NES-capable UEs in idle mode

- Whether to discuss the RRM requirements for the scenarios of Rel-19 NES-capable UEs in inactive mode should wait for RAN1 progress

Since Cell DTX (Rel-18 features) extension to SSB adaptation for connected mode UEs is not introduced in Rel-19 RAN1.

Proposal 2: There is no need to discuss the RRM requirements related to Cell DTX for connected mode Rel-19 NES-capable Ues

3 Discussions on SSB adaptation

3.1 Adaptation mechanism

In last meeting, RAN4 agreed to take SSB periodicity in time-domain as starting point for SSB adaptation discussion, while the details were not fully discussed.

3.1.1 SSB burst periodicity

Some companies mentioned that introducing longer SSB periodicity (> 160 ms) for SSB adaptation can achieve NW power saving gain. Actually, this issue is also discussing in RAN1. Based on the simulation results provided in TR 38.864, longer SSB periodicity can bring significant BS energy saving gains, compared to 20ms SSB transmission periodicity baseline under most cases. From this, proponent think the longer periodicity (>160 ms) should be adapted based on the traffic load to maximize the saving gain, but no conclusion achieved yet.

From RAN4 perspective, in our opinion, longer SSB periodicity may bring interesting RRM requirements impacts, e.g., measurement period expansion, SCell activation delay enlargement, etc, depending on the modified TSSB, TSMTC_X related to new upper bound of transmission period of SSB. In this sense, the corresponding impacts on RRM requirements based on the introduced longer SSB adaptation periodicity shall be further discussed.

Observation 2: Based on the simulation results provided in TR 38.864, larger periodicity can bring BS energy saving gains

Proposal 3:

- Suggest RAN4 to consider legacy SSB burst periodicity as starting point to define the requirements

- For discussion on RRM requirements impacts based on longer SSB adaptation periodicity can wait for RAN1 progress on whether longer SSB burst adaptation periodicity can be supported

3.1.2 How to trigger SSB adaptation

Actually, based on RAN1 agreement achieved in the previous meetings, SSB adaptation triggered by NW was taken as a preferred solution

Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger

While, the discussions on by which way the UE can achieve the adapted SSB periodicity, RRC reconfiguration, MAC-CE or DCI is ongoing for a few meetings. From our understanding,

- SIB1 update/RRC reconfiguration based adaptation can be considered as a supported way.

- MAC-CE. Since it was agreed in RAN1#117 and RAN1 #118 that MAC-CE based signaling to indicate on-demand SSB transmission on the SCell. Considering the commonality between on-demand SSB and SSB periodicity adaptation at least from SCell operation perspective, whether the same scheme/MAC-CE framework for on-demand SSB can be supported in SSB adaptation could be arguable

- DCI. Since it was agreed in RAN1 that DCI based signaling (at least DCI format 1_0) to indicate whether (a subset of) the additional PRACH resources provided by semi-static signaling are available or not. Considering the association between SSB and valid RO, whether it is doable and natural to extend the DCI scheme of PRACH to SSB adaptation could be arguable

Either way (MAC-CE/DCI) is feasible but with different processing time.

Proposal 4:

- RAN4 to focus on NW controlled SSB adaptation firstly

  - Suggest to wait for RAN1 progresses on whether MAC-CE and/or DCI can be supported to indicate SSB adaptation transmission on top of the RRC reconfiguration

3.1.3 SSB adaptation scenario in time domain

The two options come from the agreements achieved in RAN1 #117, copied as below

Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

And the proposals in relation to two SSB configurations can be found in the final summary R1-2407552

<Proposal 2.1.2a-rev1> For adaptation of SSB in time-domain, support SSB adaptation based on two SSB configurations where at least one configuration is active - At least SSB periodicity can be same or different between the SSB configurations - ssb-PositionsInBurst" and "ss-PBCH-BlockPower" are same between the SSB configurations - FFS: whether ssb-PositionsInBurst" and "ss-PBCH-BlockPower" can be different between the SSB configurations is supported - FFS: other settings between the SSB configurations - FFS: whether more than one SSB configuration can be active

From our understanding, the main difference of the two options lies on what solution to do the adaptation. For example,

- For pure option 1, one solution is that one configuration is provided to NES-capable UE initially, and DCI/MAC-CE may carry the indication/configuration of which SSB configuration subset to be used/not to be used to perform the periodicity adaptation. In this sense, maybe additional processing time is needed

- For pure option 2, one solution is that multiple configurations are provided to NES-capable UE through RRCReconfiguration initially, one configuration can be considered as the default one. When it comes to adaptation, gNB activate/deactivate the additional configuration. Besides, based on option 2 not only periodicity, broader SSB-related parameters would be expected to adapt. From this, seems that option 2 can enable more scheduling flexibility, better UE experience and more NW energy saving gain.

- As the RAN1 agreement says, Option 1 and option 2 are not completely separated, option 2 can be deemed one of the solutions/implementations to realize option 1 for some functionalities/in some scenarios.

Observation 3: Option 2 can enable broader SSB-related parameters adaptation and more scheduling flexibility, but is more complicated, and it is one of the solutions to realize option 1 for some functionalities

Since some issues are under discussion in RAN1, including: option 1 applicable scenarios and SSB transmission pattern of the two SSB configurations, we proposal

Proposal 5: Suggest to wait for RAN1 agreements on the applicable scenarios and functionalities of option 1 and option 2 (SSB adaptation based on two SSB configurations) for SSB adaptation

3.2 Scenario clarification – others

As we discussed in our previous papers, the applicable cases for SSB adaptation presented in RAN1 contain: Serving cell with only Rel-19 NES-capable UE, that is NES only cell and serving cell with both legacy and Rel-19 NES-capable UE, that is NES cell. For UEs in RRC connected mode, to minimize/avoid the impact on the legacy UE, one solution is to indicate “cell barred” on a MIB (cellbarred) to the legacy UE. Then only Rel-19 NES-capable UE is served in the cell area. The legacy UEs can be barred based on NW choice or are forbidden to camp on the NES cell via other ways and would try to reselect a different cell (e.g., an overlaid cell, the overlaid cell is used as a PCell for legacy UEs).

Observation 4: Based on RAN1 discussions, there are two applicable cases for adaptation of SSB in time domain. 1) Serving cell with only Rel-19 NES-capable UE; 2) Serving cell with both legacy and Rel-19 NES-capable UE

While, for on-demand SSB, both no always-on SSB transmission and always-on SSB transmission cases are considered on SCell. And for PRACH adaptation mechanism, additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs were determined. In this sense, in our opinion, it is natural to say: support adaptation mechanisms of SSB in time-domain for cell with both legacy and Rel-19 UE and cell with only Rel-19 UEs

Proposal 6: For the SSB adaptation deployment assumption, RAN4 to confirm whether the following applicable case should be considered to define the new requirements (if any)

- Serving cell with only Rel-19 NES-capable UE

- Serving cell with both legacy and Rel-19 NES-capable UE

At the same time, from our understanding, not only intra-cell but also inter-cell case can be involved in Rel-19 NES, and there could be an applicable case for inter-cell deployment is that Rel-19 NES (only) cell within the coverage of sync normal cell 1. From this,

Proposal 7: For adaptation of common signal/channel transmissions, RAN4 to discuss:

- Which deployment assumption should be considered to define the new requirements (if any)

  - Intra-cell and/or inter-cell deployment

3.3 Whether and how to define requirements for “transition period”

From our understanding, the change of the SSB periodicity will present the transition of measurement procedure/state. After X ms period, UE can ready to receive the second state (i.e., adapted/legacy) SSB to measure. The corresponding RRM transition requirements including transition delay and/or interrupts at state transitions shall be discussed. To be specific, when the UE transitions between any two states when changing SSB periodicity configuration, the UE shall meet the transition requirement. The reason why we mentioned interruption is that some scenarios including frequency domain adaptation (e.g, inter-frequency) and ss-PBCH-BlockPower adaptation were not precluded in RAN1.

Proposal 8:

- RAN4 to discuss the transition period requirements by SSB periodicity adaptation

  - FFS: Requirements on interrupts at state transitions

3.4 L3 requirements for “transition period”/ after “transition period”

3.4.1 Mis-match of adapted SSB transmission and SMTC window

From our understanding, for L3 measurement, not only the “transition period”, but also the mis-match issue of adapted SSB transmission and SMTC window may arise. More specifically,

- Based on the current spec. 38.331, the UE shall not consider SSB outside the SMTC occasion for RRM measurement

#### 5.5.2.10 Reference signal measurement timing configuration On the indicated ssbFrequency, the UE shall not consider SS/PBCH block transmission in subframes outside the SMTC occasion for RRM measurements based on SS/PBCH blocks and for RRM measurements based on CSI-RS except for SFTD measurement (see TS 38.133 [14], clause 9.3.8).

- NW to configure UE SMTC in MeasObjectiveNR, and legacy SMTC is periodicity pattern, which can not well adapt to the adapted SSB, which the periodicity can be dynamically changed via L1/L2 signalling during the transmission period.

Fig. 1. The mis-match of adapted SSB and SMTC window

In this sense, since the legacy SMTC configuration may not be updated as fast as SSB adaptation (MAC-CE/DCI), if the adapted SSB does not fall into the configured SMTC window, the UE cannot perform L3 measurement based on the adapted RSs even it receives the indication signaling used to trigger adapted SSB transmission, as the Fig.1 says.

Observation 5:  Due to the mis-match of adapted SSB and SMTC window, if the adapted SSB triggered by MAC-CE/DCI does not fall into the configured SMTC window, the UE cannot perform L3 measurement based on the adapted SSBs

The ongoing solutions from RAN1/RAN2 perspective are at least:

- UE autonomously adjust the SMTC periodicity based on the latest SSB periodicity;

- Dedicated SMTC supporting SSB adaptation;

- Dynamic SMTC adaptation configuration

- Explicitly configure the offset of the adapted SSB pattern together with the periodicity

From our understanding, different solutions may bring different impact on RRM requirements. Since we already sent a LS to RAN2 to ask the SMTC impact, we’d better to wait for the RAN2 reply LS on how to cope with the mis-match issue by using which SMTC mechanism (s).

Proposal 9: RAN4 to wait RAN2 reply LS on how to cope with the mis-match of adapted SSB transmission and SMTC window by using which SMTC mechanism (s).

3.4.2 The possible UE measurement behavior

Fig.2 The possible UE measurement behavior

- Assume that: 1) The UE receives the DCI-based/MAC-CE based SSB adaptation indication in the middle of the overall measurement period T SSB_measurement_period#1. 2) When the UE receives the indication signaling, after Time Gap 1, the UE believes the SSB resource is transmitted with periodicity #2

  - After receiving the indication signaling and after Time Gap 1 (i.e., at starting time A), the first candidate additional SSB resources (periodicity #2) are activated Time Gap 1 can be signaling processing time

Since UE perform measurements by taking, e.g., 5 consecutive samples using Rx beam X and averaging these samples (5 samples L1 measurement filter), the UE behaviours can be:

- Both SSB resources (SSBs with periodicity #1, and SSB with periodicity #2), the NES-capable UEs can to measure. While, the new measurement rules, the combination/sample average rules of measurement results based on two SSBs with different periodicities shall be further discussed in RAN4

- Assume that: 1) The UE receives the DCI-based/MAC-CE based SSB adaptation indication in the middle of the overall measurement period T SSB_measurement_period#1. 2) When the UE receives the indication signaling, after Time Gap 2, the UE believes the SSB resource is transmitted with periodicity #2

  - After receiving the indication signaling and after Time Gap 2 (i.e., at starting time B), the candidate additional SSB resources (periodicity #2) are activated beyond the first whole configured periodicity unit, Time Gap 2=Time Gap 1+offset

From our understanding, there is no need to combine the measurement results based on two SSB resources with different periodicities. But

- Whether to/How to define the NES related measurement period (Based on the assumption of RAN2 solved the mis-match of SSB and SMTC successfully)

- Whether to/How to define NES related TFirstSSB

Proposal 10: For L3 requirements for transition period, the possible UE measurement behavior can be:

- If the first candidate additional SSB resource is activated after receiving the indication signaling and after Time Gap 1

  - Both SSB resources, the NES-capble UEs can to measure.

  - Whether to/How to define the new measurement rules and the combination/sample average rules of measurement based on two SSBs with different periodicities shall be further discussed in RAN4

  - FFS: Time Gap 1

- If the candidate additional SSB resources are activated beyond the first whole configured periodicity unit after receiving the indication signaling and after Time Gap 2

  - No need to combine the measurements

  - How to define the NES related measurement period

  - Whether to/How to define NES related TFirstSSB

  - FFS: Time Gap 2

Besides, under NES cell case, if we consider option 2: adaptation based on two SSB configurations where two configurations are active, then for NES-capable UE, whether to receive and measure the legacy SSB resources in addition to NES-based SSB resources after “transition period” is needed to consider, which is similar as on-demand SSB always-on SSB case. While, in such case, we think maybe resources overlapped case will be introduced due to the flexible configurations

Proposal 11: If adaptation based on two SSB configurations where two configurations are active, for L3 requirements after transition period, the possible UE measurement behavior can be:

- NES-capable UEs to measure these additional NES-based SSB resources with the highest priority

- Both SSB resources (legacy SSBs and adapted SSBs) the NES-capable UEs can to measure. FFS how to/whether to combine the measurements, FFS how to define the NES based measurement period

- Based on UE implementation. Follow the measurement rules concluded in on-demand SSB.

3.5 SCell activation requirement for SSB adaptation

Based on the SSB adaptation SCell operation we discussed in our previous contributions

In practice, for SSB adaptation SCell operation, the UE can receive the adaptation indication at #1 Before SCell activation command reception (SCell deactivate state) #2 Along with MAC-CE activation command; #3 During SCell activation procedure but after MAC-CE activation command reception; #4 After/When SCell activation is completed and SCell is activated. The similar procedure as on-demand SSB based SCell activation. From our understanding, there is no need to consider # 3 SSB time adaptation indication case. Since the activation/deactivation is opportunistic, we have concerns on whether additional complexities will be introduced for requirements definition if SSB time adaptation happens during the SCell activation procedure.

Proposal 12: No need to define RRM requirements for the case: UE receive the adaptation indication during SCell activation procedure but after MAC-CE activation command reception

For other cases, considering the commonality between on-demand SSB and adaptation SSB, we think the MAC CE based SCell activation case can also be considered as the starting point for SSB adaptation SCell operation.

However, since the following issues were discussed but without any clear conclusions in RAN1: How NW to indicate SSB adaptation, by RRC/MAC-CE/DCI; How to adapt SSB transmission to reduce the SCell activation delay; Unified/Common MAC-CE framework for on-demand SSB and adaptation SSB

From this, we have

Proposal 13: For Rel-19 NES-capable UE’s SCell

- RAN4 to focus on MAC CE based SCell activation case as starting point for SSB adaptation SCell operation

- RAN4 to study the SCell activation and deactivation delay impact.

  - Combine the RAN1 conclusions of on-demand SSB and SSB adaptation to define the corresponding SCell requirements

3.6 L1 Requirements for “transition period”/after “transition period”

For L1 requirements, M=1 and M=3 are both contained. Considering M=1 is for one shot, M=3 is more appropriate.

Proposal 14: To define the L1 requirements for “transition period”/after “transition period”, M=3 assumption should be considered.

4 Conclusion

In this contribution, we provided our viewpoints to requirement impact analyses on NES adaptation. The following observations and proposals are obtained:

Observation 1: Based on RAN1 agreements on SSB adaptation:

- Rel-19 NES-capable UE in idle mode is not supported

- For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, no RAN1 spec. impact

Observation 2: Based on the simulation results provided in TR 38.864, larger periodicity can bring BS energy saving gains

Observation 3: Option 2 can enable broader SSB-related parameters adaptation and more scheduling flexibility, but is more complicated, and it is one of the solutions to realize option 1 for some functionalities

Observation 4: Based on RAN1 discussions, there are two applicable cases for adaptation of SSB in time domain. 1) Serving cell with only Rel-19 NES-capable UE; 2) Serving cell with both legacy and Rel-19 NES-capable UE

Observation 5:  Due to the mis-match of adapted SSB and SMTC window, if the adapted SSB triggered by MAC-CE/DCI does not fall into the configured SMTC window, the UE cannot perform L3 measurement based on the adapted SSBs

Proposal 1: There is no need to discuss the RRM requirements for the scenarios of Rel-19 NES-capable UEs in idle mode

- Whether to discuss the RRM requirements for the scenarios of Rel-19 NES-capable UEs in inactive mode should wait for RAN1 progress

Proposal 2: There is no need to discuss the RRM requirements related to Cell DTX for connected mode Rel-19 NES-capable UEs

Proposal 3:

- Suggest RAN4 to consider legacy SSB burst periodicity as starting point to define the requirements

- For discussion on RRM requirements impacts based on longer SSB adaptation periodicity can wait for RAN1 progress on whether longer SSB burst adaptation periodicity can be supported

Proposal 4:

- RAN4 to focus on NW controlled SSB adaptation firstly

  - Suggest to wait for RAN1 progresses on whether MAC-CE and/or DCI can be supported to indicate SSB adaptation transmission on top of the RRC reconfiguration

Proposal 5: Suggest to wait for RAN1 agreements on the applicable scenarios and functionalities of option 1 and option 2 (SSB adaptation based on two SSB configurations) for SSB adaptation

Proposal 6: For the SSB adaptation deployment assumption, RAN4 to confirm whether the following applicable case should be considered to define the new requirements (if any)

- Serving cell with only Rel-19 NES-capable UE

- Serving cell with both legacy and Rel-19 NES-capable UE

Proposal 7: For adaptation of common signal/channel transmissions, RAN4 to discuss:

- Which deployment assumption should be considered to define the new requirements (if any)

  - Intra-cell and/or inter-cell deployment

Proposal 8:

- RAN4 to discuss the transition period requirements by SSB periodicity adaptation

  - FFS: Requirements on interrupts at state transitions

Proposal 9: RAN4 to wait RAN2 reply LS on how to cope with the mis-match of adapted SSB transmission and SMTC window by using which SMTC mechanism (s).

Proposal 10: For L3 requirements for transition period, the possible UE measurement behavior can be:

- If the first candidate additional SSB resource is activated after receiving the indication signaling and after Time Gap 1

  - Both SSB resources, the NES-capble UEs can to measure.

  - Whether to/How to define the new measurement rules and the combination/sample average rules of measurement based on two SSBs with different periodicities shall be further discussed in RAN4

  - FFS: Time Gap 1

- If the candidate additional SSB resources are activated beyond the first whole configured periodicity unit after receiving the indication signaling and after Time Gap 2

  - No need to combine the measurements

  - How to define the NES related measurement period

  - Whether to/How to define NES related TFirstSSB

  - FFS: Time Gap 2

Proposal 11: If adaptation based on two SSB configurations where two configurations are active, for L3 requirements after transition period, the possible UE measurement behavior can be:

- NES-capable UEs to measure these additional NES-based SSB resources with the highest priority

- Both SSB resources (legacy SSBs and adapted SSBs) the NES-capable UEs can to measure. FFS how to/whether to combine the measurements, FFS how to define the NES based measurement period

- Based on UE implementation. Follow the measurement rules concluded in on-demand SSB.

Proposal 12: No need to define RRM requirements for the case: UE receive the adaptation indication during SCell activation procedure but after MAC-CE activation command reception

Proposal 13: For Rel-19 NES-capable UE’s SCell

- RAN4 to focus on MAC CE based SCell activation case as starting point for SSB adaptation SCell operation

- RAN4 to study the SCell activation and deactivation delay impact.

  - Combine the RAN1 conclusions of on-demand SSB and SSB adaptation to define the corresponding SCell requirements

Proposal 14: To define the L1 requirements for “transition period”/after “transition period”, M=3 assumption should be considered.

5 References

[1] R4-2416860 “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei