3GPP TSG-RAN WG4 Meeting#113                                R4-2419200

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.21.3.1

Source: 	ZTE Corporation, Sanechips

Title: 	TP to TR38.769: Ambient BS Rx requirements

Document for:	Approval

Introduction

In the RAN#102 meeting, Study on solutions for Ambient IoT (Internet of Things) in NR has been approved and it’s expected to start the normative work from this April meeting. During the last RAN4#110bis meeting, due to the TU limitation, RF requirement impacts for Ambient IoT BS was not discussed yet, in this contribution, we would like to share some further consideration on this issue.

Discussion

- 2.1 General aspects for A-IoT BS in D1T1

- As shown in the following figure, for D1T1 scenario, there are three different deployment assumptions(e.g. D1T1-A1, D1T1-A2 and D1T1-B), therefore there will be different kind of hardware implementation for the transmission and reception accordingly and also corresponding different applicability of RF requirements. Please find the detailed in the following table from our understanding.

Figure 1. the illustration of D1T1-A1/A2 and B

- Table 2.1-1. The applicability of Tx and Rx requirement for different D1T1 assumption

| - Assumption for D1T1 | - Tx | - Rx |
| --- | --- | --- |
| - D1T1-A1 | - To define Tx only for Micro BS1 | - To define Rx requirement for Micro BS2 - CW cancellation could be done different from D1T1-A2 and similar as D1T1-B. |
| - D1T1-A2 | - To define both Tx and Rx for Micro BS, CW cancellation is done internally. |  |
| - D1T1-B | - To define both Tx and Rx for Micro BS, CW cancellation could be different from D1T1-A2 |  |

- Proposal 1: consider the applicability of Tx and Rx requirement for different D1T1 assumption as following and BS need to declare its supported D1T1 assumption.

| - Assumption for D1T1 | - Tx | - Rx |
| --- | --- | --- |
| - D1T1-A1 | - To define Tx only for Micro BS1 | - To define Rx requirement for Micro BS2 - CW cancellation could be done different from D1T1-A2 and similar as D1T1-B. |
| - D1T1-A2 | - To define both Tx and Rx for Micro BS, CW cancellation is done internally. |  |
| - D1T1-B | - To define both Tx and Rx for Micro BS, CW cancellation could be different from D1T1-A2 |  |

- 2.2 RF architecture and corresponding RF requirement impacts for A-IoT BS

- In the following section, we would like to share some initial understanding on the potential RF architecture for A-IoT BS with individual RF hardware assumption in D1T1-A1/A2 and D1T1-B. Regarding the self interference method from CW towards backscattering signal reception, it is not limited to antenna isolation, other RFIC approach or RF/digital IC approach is also not precluded.

- Figure 2.2-1. potential RF architecture of D1T1-A1

- Figure 2.2-2. potential RF architecture of D1T1-A2 and A1

- Figure 2.2-3. potential RF architecture of D1T1-B [TDM operation between Tx and Rx]

- 2.3 The feasibility study for A-IoT BS

- In the previous RAN4 meeting and RAN task, there were some discussion on the feasibility study for A-IoT BS especially from CW cancellation similar as SBFD DL sub-band to UL sub-band interference analysis. As we know that A-IoT BS will receive the CW signal from the transmitter side. CW interference could be mainly dominated by phase noise model. In the following figure 2.3-1, we proposed the phase noise model according to FR2 phase model and update it according to its operating frequency according to the Lesson theory. Based on the initial analysis from phase noise model according to the following figure 2.3-1, the CW leakage would be roughly around 50dB (0dB-[-91+10*log10(15*10^3)]) which is much better than 28dBc assumption at UE side.

- In addition, the spatial isolation and RF CW cancellation or digital CW cancellation at receiver side could be considered. The details could be found in the following table 2.3-1. The feasibility study is not complete yet, however it seems very challenging without any RF/digital CW cancellation receiver side. In addition, instead of assuming 1dB desens target, 6dB desens target is assumed in the following feasibility table since 6dB desens is widely used for lots of receiver requirement. This 6dB desens target could be also used for CW cancellation from other CW transmission node.

- Figure 2.3-1. Phase noise model at 1GHz [7.5kHz: -117dBc/Hz，90kHz: -128dBc/Hz]

- Table 2.3-1. the feasibility study for A-IoT BS

|  |  | ZTE | ZTE |
| --- | --- | --- | --- |
|  | BS TX Power dB(PTX ) = ① dBm | 30 | 30 |
| Frequency isolation at TX | Frequency isolation capability dB(α_(SI-frequency,TX) ) = ② dBc | 50 | 50 |
|  | Frequency isolation techniques used | DPD utilized | DPD utilized |
| Spatial isolation | Spatial isolation capability dB(α_(SI-spatial) ) = ③ dBc | 60 | 60 |
|  | Spatial isolation techniques used | TX/RX panel separation | TX/RX panel separation |
| TX Beam nulling /isolation | TX Beam nulling /isolation in TX CW signal transmission dB(α_(SI-beam) ) = ④ dBc | 0 | 0 |
|  | DL EIRP impact due to beam nulling in TX CW signal transmission | N/A | N/A |
| Self-interference leakage in gNB RX backscattering reception due to non-ideal TX, measured at RX ant. dB(P_(SI_leakage) ) (Note 1) |  | -110 | -110 |
| RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dB(α_(SI-RFIC) ) = ⑤ dBc | 0 | 0 |
|  | RF IC capability and other tech. in RX sub-band dB(α_(SI-RFIC) ) = ⑧ dBc | 0 | 0 |
|  | RF IC techniques and other tech. (before LNA) |  |  |
|  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  |  |
| Self-Interference signal in gNB TX subband, measured at the input of LNA dB(P_(SI_TXSB) ) (Note 1) |  | -30 | -30 |
| Blocker Suppression at RX | Frequency isolation capability dB(α_(SI-frequency,RX) )= ⑥ dBc | 140 | 140 |
|  | Frequency isolation techniques | digitial Filtering | digitial Filtering |
|  | Rx IIP3 capability (dBm) |  |  |
|  | Rx IM3 contribution (dBm) |  |  |
|  | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  |  |
| Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dB(P_(SI_Selectivity) )(Note 1, 2) |  | -170 | -170 |
| RX Beam nulling /isolation | RX Beam nulling /isolation in RX sub-band dB(α_(SI-beam) ) = ⑨ dBc | 0 | 0 |
|  | RX sensitivity degradation caused by RX beam nulling | N/A | N/A |
| Digital IC dB(α_(SI-digtial) ) = ⑦ dBc |  | 30 | 30 |
| Overall RSIC capability dB(RSIC) (Note 1) |  | 140.0 | 140.0 |
| Noise | Noise Figure (dB) | 10 | 10 |
|  | UL bandwidth (KHz) | 15 | 180 |
|  | Noise Floor ⑩dBm | -122.2 | -111.4 |
| Residual Interference budget with 6 dB desens target (⑪dBm=⑩dBm-6dB) |  | -117.5 | -106.7 |
| Required RSIC budget (①-⑪dBc) |  | 147.5 | 136.7 |

- Observation 1: based on the initial feasibility study, it seems feasible for CW cancellation with 6dB desens  target compared without CW interference under the 180kHz backscatttering signal reception.

- 2.4 RF requirement impacts for A-IoT BS

- Table 2.4-1. The initial analysis for RF requirement of A-IoT BS in D1T1.

| Tx part |  |
| --- | --- |
| Base station output power | To follow the FR1 MR and LA BS output power limitation and power accuracy requirement; |
| Output power dynamics | Some power boosting for OOK signal might be needed for in-band/guard band operation of A-IoT service if A-IoT BS share the same hardware with eMBB PRBs; |
| Transmit ON/OFF power | The transmit ON-OFF power is only limited for TDD bands, however A-IoT BS might need to send the R2D signal and CW signal in the sequential way, then some transition period might be needed for the switch between R2D signal transmission and CW transmission in D1T1-A1 and D1T1-A2. |
| Transmitted signal quality | The legacy transmit frequency error requirement could be reused for A-IoT BS. EVM for OOK signal is not needed since OOK signal is not mapped by the legacy constellation, instead it’s reflected in the envelope level as following figure. Tag also don’t need to have channel estimation and equalization to decode the OOK signal, instead of the power stability or power accuracy for OOK ON signal and OOK OFF signal is important and also power difference between OOK ON and OOK OFF is essential to ensure the tag OOK detection performance. Figure 2.4-1. time domain waveform for OOK-4 In addition, as pointed out in [3], RFID uses ASK modulation with PIE data encoding. The ASK modulation mask was defined as shown in Figure 2.4-2 below. The pulsewidth (PW), Modulation Depth ((A–B)/A), RF Envelope Ripple (M), RF envelope Rise time (tr,10–90), and RF envelope fall time (tf,10–90) are the major requirements. Figure 2-4-2: Interrogator-to-Tag RF envelope For A-IoT BS EVM requirement, we propose to define the EVM requirement similar as RFID with OOK signal considered. The specific EVM requirement could be finalized at WI phase. TAE requirement is not needed for R2D signal transmission or CW signal transmission. |
| OBW | The legacy OBW requirement could be reused for A-IoT BS; |
| Tx intermodulation | Based on some operator’s initial feedback on 900MHz indoor deployment, it seems that general Tx intermodulation requirement for A-IoT BS operating at 900MHz is not needed. FFS for other frequency e.g. 2GHz. |
| ACLR | Based on the initial simulation results, 45dBc ACLR requirement should be sufficient. |
| Operating band unwanted emissions | This depends on the conclusion of ACLR and carrier bandwidth requirement etc. |
| Transmitter spurious emissions | The legacy transmitter spurious emission requirement could be reused since this is coming from the ITU regulatory definition. |
| Rx part |  |
| REFSENS | The REFSENS requirement might be not based on the throughput metric and it should be dependent on the miss detection ratio and false alarm detection ratio instead if without any HARQ-ACK feedback. In addition, the impacts on CW signal transmission on D2R reception should be also taken into account especially for D1T1-A2. The feasibility study for Tx-Rx rejection from CW signal to D2R could be found in section 2.3. |
| Dynamic range requirement | Similar analysis for REFSENS requirement. IoT level could be further discussed in the WI phase. |
| ACS, | Based on the initial simulation results, 46dBc ACS requirement might be problematic for SINR degradation of A-IoT BS receiver performance. Maybe some guard band could be provided to mitigate the in-band emissions from NR UE UL transmission towards the A-IoT BS receiver. |
| Blocking requirement | This depends on further coexistence study. |
| OOBB | Don’t see the reason not to reuse the -15dBm CW signal as interference signal of OOBB requirement. For f_OOBB requirement, this could be further discussed once we have more clear assumption on A-IoT BS. |
| RX IMD | This is somehow similar as Tx intermodulation requirement. |
| Rx spurious emission | The legacy receiver spurious emission requirement could be applicable. |
| ICS | We don’t see the necessity to have this requirement for standalone A-IoT BS or in-band/guard band operation with the individual RF hardware similar as standalone NB-IoT ICS requirement. |

Conclusions

In this contribution, we shared some initial views on the RF requirement for A-IoT BS in D1T1 and proposals are made as following:

- Proposal 1: consider the applicability of Tx and Rx requirement for different D1T1 assumption as following and BS need to declare its supported D1T1 assumption.

| - Assumption for D1T1 | - Tx | - Rx |
| --- | --- | --- |
| - D1T1-A1 | - To define Tx only for Micro BS1 | - To define Rx requirement for Micro BS2 - CW cancellation could be done different from D1T1-A2 and similar as D1T1-B. |
| - D1T1-A2 | - To define both Tx and Rx for Micro BS, CW cancellation is done internally. |  |
| - D1T1-B | - To define both Tx and Rx for Micro BS, CW cancellation could be different from D1T1-A2 |  |

- Observation 1: based on the initial feasibility study, it seems feasible for CW cancellation with 6dB desens  target compared without CW interference under the 180kHz backscatttering signal reception.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410597, WF on impacts of A-IoT on RF requirements, Approved.

- R4-2413282	RF requirements for Ambient IoT BS, Huawei,

- [4] R4-2417088, WF on RF requirement impact of A-IoT, Huawei, Approved.

- R4-2414305,WF on RF requirements for A-IoT, Huawei, approved.

- R4-2410597,WF on impacts of A-IoT on RF requirements, Approved.

Text proposal

## <<Start of Change>>

6.7.2.2 Ambient IoT BS -RX

#### 6.7.2.2.1 Reference sensitivity level

Regarding the reference sensitivity level for A-IoT BS, RAN4 reached the following consensus:

- For BS type 1-C,

PREFSENSdBm=−174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]

-	BW is the transmission bandwidth for D2R.

-	NF is the noise figure of A-IoT BS, equal to 10 dB for Medium Range BS and 13 dB for Local Area BS.

-	IM is the implementation margin, equal to 2dB.

-	SNR is the SNR value for D2R decoding, the detailed metrics would be discussed in the WI phase.

-	Desens target is considered for CW signal interference cancellation in D1T1 scenario. The desens level for CW signal cancellation might be different for D1T1-A1/D1T1-A2 and D1T1-B. For D1T1-A2, CW signal interference is coming from self-interference and also potential CW interference from other nodes transmitting CW signal. For D1T1-A1, the CW signal interference is coming from other nodes transmitting CW inside the topology. For D1T1-B, the CW signal interference is coming from CW nodes transmitting outside topology. FFS how to test CW signal in the conducted receiver testing.

#### 6.7.2.2.2 Dynamic range

Regarding the dynamic range requirement, this is agreed to be discussed in WI phase.

#### 6.7.2.2.3 In-band selectivity and blocking

Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus:

- ACS requirement and the interference level shall be determined by RAN4 co-existence study and wanted signal configuration and its corresponding configuration could be postponed to WI phase.

- In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, the wanted signal configuration and its corresponding configuration could be postponed to WI phase.

#### 6.7.2.2.4 Out-of-band blocking

Regarding Out-of-band blocking requirement, the existing OOBB requirement is still applicable for A-IoT BS except for the wanted signal for OOBB requirement.

#### 6.7.2.2.5 Receiver spurious emissions

Regarding the receiver spurious emission requirement, this requirement might differ as following:

- For D1T1-A1 and D1T1-B, the receiver spurious emission requirement is still needed due to is half duplex operation mode.

- For D1TA-A2, due to the simultaneous transmission of CW signal and D2R reception, if the spurious emission requirement of CW signal is exceeding the receiver spurious emission requirement, then the receiver spurious requirement is not needed similar as FDD BS otherwise this requirement is still needed to regulate the receiver RF performance.

#### 6.7.2.2.6 Receiver intermodulation

- Regarding the general intermodulation requirement, it is agreed to postpone to WI phase due to the potential relationship between D2R operation assumption e.g. single tone or multiple tone D2R assumption.

- Regarding the NBB intermodulation requirement, it is agree to necessary due to the potential Narrow band blocking at the operating frequency range, however its detailed requirement could be postponed to WI phase.

#### 6.7.2.2.7 In-channel selectivity

Regarding the receiver in-channel selectivity requirement, [this is agreed to be discussed in WI phase.]

## <<End of Change>>