3GPP TSG-RAN WG4 Meeting#113                              R4-2419208

Orlando, USA, 18th – 22th Nov, 2024

Agenda item:	7.11.3

Source: 	ZTE Corporation, Sanechips

Title: 	Further discussion on OTA test enhancement

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In last RAN4 meeting, we have some initial discussions on the necessity of improvement of co-location issues. In this contribution, we would like to share further views on the OTA co-location requirements.

Sub-topic 1-1: CLTA definition Way forward: Potential outcomes of discussion on co-location reference antenna concept to be considered as alternatives to CLTA, rather than replacing it in Rel-19 OTA test specifications. Sub-topic 1-2: BS-to-BS isolation analysis Way forward: BS-to-BS isolation analysis can be further discussed in this WI, with no additional objectives needed. Sub-topic 1-3: Beam steering consideration for co-location scenario Way forward: Evaluation of the beam steering impacts is considered as applicable to the co-location scenarios analyses. Applicable frequency range within FR1 is FFS. Sub-topic 1-4: TX IMD co-location Way forward: Continue analysis of potential consequences due to TX IMD co-location requirement removal from Rel-19 specification. Applicable frequency boundary for TX IMD requirement removal (within FR1) is FFS.

Discussion

As we discussed in the last RAN4 meeting, the following issues could be further discussed to identify the necessity for co-location reference antenna improvement.

  - .Coupling loss between two AAS BS in FR1 high bands

First of all, NR BS coexisting with non-AAS BS in FR1 high bands are not very common. Instead, two AAS BS coexisting with each other are more ubiquitous which are one candidate issue captured in the agreed WF [1]. Based on the above consideration from the practical deployment perspective, we conducted some coupling loss measurement shown as following:

- (left figure) Two AAS BS are separated in the vertical domain (10cm separation as worst assumption).

- (Right figure) Two AAS BS are separated in the horizontal domain (10cm separation as worst assumption).

Figure 2.1-1. coupling loss measurement for FR1 high bands with separation distance as 10cm from edge to edge

Figure 2.1-2a. antenna ports for coupling loss measurement with vertical separation [expected worst coupling loss]

Figure 2.1-2b. antenna ports for coupling loss measurement with horizontal separation [expected worst coupling loss]

According to the above testing conditions/assumptions, some initial measurement results for FR1 high bands are provided in the following Table 2.1-1. As shown in the following table, if two NR AAS BS are placed with vertical separation which is quite typical for the mast installation, then the coupling loss would be larger than 50dB under the vertical isolation (closet antenna elements as shown in Figure 2.1-2a). However if two NR BS are placed with horizontal separation which is typical for roof based installation, the coupling loss is still larger than 30dBc (closet antenna elements as shown in Figure 2.1-2b). It also depends on the BS’s implementation, for different BS RF architecture, coupling loss among different antenna ports are varying from 31dB to 46dBc.

Table 2.1-1. summary of measured coupling loss different frequency ranges for FR1 WA BS

| Antenna type | 2.6G-BS1 | 2.6G-BS2 | 3.5G-BS1 | 3.5G-BS2 | 4.9G-BS1 |
| --- | --- | --- | --- | --- | --- |
| Isolation in vertical domain(dB) | <-54 | <-57 | <-52 | <-53 | <-56 |
| Isolation in horizontal domain | <-31 | <-40 | <-34 | <-35 | <-46 |

Observation 1:if two NR AAS BS are placed with vertical separation which is quite typical for the mast installation, then the coupling loss would be larger than 50dB with10cm separation distance.

Observation 2: if two NR BS are placed with horizontal separation which is typical for roof based installation, the coupling loss is still larger than 30dBc and the coupling loss among different antenna ports are varying from 31dB to 46dBc.

  - .The typical co-location deployment

During the previous RAN4 meeting, there were some discussion whether the side by side is still typical scenario for co-location deployment especially considering that Tx and Rx of SBFD BS might be not at the same height.

First of all, the side by side co-location deployment should be still typical for the roof based installation even though Tx and Rx of SBFD BS might be somehow separated deployed to ensure its Tx/Rx isolation as shown in Figure 2.2-1. The minimum horizontal separation distance among DUT and CLTA as 10cm from edge to edge in the specification is still the worst assumption to ensure the largest interfering signal coming into the DUT. Indeed, according to the AAU/RRU installation guidance, the minimum horizontal separation distance should be 0.6m and minimum vertical separation distance should be 0.2m which are both larger than the specified 0.1m separation distance. In other words, the co-location requirement itself in the specification is still very stringent requirement compared with the practical deployment.

Figure 2.2-1. antenna installed on top of the roof

In addition, as shown in Figure 2.2-2 and Figure 2.2-3, there are two other co-location deployment. The 1st one is different operators sharing the same mast and different operators as shown in Figure 2.2-2 utilize the different platform or heights to generate the 3 sector or more sectors to provide the coverage. The vertical separation distance would vary from 0.35m to 0.6m which is highly dependent on the installation restrictions from the mast.

Figure 2.2-2. antenna installed on the mast

The 2nd one as shown in Figure 2.2-3 is the different operators sharing the same BS tower and different operators utilize the different platform to generate the 3 sector or more sectors to provide the coverage as shown in Figure 2.2-3, the vertical separation distance could be in meters level which should be most relaxed co-location requirement compared with the previous two deployments.

Figure 2.2-3. antenna installed on top of the BS tower

Observation 3: the side by side co-location deployment should be still typical for the roof based installation

Observation 4: according to the AAU/RRU installation guidance, the minimum horizontal separation distance should be 0.6m and minimum vertical separation distance should be 0.2m which are both larger than the specified 0.1m separation distance. The co-location requirement itself in the specification is very stringent requirement compared with the practical deployment.

Observation 5: it might be very difficult to have Tx and Rx installed in the different heights of BS mast or tower according to the legacy deployment practice;

  - .Difference between AAS BS vs AAS BS and AAS vs co-location reference antenna

In the previous discussions, one of the critical issue is to identify the difference between AAS vs AAS BS and AAS vs co-location reference antenna. In the following section,we would like to provide the further analysis from both transmitter and receiver requirements perspective.

| Clause number | Requirement | Co-location reference antenna operation | Type |
| --- | --- | --- | --- |
| 6.5 | OTA transmit ON/OFF power for FR1 | Measure emission | Mandatory |
| 6.7.5.3 6.7.5.5 | OTA spurious emission: Protection of the BS receiver of own or different BS | Measure emission | Optional based on declaration |
|  | OTA spurious emission: Co-location with other base stations |  |  |
| 6.8 | OTA transmitter intermodulation | Inject the interferer signal | Mandatory |
| 7.6.3 | OTA out-of-band blocking: Co-location with other base stations | Inject the interferer signal | Optional based on declaration |

Figure 2.3-1. the legacy co-location reference antenna for OFF power and spurious emission requirement

For the legacy co-location reference antenna, the received power at co-location reference antenna output should be Ptx+G_Tx-Coupling_loss+G_Rx_CLTA.

Figure 2.3-2. the proposed new co-location reference antenna for OFF power and spurious emission requirement

For the proposed new co-location reference antenna for FR1 high bands, received power at co-location reference antenna output should be Ptx+G_Tx-Coupling_loss+G_Rx_sub-array.

The main difference for OFF power and spurious emission measurement would be G_Rx_CLTA and G_Rx_sub-array pattern. These two received gain for passive antenna of CLTA and AAS sub-array would be quite different. In other words, the measured spurious emission or transmitter OFF power from CLTA cannot be reused or applicable for AAS BS co-locating with AAS BS at sub-array level.

Observation 6: These two received gain for passive antenna of CLTA and AAS sub-array would be quite different. In other words, the measured spurious emission or transmitter OFF power from CLTA cannot be reused or applicable for AAS BS co-locating with AAS BS at sub-array level.

Figure 2.3-3. the legacy co-location reference antenna for Tx intermodulation and co-location OOBB requirement

For the legacy co-location reference antenna, the received power at sub-array of AAS BS should be Ptx+G_Tx_CLTA -Coupling_loss+G_Rx_sub_array

Figure 2.3-4 the proposed new co-location reference antenna for Tx intermodulation and co-location OOBB requirement

For the proposed new co-location reference antenna for FR1 high bands, received power at sub-array of AAS BS should be Ptx+G_Tx_AAS -Coupling_loss+G_Rx_sub_array.

The main difference for Tx intermodulation and co-location reference antenna would be G_Tx_CLTA and G_Tx_AAS antenna pattern. These two passive antenna and AAS BS pattern would be quite different. G_Tx_CLTA should provide much more stringent requirement for it since it provides the much wider beam in azimuth plane towards the victim AAS BS with narrow beam. In other words, from Tx intermodulation and co-location OOBB requirement, it seems that the legacy CLTA requirement would provide more stringent conformance testing for it.

Observation 7: These two passive antenna and AAS BS pattern would be quite different, from Tx intermodulation and co-location OOBB requirement, it seems that the legacy CLTA requirement would provide more stringent conformance testing for it.

  - .Initial simulation results for coupling loss in FR1 high bands

During the last RAN4 meeting, there were some discussions on the technical analysis for coupling loss in FR1 high bands. According to the simulation assumption agreed for Rel-18 SBFD coexistence study, we provided some initial evaluation results for coupling loss at 4GHz. Based on the simulation results got far, the coupling loss is definitely larger than 50dBc instead of 30dBc in FR1 WA, MR and LA scenario.

FR1 WA BS:

FR1 MR BS:

FR1 LA BS:

Observation 8: Based on the initial simulation results for FR1 high bands, coupling loss is expected to be larger than 50dBc regardless from array to array or array to antenna port.

Conclusions

In this contribution, we shared some further views on co-location reference antenna concept in RAN4 and proposals are made as following:

Observation 1:if two NR AAS BS are placed with vertical separation which is quite typical for the mast installation, then the coupling loss would be larger than 50dB with10cm separation distance.

Observation 2: if two NR BS are placed with horizontal separation which is typical for roof based installation, the coupling loss is still larger than 30dBc and the coupling loss among different antenna ports are varying from 31dB to 46dBc.

Observation 3: the side by side co-location deployment should be still typical for the roof based installation

Observation 4: according to the AAU/RRU installation guidance, the minimum horizontal separation distance should be 0.6m and minimum vertical separation distance should be 0.2m which are both larger than the specified 0.1m separation distance. The co-location requirement itself in the specification is very stringent requirement compared with the practical deployment.

Observation 5: it might be very difficult to have Tx and Rx installed in the different heights of BS mast or tower according to the legacy deployment practice;

Observation 6: These two received gain for passive antenna of CLTA and AAS sub-array would be quite different. In other words, the measured spurious emission or transmitter OFF power from CLTA cannot be reused or applicable for AAS BS co-locating with AAS BS at sub-array level.

Observation 7: These two passive antenna and AAS BS pattern would be quite different, from Tx intermodulation and co-location OOBB requirement, it seems that the legacy CLTA requirement would provide more stringent conformance testing for it.

Observation 8: Based on the initial simulation results for FR1 high bands, coupling loss is expected to be larger than 50dBc regardless from array to array or array to antenna port.

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- R4-2319398, Background and technical challenges with BS RF co-location requirements in relation to development of NCR RF co-location requirements, Ericsson, Noted

- R4-1703389, “On BS to BS co-location/co-siting background”, Ericsson

- R4-1706766,Co-location isolation between different frequency bands, Ericsson, Noted

- R4-2406088,Way Forward for [110bis][311] NR_BS_RF, ZTE, approved.

- R4-2404259,OTA test enhancements - CLTA, Nokia

- R4-2409955, Way forward for [111][311] NR_BS_RF, Approved.

- R4-2416574,Way Forward for [112bis][303] NR_BS_RF_Part2_CLTA, Huawei, approved.