3GPP TSG-RAN WG4 Meeting#113                             R4-2419209

Orlando, USA, 18th – 22th Nov, 2024

Agenda item:	7.11.4.1

Source: 	ZTE Corporation, Sanechips

Title: 	TP to TR38.908: Clause 6.4 spatial sampling grid

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In last RAN4 meeting, we reached lots of consensus for U6GHz EIRP mask as captured in [5], however there are still lots of open issues left for further discussions. In this contribution, we would like to share further understandings on these remaining issues.

Issue 2-8 Expected EIRP sampling grid for average EIRP - Recommended for further discussion: - Need further discussions Issue 2-9 Other related with conformance testing declaration and RF channels - Recommended for further discussion: - The following issues to be considered: - the number of mechanical down-tilt and its corresponding angular coverage range; - For each angular coverage range, the number of beams for conformance testing and its corresponding weighting factor; - To balance the number for beams for conformance testing/complexity and test accuracy; - The impacts of potential factors (measurement sampling grid for summation error etc) on EIRP accuracy. - in conformance testing, consideration of converting coordinate system to 3GPP coordinate system, to align with other test cases - Other issues are not precluded. Issue 2-10 Other related with RF channels - Recommended for further discussion: - Need further discussions

Discussion

  - .The equation for the EEIRP calculation

In the previous RAN4 meeting, we have some extensive discussions on EEIRP equation and whether this EEIRP equation should be specified in radian units or degree unit. From our understanding, both approach is expected to have the exact same results. Considering the positioner in OTA chamber is rotated in degree, we are also fine to update the EEIRP equation in degree.

Proposal 1: to define the EEIRP equation in degree as following:

The EEIRP calculation methodology in the discrete form could be calculated according to the following steps. Alternative 1 and alternative 2 will achieve the same results.

Alternative 1:

Step 1: to calculate the average EIRP from different beams for certain direction:

EIRPθn,φm=k=1KwkEIRPkθn,φm,αk,βk

, where wk is a beam weighting and EIRPkθn,φm,αk,βk is the EIRP pattern per measured test beam.

Step 2: to calculate the EEIRP for different elevation bins:

EEIRPi=θHHi−θHLi∙π/180sinθHHi−sinθHLiMN∙n=1Nm=1MEIRPθn,φmcosθn

Note: the order of step 1 and step 2 can be swapped.

Alternative 2:

Step 1: to calculate the EEIRP for different elevation angular bins per beam basis:

EEIRPi,k=θHHi−θHLi∙π/180sinθHHi−sinθHLiMN∙n=1Nm=1MEIRPθn,φmcosθn

, where wk is a beam weighting and EIRPkθn,φm,αk,βk is the EIRP pattern per measured test beam.

Step 2: to calculate the EEIRP for different elevation bins:

EEIRPi=k=1Kwk∙EEIRPi,k

where

- M is the number of the equally divided intervals over the horizontal (azimuth) range of −180 to +180;

- N is the number of the equally divided intervals over the specified vertical (elevation) range of θHLi to θHHi;

- θHLi is the lowest elevation sampling angle within the θHLi , θHHi;

- θHHi is the highest elevation sampling angle within the θHLi , θHHi;

- θn is the vertical angle measured within the n-th interval out of the vertical range of θHLito θHHi, where the interval is from θHLi+n−1NθHHi−θHLi to θHLi+nNθHHi−θHLi;

- φmis the horizontal angle measured within the m-th interval out of the horizontal range of −180 to +180, where the interval is from −180+360∙m−1M to −180+360∙mM.

Figure 2.1-1: Measured EIRP points and the spherical strip

  - .The number of beams for conformance testing and weighting factor

In the previous RAN4 meetings, there were some initial discussions on how to balance the number for test beams for conformance testing/complexity and test accuracy. Technically speaking, the number of serving beams for practical NR BSs could be up to 300-400 beams considering the finer steering granularity in the elevation domain and the relative coarse granularity in the azimuth domain since its steering range is supposed to range from -60 degree to +60degree. If all these practical serving beams to tested within the OTA chamber, then to be further combined according to the appropriate weighting factors, then the measurement time in total would be quite long compared with Rel-15 TRP or EIRP measurement. In addition, if considering repeating the measurement among different mechanical down-tilt if needed, then the measurement time for EEIRP mask would be further longer which is not quite efficient for the BS conformance testing. The detailed analysis for the measurement time would be discussed in section 2.4 accordingly.

In the attached excel sheet, we provided some further evaluation results for the comparison between different test beam patterns.

Proposal 2: propose to use the beam set D with 9 testing beams for EEIRP conformance testing.

  - .The declaration of mechanical downtilt and its corresponding coverage range

As mentioned in the ITU-R resolution requirements, the EEIRP mask requirement should be applicable for any mechanical down-tilt which is reasonable since NR BSs deployed in different targeted scenarios are supposed to be configured with different mechanical down-tilts to provide the corresponding coverage. If NR BS is capable to be deployed in lots of different scenarios, then NR BS should fulfill the EEIRP mask requirement at its supported mechanical down-tilt. In short, the concept proposed in [3] seems more comprehensive solutions to consist of both coverage angular range and steering ranges for each mechanical down-tilt.

Table 2.2-1: Manufacturer declarations

| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) Φmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) Φmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) θmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) θmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). | x | x | X |
| --- | --- | --- | --- | --- | --- |

Proposal 3: to consider the manufacture declarations in Table 2.2-1 for different mechanical down-tilt and its the corresponding coverage angular range for the declaration.

As mentioned before, to repeat the EEIRP measurement under the different mechanical downtilt, the measurement time would also increase significantly, therefore it is essential to consider the reduction of measurement time for different mechanical downtilt. In the following tables 2.2.2a/b/c/d, some initial EEIRP measurement comparison under the different mechanical downtilts are provided for the initial analysis. It should be noted that electrical testing beams are kept the same for different mechanical downtilt. It could be observed according to the following simulation results, the EEIRP measurement would decrease when the mechanical downtilt increase. It could be understandable since the lower sidebloes and grate lobes steering towards direction over the horizontal plane. In other words, EEIRP measurement under the mechanical downtilt should be the most stringent scenario expect for beam steering upwards the horizontal.

Table 2.2-2a: EEIRP measurement comparison for different mechanical down-tilt

| Vertical angle range | Beams set to be tested [R4-2409118] Beam set A |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Mechanical downtilt | 0 degree | 1 degree | 2 degree | 4 degree | 6 degree |
| 0° - 5° | 26.18 | 24.82 | 23.35 | 20.57 | 18.78 |
| 5° - 10° | 17.44 | 16.73 | 16.22 | 15.26 | 14.33 |
| 10° - 15° | 14.11 | 13.60 | 13.17 | 12.47 | 11.77 |
| 15° - 20° | 11.86 | 11.48 | 11.10 | 10.46 | 9.92 |
| 20° - 30° | 9.43 | 9.16 | 8.88 | 8.30 | 7.75 |
| 30° - 60° | 5.35 | 5.07 | 4.78 | 4.21 | 3.65 |
| 60° - 90° | -0.79 | -1.31 | -1.83 | -2.84 | -3.84 |
| NOTE: for the different mechanical down-tilt, the beam steering range doesn’t change in this simulation. |  |  |  |  |  |

Table 2.2-2b: EEIRP measurement comparison for different mechanical down-tilt

| Vertical angle range | Beams set to be tested [R4-2409118] Beam set B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Mechanical downtilt | 0 degree | 1 degree | 2 degree | 4 degree | 6 degree |
| 0° - 5° | 26.45 | 25.30 | 24.02 | 21.45 | 19.59 |
| 5° - 10° | 17.20 | 16.41 | 15.96 | 15.29 | 14.48 |
| 10° - 15° | 14.02 | 13.43 | 12.90 | 12.20 | 11.69 |
| 15° - 20° | 11.64 | 11.35 | 10.99 | 10.20 | 9.62 |
| 20° - 30° | 8.91 | 8.65 | 8.43 | 8.03 | 7.51 |
| 30° - 60° | 4.68 | 4.43 | 4.16 | 3.57 | 2.97 |
| 60° - 90° | -1.60 | -2.07 | -2.54 | -3.53 | -4.58 |
| NOTE: for the different mechanical down-tilt, the beam steering range doesn’t change in this simulation. |  |  |  |  |  |

Table 2.2-2c: EEIRP measurement comparison for different mechanical down-tilt

| Vertical angle range | Beams set to be tested [R4-2409118] Beam set C |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Mechanical downtilt | 0 degree | 1 degree | 2 degree | 4 degree | 6 degree |
| 0° - 5° | 26.83 | 25.12 | 23.20 | 19.63 | 17.71 |
| 5° - 10° | 18.03 | 17.24 | 16.60 | 15.25 | 14.32 |
| 10° - 15° | 14.65 | 14.14 | 13.75 | 12.84 | 11.75 |
| 15° - 20° | 12.47 | 11.96 | 11.57 | 11.14 | 10.40 |
| 20° - 30° | 10.20 | 9.93 | 9.59 | 8.82 | 8.32 |
| 30° - 60° | 6.73 | 6.42 | 6.13 | 5.61 | 5.05 |
| 60° - 90° | 3.22 | 2.65 | 2.12 | 1.17 | 0.33 |
| NOTE: for the different mechanical down-tilt, the beam steering range doesn’t change in this simulation. |  |  |  |  |  |

Table 2.2-2d: EEIRP measurement comparison for different mechanical down-tilt

| Vertical angle range | Beams set to be tested [R4-2409118] Beam set D |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Mechanical downtilt | 0 degree | 1 degree | 2 degree | 4 degree | 6 degree |
| 0° - 5° | 25.73 | 24.39 | 22.88 | 19.63 | 17.30 |
| 5° - 10° | 17.13 | 16.04 | 15.63 | 15.34 | 14.36 |
| 10° - 15° | 14.88 | 14.00 | 13.08 | 12.06 | 11.92 |
| 15° - 20° | 12.31 | 12.25 | 11.92 | 10.64 | 9.65 |
| 20° - 30° | 9.39 | 8.99 | 8.78 | 8.60 | 8.17 |
| 30° - 60° | 5.91 | 5.70 | 5.46 | 4.82 | 4.12 |
| 60° - 90° | 1.79 | 1.33 | 0.86 | -0.15 | -1.25 |
| NOTE: for the different mechanical down-tilt, the beam steering range doesn’t change in this simulation. |  |  |  |  |  |

In addition, during the previous RAN4 discussions, some companies clarified that EEIRP measurement under the different mechanical downtilt could be post-processed by one EIRP measurement over the testing beams since NR BS in the chamber would be also rotated for EIRP measurement. In other words, the EIRP measurement under the different mechanical downtilt could be indirectly measured by the spatial EIRP sampling in the OTA chamber. For example, in the following figure 2.2-1,

- for the mechanical downtilt 0 degree, its electrical test beam in the elevation domain is 0, 3, 6, 9, 12,

- for the mechanical downtilt 3 degree, its electrical test beam in the elevation domain is 0, 3, 6, 9,

- for the mechanical downtilt 6degree, its electrical test beam in the elevation domain is 0, 3, 6;

Indeed the test beams under the mechanical downtilt 3, 6 could be tested when the mechanical downtilt as 0 degree with BS rotate in 3 and 6 degree in the elevation angle for EIRP measurement in the OTA chamber.

Observation 1: if electrical testing beams under different mechanical tilt are the same or subset of full set beam, EEIRP measurement under different mechanical tilt could be post-processed to save the measurement time.

Figure 2.2-1. beam sets under the different mechanical down-tilt

However for different mechanical downtilt, the electrical beams might be different according to the practical deployment. For example, in the following figure 2.2-2,

- for the mechanical downtilt 0 degree, its electrical test beam in the elevation domain is 0, 3, 6, 9, 12,

- for the mechanical downtilt 3 degree, its electrical test beam in the elevation domain is -3, 0, 3, 6, 9,

- for the mechanical downtilt 6degree, its electrical test beam in the elevation domain is -6,-3, 0, 3, 6;

Figure 2.2-2. beam sets under the different mechanical down-tilt

Since the electrical testing beams are somehow different under the different mechanical downtilt and also not subset of any other mechanical downtilt, therefore it is not exchangeable or could be post-processed by full set testing beam measurement. Even though from our initial expectations, even though the number of testing beams under different mechanical downtilt are the same, the EEIRP at mechanical downtilt 6 degree still achieve the lowest EEIRP measurement since its sidelobe from antenna element is supposed to be lowest one.

In short, if the testing beams under the different mechanical tilt are somehow different, then EEIRP measurement under different mechanical tilt could not be post-processed by single EIRP measurement in the full sphere or upper sphere.

Observation 2: if electrical testing beams under different mechanical tilt are different, EEIRP measurement under different mechanical tilt could not be post-processed by single EIRP measurement in the full sphere or upper sphere.

In addition, as we know that, for the mechanical downtilt, this is quite dependent on the operators’ deployment, it is also difficult to specify the any conformance testing requirement related with mechanical tilt. What can we do in RAN4 is to try to save the measurement efforts under the different mechanical tilt. From our understanding, the following two clarification NOTE could be added in the conformance testing specification.

Proposal 4:

- if electrical testing beams under different mechanical tilt are the same or subset of others, EEIRP measurement under different mechanical tilt could be post-processed on top of full set of testing beams to save the measurement time.

- if electrical testing beams under different mechanical tilt are different, EEIRP measurement under different mechanical tilt should be tested separately.

  - .Sampling grid for EEIRP mask measurement

For the maximum measurement/spatial sampling step size in the azimuth angle, it is proposed to reuse the following approach similar as TRP measurement captured in Annex I in TS 38.141-2:

Δϕref=180∘πλDcyl

Dcyl are calculated as:

`	Dcyl=d2+w2

The definition of d, w and h is shown in figure I.2.2-1. The radiation source can be EUT antenna array or the whole of EUT.

Figure 6.4-1: Dimensions of a radiation source: depth (d), width (w) and height (h)

Alternatively, when EUT radiating dimensions are not known,

∆ϕref=λλo BeWϕ

where λo is the wavelength of the wanted signal, and BeWφ are the beamwidth of the wanted signal in the φ-axis,.

BeWφ may be set to beamwidth declared for the OTA BS radiated transmit power requirement.

Proposal 5: it is proposed to reuse the following approach similar as TRP measurement captured in Annex I in TS 38.141-2:

  - .Method of test

During the last RAN4 meeting, there were some discussions on the RF channel for the EEIRP conformance testing, however no consensus has been reached and the following options have been captured in the WF.

Issue 4-4: RF channels for the conformance testing - Proposal 1: B, T [Nokia] - Proposal 2: M. [Huawei] - Proposal 3: T [Ericsson]

During the discussions, some companies argued that the typical BS antenna implementation will use the supported highest frequency as baseline for the antenna design, however some other companies argued that the typical BS antenna implementation might use the middle frequency as baseline for the antenna design. In fact, both might be correct which is total up to the vendors’ implementation. However one common observation on top of these two different antenna design is that NR BS in the supported highest frequency would achieve higher EIRP performance which should be most stringent scenario to be optimized if necessary to comply with EEIRP mask requirement. In general, for the same antenna aperture size, if the operating frequency is increasing, then the achieved EIRP performance will be also higher. However considering the evaluation inputs from other vendors, it seems that difference between B, M and Top channels are similar or close to each other, therefore for sake of progress, it’s also fine for us to consider M RF channels for the conformance testing.

Proposal 6: propose to use the M as RF channels for the conformance testing.

6.9.4	Method of test

6.9.4.1	Initial conditions

The test environment: Normal, see TS 38.141-2, Annex B.2.

RF channels to be tested for single carrier: [M]; see clause 4.9.1. SC carrier, the one characterised by all of requirements below:

- Highest supported Power Spectral Density (PSD) level

Beams to be tested: For each declared CAR test specified test beams, as described inclause 4.9.3.2.

The BS shall be placed on the positioner with mechanical tilt set to 0 degrees.

[6.9.4.2	Procedure

- Place the BS at the positioner, such that blocking effect in the vertical domain shall be minimized.

- Align the manufacturer declared coordinate system orientation (D.2) of the BS with the coordinate system used by the test system.

- Configure the according to applicable test configuration in TS 38.141-2, subclause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1, subclause 4.9.2.

- Set the positioner at location for measurement point (θn,φm) for m=1..M and n=1..N.

- Configure test beam to the directions of k for k=1…K, as defined in clause 4.9.3.2.

- Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

- Repeat step 6 and 7 for all K test beams.

- Repeat step 5, 6, 7 and 8 for all M and N positioner locations.

- Calculate EEIRPi profile from measured EIRPkθn,φm patterns for i=1..7 as described in TS 38.141-2, Annex N.]

NOTE: the procedure order of step4 and step 5 could be swapped. The following measurement procedure are allowed.

- to set the positioner at location to measure multiple beams in the sequential way and rotate the positioner for the EIRP measurements of multiple beams;

- to set the positioner at location to measure the EIRP of single beam and rotate the positioner for the EIRP measurements of single beam and repeat to complete the measurement for multiple beams.

Proposal 7: propose to use the above test procedures for EEIRP measurement using TP measurement as basis.

  - . Measurement uncertainty

Regarding the confidence level of 95% is widely used in the RAN4 conformance testing specification as following,

the Test System shall enable the stimulus signals in the test case to be adjusted to within the specified tolerance and the equipment under test to be measured with an uncertainty not exceeding the specified values. All tolerances and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated. A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

From RAN4 RF requirement definition perspective, we need to define the measurement uncertainty and also test tolerance requirement on top of core requirement. From our understanding, we could still follow the legacy approach adopted for EIRP accuracy requirements as baseline, however considering the EEIRP mask requirements, some additional summation error due to the limited number of testing beams and spatial sampling grid for EEIRP mask measurement.

For the test tolerance requirement for U6GHz EEIRP mask, since this is the regulatory requirement. If we follow the legacy requirement defined for the transmitter spurious emission requirement, then test tolerance should be set as zero.

Proposal 8: follow the legacy approach adopted for TRP measurement uncertainty requirements as baseline, however considering the EEIRP mask requirements, some additional summation error due to the limited number of testing beams and spatial sampling grid for EEIRP mask measurement.

The detailed examples for CATR measurement could be found as following with exact value to be further discussed.

Taking into account the uncertainties highlighted we have for CATR:

| Table 11.2.3.3-1: CATR MU value derivation for OTA BS output power measurement, FR1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |
|  |  | 6.425GHz≤7f≤7.025 GHz |  |  |  | 6.425GHz≤7f≤7.025 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |
| A2-18a | Misalignment and pointing error of BS (for TRP) | 0.30 | Rectangular | 1.73 | 1 | 0.173 |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.26 | Gaussian | 1.00 | 1 | 0.260 |
| A2-2a | Standing wave between BS and test range antenna | 0.21 | U-shaped | 1.41 | 1 | 0.148 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | Gaussian | 1.00 | 1 | 0.001 |
| A2-4a | QZ ripple experienced by BS | 0.09 | Gaussian | 1.00 | 1 | 0.093 |
| A2-12 | Frequency flatness of test system | 0.25 | Gaussian | 1.00 | 1 | 0.250 |
| A2-17 | Measurement system dynamic range uncertainty | 0.51 | Gaussian | 1 | 1 | 0.51 |
| A2-XX | Sampling error for selected teat beams | 1 | Gaussian | 1.00 | 1 | 1 |
| C1-3 | Uncertainty of the network analyzer | 0.20 | Gaussian | 1.00 | 1 | 0.20 |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver | 0.33 | U-shaped | 1.41 | 1 | 0.23 |
| A2-6 | Insertion loss of receiver chain | 0.18 | Rectangular | 1.73 | 1 | 0.10 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | Gaussian | 1.00 | 1 | 0.00 |
| A2-7 | Influence of the calibration antenna feed cable | 0.02 | U-shaped | 1.41 | 1 | 0.02 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.43 | Rectangular | 1.73 | 1 | 0.25 |
| A2-8 | Misalignment positioning system | 0.00 | Exp. normal | 2.00 | 1 | 0.00 |
| A2-18b | Misalignment and pointing error of calibration antenna (for TRP) | 0.50 | Exp. normal | 2.00 | 1 | 0.25 |
| A2-9 | Rotary joints | 0.05 | U-shaped | 1.41 | 1 | 0.03 |
| A2-2b | Standing wave between calibration antenna and test range antenna | 0.09 | U-shaped | 1.41 | 1 | 0.06 |
| A2-4b | QZ ripple experienced by calibration antenna (normal test conditions) | 0.01 | Gaussian | 1.00 | 1 | 0.01 |
| A2-11 | Switching uncertainty | 0.26 | Rectangular | 1.73 | 1 | 0.15 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  | 0.84 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  | 1.65 |
| Total MU |  |  |  |  |  | [2.10] |

* note there may be an error in A2-5a, the value in the SEM TRP budget is larger – check this value

Proposal 9: consider the additional sampling error for selected teat beams as 1dB to derive the total MU for EEIRP MU.

  - . Confidence interval

As we discussed in the previous RAN4 meeting, confidence interval is highly dependent on the number of testing beams and its average value from the measured value. Based on the initial analysis in section 2.1, 9 testing number with the equal weighting factor should be sufficient for both measurement uncertainty and measurement time perspective. If we want to analyze the statistical information for individual EEIRP measurement for 9/25 beams, then we could execute according to the following steps, however this might be not needed and the standard deviation among these testing beams is also very large since the EEIRP measurement on top of 9 beams or 25 beams should be taken as one sample instead of one beam EEIRP measurement as example. For 9 beams or 25 beams measurement, we will only get one EEIRP measurement outcome, there is no statistical information behind for this sample, in other words, we don’t see the necessity to calculate the statistical information with 9/25 beams.

In addition, even we take the 9 beams/25 beams EEIRP measurement as one sample and repeat many times to get sufficient EEIRP samples, then conduct some statistical calculation which doesn’t provide any further restrictive requirement or critical value for this requirement What does it matter is that to ensure the 9 beams or 25 beams measurement is as much as close to reference measurement (e.g. 1000 or 2000 test beams measurement) with the reasonable testing time.

Step 1: Calculate average EEIRP results from 9 beams/25 beams as following:

| Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- |
| 0≤<5 | a |
| 5≤<10 | b |
| 10≤<15 | c |
| 15≤<20 | d |
| 20≤<<30 | e |
| 30≤<<60 | f |
| 60≤<<90 | g |
| NOTE: The requirement shall apply to all supported mechanical tilts. |  |

Step 2: to calculate average EEIRP results per beam basis instead of average across beams

| Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- |
| 0≤<5 | a1, a2,a3,...a9 or a25 |
| 5≤<10 | b1,b2,b3,...,b9 or b25 |
| 10≤<15 | c1,c2,c3,....,c9 or c25 |
| 15≤<20 | d1,d2,d3,...,d9, or d25 |
| 20≤<<30 | e1, e2, e3,...,e9 or e25 |
| 30≤<<60 | f1,f2,f3,...,f9 or f25 |
| 60≤<<90 | g1,g2,g3,...,g9, or g25 |
| NOTE: The requirement shall apply to all supported mechanical tilts. |  |

Step 3: to calculate the standard error per bin basis

- Standard Deviation for bin 1 = sqrt(1/(N-1)[(a1-a)^2+(a2-a)^2+(a3-a)^2+....+(a9-a)^2  or (a25-a)^2 )

- Standard Deviation for bin 2 = sqrt(1/(N-1)[(b1-b)^2+(b2-b)^2+(b3-b)^2+....+(b9-b)^2  or (b25-b)^2 )

- ...

- Standard Deviation for bin 7 = sqrt(1/(N-1)[(g1-g)^2+(g2-g)^2+(g3-g)^2+....+(g9-g)^2  or (g25-g)^2 )

Step 4: to confidence intervals which is dependent on the distribution assumption e..g. normal distribution.

- 1.96*Standard Deviation for bin 1/sqrt(N)

- 1.96*Standard Deviation for bin 2//sqrt(N)

- ...

- 1.96*Standard Deviation for bin 7/sqrt(N)

Proposal 10: for the confidence interval, don’t consider the statistical information within 9/25 test beams.

Proposal 11: to ensure the 9 beams or 25 beams measurement is as much as close to reference measurement (e.g. 1000 or 2000 test beams measurement) with the reasonable testing time.

Conclusions

In this contribution, we shared some further views on the EIRP mask requirement for band n104 and observations / proposals are made as following:

Proposal 1: to define the EEIRP equation in degree as following:

The EEIRP calculation methodology in the discrete form could be calculated according to the following steps. Alternative 1 and alternative 2 will achieve the same results.

Alternative 1:

Step 1: to calculate the average EIRP from different beams for certain direction:

EIRPθn,φm=k=1KwkEIRPkθn,φm,αk,βk

, where wk is a beam weighting and EIRPkθn,φm,αk,βk is the EIRP pattern per measured test beam.

Step 2: to calculate the EEIRP for different elevation bins:

EEIRPi=θHHi−θHLi∙π/180sinθHHi−sinθHLiMN∙n=1Nm=1MEIRPθn,φmcosθn

Note: the order of step 1 and step 2 can be swapped.

Alternative 2:

Step 1: to calculate the EEIRP for different elevation angular bins per beam basis:

EEIRPi,k=θHHi−θHLi∙π/180sinθHHi−sinθHLiMN∙n=1Nm=1MEIRPθn,φmcosθn

, where wk is a beam weighting and EIRPkθn,φm,αk,βk is the EIRP pattern per measured test beam.

Step 2: to calculate the EEIRP for different elevation bins:

EEIRPi=k=1Kwk∙EEIRPi,k

where

- M is the number of the equally divided intervals over the horizontal (azimuth) range of −180 to +180;

- N is the number of the equally divided intervals over the specified vertical (elevation) range of θHLi to θHHi;

- θHLi is the lowest elevation sampling angle within the θHLi , θHHi;

- θHHi is the highest elevation sampling angle within the θHLi , θHHi;

- θn is the vertical angle measured within the n-th interval out of the vertical range of θHLito θHHi, where the interval is from θHLi+n−1NθHHi−θHLi to θHLi+nNθHHi−θHLi;

- φmis the horizontal angle measured within the m-th interval out of the horizontal range of −180 to +180, where the interval is from −180+360∙m−1M to −180+360∙mM.

Figure 2.1-1: Measured EIRP points and the spherical strip

Proposal 2: propose to use the beam set D with 9 testing beams for EEIRP conformance testing.

Proposal 3: to consider the manufacture declarations in Table 2.2-1 for different mechanical down-tilt and its the corresponding coverage angular range for the declaration.

| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) Φmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) Φmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) θmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) θmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). | x | x | X |
| --- | --- | --- | --- | --- | --- |

Observation 1: if electrical testing beams under different mechanical tilt are the same or subset of full set beam, EEIRP measurement under different mechanical tilt could be post-processed to save the measurement time.

Observation 2: if electrical testing beams under different mechanical tilt are different, EEIRP measurement under different mechanical tilt could not be post-processed by single EIRP measurement in the full sphere or upper sphere.

Proposal 4:

- if electrical testing beams under different mechanical tilt are the same or subset of others, EEIRP measurement under different mechanical tilt could be post-processed on top of full set of testing beams to save the measurement time.

- if electrical testing beams under different mechanical tilt are different, EEIRP measurement under different mechanical tilt should be tested separately.

Proposal 5: it is proposed to reuse the following approach similar as TRP measurement captured in Annex I in TS 38.141-2:

Proposal 6: propose to use the M as RF channels for the conformance testing.

Proposal 7: propose to use the above test procedures for EEIRP measurement using TP measurement as basis.

NOTE: the procedure order of step4 and step 5 could be swapped. The following measurement procedure are allowed.

- to set the positioner at location to measure multiple beams in the sequential way and rotate the positioner for the EIRP measurements of multiple beams;

- to set the positioner at location to measure the EIRP of single beam and rotate the positioner for the EIRP measurements of single beam and repeat to complete the measurement for multiple beams.

Proposal 8: follow the legacy approach adopted for TRP measurement uncertainty requirements as baseline, however considering the EEIRP mask requirements, some additional summation error due to the limited number of testing beams and spatial sampling grid for EEIRP mask measurement.

The detailed examples for CATR measurement could be found as following with exact value to be further discussed.

Proposal 9: consider the additional sampling error for selected teat beams as 1dB to derive the total MU for EEIRP MU.

Proposal 10: for the confidence interval, don’t consider the statistical information within 9/25 test beams.

Proposal 11: to ensure the 9 beams or 25 beams measurement is as much as close to reference measurement (e.g. 1000 or 2000 test beams measurement) with the reasonable testing time.

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- WRC-23 Provisional Final Acts, https://www.itu.int/dms_pub/itu-r/opb/act/R-ACT-WRC.15-2023-PDF-E.pdf

- R4-2406088,Way Forward for [110bis][311] NR_BS_RF, ZTE, approved.

- R4-2404757,On introduction of protection requirement for FSS UL service operating within 6425 to 7125 MHz

- R4-2409955, Way forward for [111][311] NR_BS_RF, Approved

- R4-2409118,On introduction of OTA spatial emission above the horizon requirement for band n104, Ericsson

- R4-2409073,Discussion on expected EIRP mask above horizon, Nokia

- R4-2409401,Expected EIRP requirements, Huawei, Hisilicon

- TS 37.941

TP

6.1	General

In this section, it mainly provides some technical background for EEIRP conformance testing from the following issues:

- The calculation for EEIRP mask

- Testing beams including both the number of test beams and also weighting factor

- Spatial sampling grid

- The method of test including both testing conditions and also test procedure

- Measurement uncertainty

- Confidence interval

- Other

6.2	The calculation for EEIRP mask

The EEIRP calculation methodology in the discrete form could be calculated according to the following steps. Alternative 1 and alternative 2 will achieve the same results.

Alternative 1:

Step 1: to calculate the average EIRP from different beams for certain direction:

EIRPθn,φm=k=1KwkEIRPkθn,φm,αk,βk

, where wk is a beam weighting and EIRPkθn,φm,αk,βk is the EIRP pattern per measured test beam.

Step 2: to calculate the EEIRP for different elevation bins:

EEIRPi=θHHi−θHLi∙π/180sinθHHi−sinθHLiMN∙n=1Nm=1MEIRPθn,φmcosθn

Note: the order of step 1 and step 2 can be swapped.

Alternative 2:

Step 1: to calculate the EEIRP for different elevation angular bins per beam basis:

EEIRPi,k=θHHi−θHLi∙π/180sinθHHi−sinθHLiMN∙n=1Nm=1MEIRPθn,φmcosθn

, where wk is a beam weighting and EIRPkθn,φm,αk,βk is the EIRP pattern per measured test beam.

Step 2: to calculate the EEIRP for different elevation bins:

EEIRPi=k=1Kwk∙EEIRPi,k

where

- M is the number of the equally divided intervals over the horizontal (azimuth) range of −180 to +180;

- N is the number of the equally divided intervals over the specified vertical (elevation) range of θHLi to θHHi;

- θHLi is the lowest elevation sampling angle within the θHLi , θHHi;

- θHHi is the highest elevation sampling angle within the θHLi , θHHi;

- θn is the vertical angle measured within the n-th interval out of the vertical range of θHLito θHHi, where the interval is from θHLi+n−1NθHHi−θHLi to θHLi+nNθHHi−θHLi;

- φmis the horizontal angle measured within the m-th interval out of the horizontal range of −180 to +180, where the interval is from −180+360∙m−1M to −180+360∙mM.

Figure 6.2-1: Measured EIRP points and the spherical strip

6.3	Test beams

TBD

6.4	Spatial sampling grid

For the maximum measurement/spatial sampling step size in the elevation angle, since the elevation angular range for some elevation bins bin is quite limited, especially for 1st, 2nd, 3rd and 4th elevation bin with only 5 degree angular range, in order to ensure the measurement accuracy of EEIRP measurement in the elevation angle to comply with WRC-23 resolution, RAN4 agree to use 1 degree as maximum measurement/spatial sampling step size in the elevation angle.

For the maximum measurement/spatial sampling step size in the azimuth angle, RAN4 agree to use the following approach similar as TRP measurement captured in Annex I in TS 38.141-2:

Δϕref=180∘πλDcyl

Dcyl are calculated as:

`	Dcyl=d2+w2

The definition of d, w and h is shown in figure I.2.2-1. The radiation source can be EUT antenna array or the whole of EUT.

Figure 6.4-1: Dimensions of a radiation source: depth (d), width (w) and height (h)

Alternatively, when EUT radiating dimensions are not known,

∆ϕref=λλo BeWϕ

where λo is the wavelength of the wanted signal, and BeWφ are the beamwidth of the wanted signal in the φ-axis,.

BeWφ may be set to beamwidth declared for the OTA BS radiated transmit power requirement.

6.5	Method of test

6.5.1	Initial conditions

The test environment: Normal, see TS 38.141-2, Annex B.2.

RF channels to be tested for single carrier: [M]; see clause 4.9.1. SC carrier, the one characterised by all of requirements below:

- Highest supported Power Spectral Density (PSD) level

Beams to be tested: For each declared CAR test specified test beams, as described in clause 4.9.3.2.

The BS shall be placed on the positioner with mechanical tilt set to 0 degrees.

6.5.2	Test procedure

- Place the BS at the positioner, such that blocking effect in the vertical domain shall be minimized.

- Align the manufacturer declared coordinate system orientation (D.2) of the BS with the coordinate system used by the test system.

- Configure the according to applicable test configuration in TS 38.141-2, subclause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1, subclause 4.9.2.

- Set the positioner at location for measurement point (θn,φm) for m=1..M and n=1..N.

- Configure test beam to the directions of k for k=1…K, as defined in clause 4.9.3.2.

- Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

- Repeat step 6 and 7 for all K test beams.

- Repeat step 5, 6, 7 and 8 for all M and N positioner locations.

- Calculate EEIRPi profile from measured EIRPkθn,φm patterns for i=1..7 as described in Annex N.]

NOTE: the procedure order of step 4 and step 5 could be swapped. The following measurement procedure are allowed.

- to set the positioner at location to measure multiple beams in the sequential way and rotate the positioner for the EIRP measurements of multiple beams;

- to set the positioner at location to measure the EIRP of single beam and rotate the positioner for the EIRP measurements of single beam and repeat to complete the measurement for multiple beams.

6.6	Measurement uncertainty

For the measurement uncertainty of EEIRP mask requirement, considering the number of test beams and also spatial sampling grid for the EEIRP calculation, additional EEIRP summation error 1dB is assumed on top of the legacy TRP measurement uncertainty estimation. The detailed MU budget for EEIRP calculation could be found in the following table 6.6-1.

Table 6.6-1. MU budget for CATR MU value derivation

| Table 11.2.3.3-1: CATR MU value derivation for OTA BS output power measurement, FR1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |
|  |  | 6.425GHz≤7f≤7.025 GHz |  |  |  | 6.425GHz≤7f≤7.025 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |
| A2-18a | Misalignment and pointing error of BS (for TRP) | 0.30 | Rectangular | 1.73 | 1 | 0.173 |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.26 | Gaussian | 1.00 | 1 | 0.260 |
| A2-2a | Standing wave between BS and test range antenna | 0.21 | U-shaped | 1.41 | 1 | 0.148 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | Gaussian | 1.00 | 1 | 0.001 |
| A2-4a | QZ ripple experienced by BS | 0.09 | Gaussian | 1.00 | 1 | 0.093 |
| A2-12 | Frequency flatness of test system | 0.25 | Gaussian | 1.00 | 1 | 0.250 |
| A2-17 | Measurement system dynamic range uncertainty | 0.51 | Gaussian | 1 | 1 | 0.51 |
| A2-XX | Sampling error for selected teat beams | 1 | Gaussian | 1.00 | 1 | 1 |
| C1-3 | Uncertainty of the network analyzer | 0.20 | Gaussian | 1.00 | 1 | 0.20 |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver | 0.33 | U-shaped | 1.41 | 1 | 0.23 |
| A2-6 | Insertion loss of receiver chain | 0.18 | Rectangular | 1.73 | 1 | 0.10 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | Gaussian | 1.00 | 1 | 0.00 |
| A2-7 | Influence of the calibration antenna feed cable | 0.02 | U-shaped | 1.41 | 1 | 0.02 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.43 | Rectangular | 1.73 | 1 | 0.25 |
| A2-8 | Misalignment positioning system | 0.00 | Exp. normal | 2.00 | 1 | 0.00 |
| A2-18b | Misalignment and pointing error of calibration antenna (for TRP) | 0.50 | Exp. normal | 2.00 | 1 | 0.25 |
| A2-9 | Rotary joints | 0.05 | U-shaped | 1.41 | 1 | 0.03 |
| A2-2b | Standing wave between calibration antenna and test range antenna | 0.09 | U-shaped | 1.41 | 1 | 0.06 |
| A2-4b | QZ ripple experienced by calibration antenna (normal test conditions) | 0.01 | Gaussian | 1.00 | 1 | 0.01 |
| A2-11 | Switching uncertainty | 0.26 | Rectangular | 1.73 | 1 | 0.15 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  | 0.84 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  | 1.65 |
| Total MU |  |  |  |  |  | [2.10] |

6.7	Confidence intervals

TBD

6.8	Testing requirements

Since the EEIRP mask requirement defined in WRC-23 is the regulatory requirement from RAN4 standardization perspective, the test tolerance requirement should be zero without any further relaxation on top of regulatory limits.

6.9	Others