3GPP TSG-RAN WG4 Meeting#113                             R4-2419214

Orlando, USA, 18th – 22th Nov, 2024

Agenda item:	7.2.4.2

Source: 	ZTE Corporation

Title: 	Discussion on radio and antenna parameters for 14800 to 15350 MHz

Document for:	Approval

Introduction

In the latest ITU-R meeting, WP5D has sent one LS to RAN/RAN4 to request the Parameters of terrestrial component of IMT in the following frequency ranges for sharing and compatibility studies. During the RAN#103 meeting, the corresponding SID for WP5D reply LS [3] is approved to facilitate the technical discussions in RAN4. In this contribution, we would like to share some views on IMT parameters for 14800-15350MHz.

Issu 4-13: ACLR - Potential agreement: - Postpone to next meeting according to the simulation results. Issue 4-16: ACS - agreement: - Postpone to next meeting according to the simulation results. Issue 4-15: Blocking response Potential Agreement: - Further evaluate the power level for in-band requirement based on the simulation - Further evaluate the f_OOBB requirement based on the agreed IBB and OOBB power level;

Discussion

## 2.1. BS RF requirements

### 2.1.1	ACLR

According to the simulation results in clause 4.3 in TR 39.921, it is agreed to specify 38 dB ACLR for 6.425 - 7.125 GHz and 37 dB ACLR for 10.0 - 10.5 GHz. In addition, according to the companion contribution [9] on coexistence evaluation results for 15GHz, we propose to define the BS ACLR requirement around 35dBc.

Table 2.3.1-1: Base station ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 35dBc |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 35dBc |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |

Proposal 1: define BS ACLR requirement as 35dBc;

### 2.1.2	Spectral mask

In the previous RAN4 meeting, we have some initial discussion on spectral mask, however the Cat BS emission mask is still in the square bracket. At least from our understanding, this could be generally used for Europe regions and we could follow the 10-10.5GHz approach to keep this requirement in the SID and further discuss this applicability when we specify the band in the WID phase.

According to the proposed ACLR requirement for 14.8-15.35GHz and also expected adjacent carrier bandwidth, we propose to reuse the spectral mask for 7 and 10GHz for 15GHz

Table 2.3.2-1: Wide Area BS operating band unwanted emission limits for 14.8-15.35 GHz for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | −7dBm−750foffsetMHz−0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -13 dBm | 1MHz |

Table 2.3.2-2: Wide Area BS operating band unwanted emission limits for 14.8-15.35 GHz for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | −7dBm−750foffsetMHz−0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -15 dBm | 1MHz |

Proposal 2: propose to capture the Cat B spectral emission mask in the TR and further discuss its applicability when the band is introduced in the spec.

### 2.1.3	Spurious emissions

According to ERC 74-01 report as following, the lower frequency limit could be defined as 30 MHz and higher frequency limit could be 2nd of harmonic. In addition, similar as 10-10.5GHz, without any specified spurious limit for AAS BS in between 6 and 24 GHz in ERC 74-01 [6], the spurious limits specified for above 24 GHz with upper frequency limited to 26 GHz will be used for 14.8-15.35 GHz as defined in Table 2.3.3-1 and Table 2.3.3-2.

Table 2.3.3-1: BS spurious emission limits for 14.8-15.35 GHz for Category B

| Spurious frequency range | Limit | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz – 1 GHz | -36 dBm | 100 kHz |
| 1 GHz – 18 GHz | -30 dBm | 1 MHz |
| 18 GHz – 2nd harmonic of the upper frequency edge of the DL operating band | -20 dBm | 10 MHz |

Table 2.3.3-2: BS spurious emission limits for 14.8-15.35 GHz for Category A

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -13 dBm | 100 kHz | Note 1 |
| 1 GHz – 2nd harmonic of the upper frequency edge of the DL operating band |  | 1 MHz | Note 1, Note 2 |
| NOTE 1: Bandwidth as in ITU-R SM.329, s4.1 NOTE 2: Upper frequency as in ITU-R SM.329, s2.5 table 1. |  |  |  |

Table 1: Recommended measurement ranges

| Fundamental frequency range | Frequency range for measurements |  |
| --- | --- | --- |
|  | Lower frequency | Upper frequency* |
| 9 kHz - 100 MHz | 9 kHz | 1 GHz |
| 100-300 MHz | 9 kHz | 10th harmonic |
| 300-600 MHz | 30 MHz | 3 GHz |
| 600 MHz - 5.2 GHz | 30 MHz | 5th harmonic |
| 5.2-13 GHz | 30 MHz | 26 GHz |
| 13-150 GHz | 30 MHz | 2nd harmonic |
| 150-300 GHz | 30 MHz | 300 GHz |
| *The test should include the entire harmonic band and not be truncated at the precise upper frequency limit stated |  |  |

| 2.1.6 | Base Stations using AAS and beamforming with integrated antennas operating below 24.25 GHz | 9 kHz f 1 GHz | -36 dBm (Note 4) |
| --- | --- | --- | --- |
|  |  | 1 GHz < f Fupper (see recommend 3) | -30 dBm (Note 4) |
| 2.1.7 | Base Stations using AAS and beamforming with integrated antennas operating above 24.25 GHz | 9 kHz f 1 GHz | -36 dBm (Note 4) |
|  |  | 1 GHz < f 18 GHz | -30 dBm (Note 4) |
|  |  | 18 GHz < f Fupper (see recommend 3) | -20 dBm/10 MHz (other limits apply for specific frequency separations, see Figure 7) (Note 4) |

Proposal 3: propose to capture the Cat B spurious emission mask in the TR and further discuss its applicability when the band is introduced in the spec.

In the previous RAN4 meeting, the offset ΔfOBUE for the boundary between OBUE and spurious emissions is not decided yet, according to the agreed spectral emission mask and also spurious emission requirement, the in-band spurious emission and out of band spurious emission is the same for Cat A and very close for Cat B. In addition, considering the narrow relative channel bandwidth compared with U6GHz and 7GHz, it should be okay to reuse ΔfOBUE = 100 MHz for 15GHz.

Proposal 4: propose to reuse ΔfOBUE = 100 MHz for 15GHz.

### 2.1.4	Sensitivity

Similar as 6GHz and 10GHz discussion, we propose to define BS OTA sensitivity requirement based on the NF and the antenna gain:

EISREFSENS=PkT+10∗log10BW+NF+IM+SNR−G    (dBm)

Where:

-	BW is the configured bandwidth of the FRC,

-	NF is the noise figure,

-	IM is implementation margin not related to antenna array,

-	SNR is the required SNR to reach 95% throughput, and

-	G is the antenna gain including RF losses and 3dB off peak margin.

Proposal 5:propose to define BS OTA sensitivity requirement based on the NF and the antenna gain similar 7 and 10GHz;

### 2.3.7	Blocking response

The in-band blocking levels are reused from existing FR1 requirements.

Table 6.2.3-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 100 | PREFSENS + 6 dB | Wide Area BS: -43 Medium Range BS: -38 Local Area BS: -35 | ±30 | 20 MHz DFT-s-OFDM NR signal 15 kHz SCS, 100 RBs |
| NOTE: PREFSENS depends on the RAT. |  |  |  |  |

For OOBB requirement, in the previous RAN4 meeting, there were some initial discussion on OOBB requirement, however there are no consensus reached so far. Considering the OOBB interference for FR1 BS and FR2-1 BS is also applicable for 15GHz, in other words, we think that we could reuse the existing OOBB interference power level for 15GHz.

Table 10.6.3.1-1: OTA out-of-band blocking performance requirement

| Frequency range of interfering signal (MHz) | Wanted signal mean power (dBm) | Interferer RMS field-strength (V/m) | Type of interfering signal |
| --- | --- | --- | --- |
| 30 to 12750 | EISREFSENS + 6 dB | 0.36 [-15dBm] | CW |
| 12750 to FUL,low – 1500 | EISREFSENS + 6 dB | 0.1 [-30dBm] | CW |
| FUL,high + 1500 to 2nd harmonic of the upper frequency edge of the operating band | EISREFSENS + 6 dB | 0.1 [-30dBm] | CW |

Considering the background of FR1 and FR2 OOOB requirement as following,

FR1:

including the downlink frequency range of the FDD operating band for BS supporting FDD. The ΔfOOB for BS type 1-C and BS type 1-H is defined in table 7.4.2.2-0.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C and at the TAB connector for BS type 1-H.

Table 7.5.2-1: Out-of-band blocking performance requirement for NR

| Wanted Signal mean power (dBm) | Interfering Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- |
| PREFSENS +6 dB (Note) | -15 | CW carrier |

FR2-1:

the aggressor out of band BS is considered to have a total power of 29 dBm, an antenna gain of 26 dBi hence an EIRP in the main beam of 55 dBm. As the beam has high gain and is pointed at UE's on the ground then when considering the interference at a victim BS there is a down tilt loss (LDT) of 13 dB is considered.

With the interferer at a distance (d) of 200 m this gives an interfering field strength of:

E=30EIRP(W)d=30∗10EIRPdBW−LDT10d=30∗1055−30−1310200=0.1V/m

This core requirement is applied from 12.57 GHz to 2nd harmonic of the upper frequency edge of the operating band (excluding the operating band +/- 1.5 GHz).

Proposal 6: to define the OOBB requirement for 15GHz BS as following

| Frequency range of interfering signal (MHz) | Wanted signal mean power (dBm) | Interferer RMS field-strength | Type of interfering signal |
| --- | --- | --- | --- |
| 30 to 12750 | EISREFSENS + 6 dB | 0.36 [-15dBm] | CW |
| 12750 to FUL,low – ΔfOOBB | EISREFSENS + 6 dB | 0.1 [-30dBm] | CW |
| FUL,high + ΔfOOBB to 2nd harmonic of the upper frequency edge of the operating band | EISREFSENS + 6 dB | 0.1 [-30dBm] | CW |

Similar consideration as Tx ΔfOBUE, given the narrow relative channel bandwidth,

Proposal 7: propose to reuse ΔfOOBB = 100 MHz for 15GHz.

### 2.3.8	ACS

According to the simulation results in clause 4.3 in TR 38.921, it is agreed to specify 42 dB ACS for 6.425 - 7.125 GHz and 40 dB ACLR for 10.0 - 10.5 GHz. In addition, According to the companion contribution [9], we propose to define the BS ACS requirement around 38dBc for 14.8-15.35GHz.

Proposal 7:propose to define BS ACS requirement as 38dBc.

## 2.3. UE RF requirements

Regarding the UE RF requirement for 14800-15350MHz, according to the simulation results so far in the companion contribution, whether it is possible to relax ACLR requirement similar as BS side, this is highly dependent on the UE architecture design and also target deployment scenario.

- If FR1 like UE implementation assumption with the indoor ratio as 20%, then it will be very challenging to meet the 5% performance degradation criteria, however to reduce the indoor deployment ratio, then it is possible to relax ACLR requirement (e.g. 10dBc for ACLR R relaxation with 0% indoor deployment).

- For the UE ACS requirement, it seems that DL ACIR requirement could be relaxed around 8-10dBc, in other words, it is acceptable to have further relaxation on top of the legacy 33dBc ACS requirement. In addition, based on the agreed adjacent channel selectivity (ACS) is agreed as 32 dBc for 6425 – 7125 MHz and 31 dBc for 10 - 10.5 GHz, we propose to define the ACS 30dBc for 15GHz..

- Proposal 8: define UE ACS requirement around 30dBc for 15GHz.

Conclusions

In this contribution, we want to share some initial views on IMT parameters for 14800-15350MHz and proposals/observations are made as following:

Proposal 1: define BS ACLR requirement as 35dBc;

Proposal 2: propose to capture the Cat B spectral emission mask in the TR and further discuss its applicability when the band is introduced in the spec.

Proposal 3: propose to capture the Cat B spurious emission mask in the TR and further discuss its applicability when the band is introduced in the spec.

Proposal 4: propose to reuse ΔfOBUE = 100 MHz for 15GHz.

Proposal 5:propose to define BS OTA sensitivity requirement based on the NF and the antenna gain similar 7 and 10GHz;

EISREFSENS=PkT+10∗log10BW+NF+IM+SNR−G    (dBm)

Where:

-	BW is the configured bandwidth of the FRC,

-	NF is the noise figure,

-	IM is implementation margin not related to antenna array,

-	SNR is the required SNR to reach 95% throughput, and

-	G is the antenna gain including RF losses and 3dB off peak margin.

Proposal 6: to define the OOBB requirement for 15GHz BS as following

| Frequency range of interfering signal (MHz) | Wanted signal mean power (dBm) | Interferer RMS field-strength | Type of interfering signal |
| --- | --- | --- | --- |
| 30 to 12750 | EISREFSENS + 6 dB | 0.36 [-15dBm] | CW |
| 12750 to FUL,low – 1500 | EISREFSENS + 6 dB | 0.1 [-30dBm] | CW |
| FUL,high + 1500 to 2nd harmonic of the upper frequency edge of the operating band | EISREFSENS + 6 dB | 0.1 [-30dBm] | CW |

Proposal 7:propose to define BS ACS requirement as 38dBc.

- Proposal 8: define UE ACS requirement around 30dBc for 15GHz.

References

- R4-2400333 Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz.

- RP-210037, Reply LS to WP5D, Approved.

- RP-240787,New SI proposal: Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz and 14.8-15.35 GHz, Approved.

- TR 38.921

- https://www.itu.int/en/ITU-D/Regional-Presence/AsiaPacific/Documents/Events/2017/Aug-ISS2017/PAPER_Workshop_S3_Timur.pdf

- R4-2211224, Introduction of NR licensed band 6425 -7125 MHz, Approved.

- R4-2414283	WF on 15GHz BS parameters, ZTE, Approved.

- R4-2414284	WF on 15GHz UE parameters,Qualcomm, Approved.

- R4-24xxxx Discussion on co-existence evaluation for 14800 to 15350 MHz, ZTE Corporation

Text proposal

<Start of text proposal>

6.5.1	BS antenna characteristics

6.5.1.1		Antenna model

Antenna model for AAS BS is described in clause 6.1.2.3.2.

6.5.1.2	Antenna parameters

In Table 6.5.1.2-1, representable parameter sets relevant for an AAS base station operating within 14800 - 15350 MHz are provided.

Table 6.5.1.2-1: Antenna array parameters

| Parameter | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 5 | 6.4 | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V | 90º for H 65º for V | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º |
| Antenna array configuration (Row × Column) (Note 4) | 4 x 4 | 16 x 24 | 16 x 24 |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 0.5 of wavelength for V | 0.5 of wavelength for H, 2.8 of wavelength for V (Note 5) | 0.5 of wavelength for H, 2.8 of wavelength for V (Note 5) |
| Number of element rows in sub-array | N/A | 4 | 4 |
| Vertical element separation in sub-array (dv,sub) | 0.5 of wavelength of V | 0.7 of wavelength of V | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | N/A | 3 | 3 |
| Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 |
| Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | [2/8] | 17.1 | 7.1 |
| Base station horizontal coverage range (degrees) | +/-90 | +/-60 | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 0-180 | 90-100 | 90-100 |
| Mechanical down-tilt (degrees) | 90 | 6 | 6 |
| Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180°. Note 2: The element gain includes the loss and is per polarization. Note 3: The conducted power per sub-array assumes 16 × 24 sub-arrays and 2 polarizations for urban macro and dense urban cases; the conducted power per element assumes 4 x 4 elements and 2 polarizations for indoor case. Note 4: 16 × 24 means there are 16 vertical and 24 horizontal radiating sub-arrays for urban macro and dense urban cases; 4 x 4 means there are 4 vertical and 4 horizontal radiating elements for indoor case. Note 5: For the case of 4 elements per sub array, dv will be 2.8 wavelengths. |  |  |  |

<End of text proposal>