3GPP TSG-RAN WG4 Meeting#113                              R4-2419222

Orlando, USA, 18th – 22th Nov, 2024

Agenda item:	7.20.2.3

Source: 	ZTE Corporation

Title: 	Discussion on modification of existing Rx requirements for FR1 and FR2-1 for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. In this contribution, we would like to share some further views on these remaining open issues.

Discussion

## 2.1. Rx requirements

### 2.1.1. Reference sensitivity level

During the SI phase, we reached the following consensus how to define the REFSENS requirement for SBFD receiver especially from the degradation from its self interference. From our understanding, 1.0dB performance degradation is acceptable for us considering the feasibility study captured in TR 38.858 and also coexistence assumptions agreed in the SI phase, if degradation is tightened from 1.0dB to 0.5dB which means the required RSIC capability will increased 3dB which would go against with some vendor’s feasibility study. For the details, it could refer to the feasibility study in TR 38.858. Based on the above consideration, we propose to use 1.0dB degradation for SBFD BS REFSENS requirements.

Regarding Reference sensitivity requirement for SBFD-capable BS, due to the self interference caused internally to receiver side, RAN4 reached the following consensus: - For BS type 1-H if supported: The existing requirement for conducted reference sensitivity level shall also be applied to BS in SBFD symbols, i.e, no sensitivity degradation is allowed. - Otherwise, OTA sensitivity requirement could be derived based on the following equation: PREFSENSdBm=−174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]-G - The candidate value [0.5~1.0]dB degradation and final value will be specified in the WI phase. - The following aspects need more discussion during a WI phase - The declaration of maximum TRP for the requirement of OTA sensitivity within SBFD time slot - If OTA sensitivity should be defined considering all of the scenarios including self-interference, inter-site interference and inter-sector interference.

Proposal 1: agree with 1.0dB degradation for SBFD BS REFSENS requirements;

### 2.1.2. Dynamic range requirements

During the SI phase, we reached the consensus on receiver dynamic range requirement for SBFD BS. Compared with the IoT level defined for the legacy BS dynamic range requirements, we need to consider the BS2BS CLI additionally for SBFD since the IoT level of SBFD receiver should be contributed by both uplink signals and BS2BS CLI signals.

Regarding the dynamic range requirement, this requirement is still applicable for SBFD-capable BS. The IoT level and wanted signal power level could be further discussed in the WI phase. WF: - FFS on the followings: - Option 1: RAN4 to discuss if the IoT level and wanted signal power level will be derived based on simulation work. - Option 2: The existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations. - Option 3: for receiver dynamic requirement, both uplink signals and BS2BS CLI signal should be considered for IoT levels.

During the initial simulation results provided as following without considering the co-channel BS2BS beam coordination/nulling scheme, IoT level without or without considering co-channel BS2BS CLI is quite close. In other words, the IoT level from Rel-15 NR BS could be reused.

Proposal 2: for receiver dynamic requirement, the legacy IoT level defined in Rel-15 could be reused.

### 2.1.3. Receiver intermodulation

Regarding the receiver intermodulation requirement, in general, RX intermodulation requirement and the interference levels shall be determined by RAN4 co-existence study, and for the definition of RX intermodulation requirement RAN4 reached the following consensus: - Conducted RX intermodulation: Adopt the existing wanted signal of RX intermodulation requirement by using the existing reference sensitivity level. - OTA RX intermodulation: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

Similar as receiver dynamic range requirement, for Rx IMD requirement, the BS2BS CLI should be also taken into account when specifying the power level for the interference signals.

For the legacy/normal slot/symbols, the existing receiver intermodulation requirement in section 7.7.2 of TS 38.104 are still applicable.

For the SBFD slot/symbols, there might be some self interference and additional Rx intermodulation caused by downlink part especially when considering with CW signal or NBB or general Rx intermodulation signals configured during the Rx IMD test, therefore it’s suggested to have further study for it.

Figure 2.2.3-1: Example of Rx intermodulation requirement for SBFD BS

- Proposal 3: for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal.

- Proposal 4: for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver as shown in Figure 2.2.3-1.

### 2.1.4. IBB requirements

- During the last RAN4 meeting, there were some discussions on IBB evaluation to figure out the appropriate requirement definition. In the following section, we provide some initial simulation results according to the simulation assumption captured in TR 38.858.

- FR1:

Figure 2.1.4-1. the evaluation results for Scenario 1

Figure 2.1.4-2. the evaluation results for Scenario 3

Figure 2.1.4-2. the evaluation results for Scenario 5

Considering the 99% CDF to define the IBB interfering power level, the blocking level over antenna elements or over antenna array is collected as following. It could be observed that the received power level for Scenario 1, namely FR1 Urban macro scenario are much higher than the existing IBB requirement -43dBm which would be quite challenging for SBFD BS to handle the receiver dynamic range together with the REFSENS requirements.

For Scenario 3 indoor scenario, the received power is still close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

| Scenario | Grid shift | Per antenna elements [dBm] | Per antenna array [dBm] | Per sub-array [dBm] |
| --- | --- | --- | --- | --- |
| 1 | 100% | -25.3301 | -19.7623 | TBD |
|  | 10% | -1.5067 | 3.4934 | TBD |
|  | 20% | -7.4879 | -1.6402 | TBD |
|  | 50% | -17.4928 | -13.0954 | TBD |
| 3 | NA | -42.4091 | -34.4724 | TBD |
| 5 | 100% | -38.9889 | -35.7181 | TBD |
|  | 10% | -15.4155 | -11.2276 | TBD |
|  | 20% | -22.3199 | -18.4471 | TBD |
|  | 50% | -32.3749 | -29.5694 | TBD |

- Observation 1: for Scenario 1 FR1 Urban macro scenario, the received power level are much higher than the existing IBB requirement -43dBm which would be quite challenging for SBFD BS to handle the receiver dynamic range together with the REFSENS requirements.

- Observation 2: for Scenario 3 FR1 indoor scenario, the received power is still close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

- Observation 3: for Scenario 5 FR1 micro scenario, the received power under the 100% grid shift is still close to the existing requirement, however for the other grid shift assumption, the received power level (10% grid shift) are much higher than the existing IBB requirement -38dBm which would be quite challenging for SBFD BS to handle the receiver dynamic range together with the REFSENS requirements.

- FR2-1:

Figure 2.1.4-3a. the evaluation results for Scenario 6

Figure 2.1.4-3b. the evaluation results for Scenario 6

Figure 2.1.4-4a. the evaluation results for Scenario 9

Figure 2.1.4-4b. the evaluation results for Scenario 9

Similar as FR1, considering the 99% CDF to define the IBB interfering power level, the blocking level over antenna array and relative power difference between wanted signal and IBB signal is collected as following. It could be observed that the relative power difference between wanted signal and IBB signal in Scenario 6 and Scenario 9 is still close to the existing IBB requirement if without considering 6dB performance degradation of wanted signal. In other words, for Scenario 6 and 9, the received power is still close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

| Scenario | Grid shift | Per antenna array [dBm] | Relative power difference [dBm] |
| --- | --- | --- | --- |
| 6 | 100% | -48.3600 | 25.6302 |
|  | 10% | -36.5674 | 37.2361 |
|  | 20% | -41.2973 | 32.3168 |
|  | 50% | -45.7134 | 27.8767 |
| 9 | N/A | -56.5661 | 16.8709 |

- Observation 4: for Scenario 6 FR2 urban macro scenario and Scenario 9 FR2 indoor scenario, the received power is close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

Conclusions

In this contribution, we want to share some further views on RF requirements for SBFD BS and proposals/observations are made as following:

Proposal 1: agree with 1.0dB degradation for SBFD BS REFSENS requirements;

Proposal 2: for receiver dynamic requirement, the legacy IoT level defined in Rel-15 could be reused.

- Proposal 3: for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal.

- Proposal 4: for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver as shown in Figure 2.2.3-1.

- Observation 1: for Scenario 1 FR1 Urban macro scenario, the received power level are much higher than the existing IBB requirement -43dBm which would be quite challenging for SBFD BS to handle the receiver dynamic range together with the REFSENS requirements.

- Observation 2: for Scenario 3 FR1 indoor scenario, the received power is still close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

- Observation 3: for Scenario 5 FR1 micro scenario, the received power under the 100% grid shift is still close to the existing requirement, however for the other grid shift assumption, the received power level (10% grid shift) are much higher than the existing IBB requirement -38dBm which would be quite challenging for SBFD BS to handle the receiver dynamic range together with the REFSENS requirements.

- Observation 4: for Scenario 6 FR2 urban macro scenario and Scenario 9 FR2 indoor scenario, the received power is close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2409958, Way forward for [111][313] NR_duplex_evo, Approved.

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Approved