3GPP RAN WG4 Meeting #113		R4-24xxxx

# Orlando, US, 18th – 22nd November, 2024

Source: 	Huawei, HiSilicon

Title: 	Simulation methodologies and results of AAS Modelling

Agenda Item:	7.2.5

Document for:	Approval

1 Introduction

WP 5D sent a LS to 3GPP requesting information to make their compatibility studies (R4-2400333), if available, already by June 2024 and final submission June 2025 if the studies are extended.

Considering the system-level simulation work typically conducted in ITU-R WP 5D for sharing and compatibility studies using 3D modelling of antenna pattern, ITU-R WP 5D is seeking information on whether for implementation of IMT BS multiuser spatial beamforming techniques, such as zero-forcing (ZF) or minimum mean-square error (MMSE) based schemes, would be necessary to improve the assessment of interference and the accuracy of studies. If considered necessary, guidance on the process of deriving the necessary beamforming weights for the IMT AAS BS to compute its radiation pattern.

In Aug meeting, we evaluated zero-forcing (ZF) multiuser spatial beamforming method and the antenna radiate pattern towards to a number of interfered-with receivers located in the upper hemisphere, see R4-2413281. Based on the simulation results, ZF impact on the AAS BS gain towards potential interfered-with receivers is minor and the interference is even less than M.2101 considering extended AAS modelling for sub-array.

In Oct meeting, we further provided update assumptions and results based on ITU discussions, see R4-2416281. The assumptions align to multiple company’s co-sign contribution for Oct ITU-R WP5D Meeting in 5D/372. Additionally, the modelling proposal from ITU-R WP5D/380 contribution is also evaluated. All the simulation results show the same observation which zero-forcing (ZF) impact on the AAS BS gain very minor.

After the discussion in #112bis 3GPP RAN4 meeting in Oct, a WF on other issues (MIMO) is approved as below, see R4-2417191.

MIMO modelling

# Agreements:

- It is agreed to put all the simulation methodologies and results with corresponding assumptions in a dedicated subclause in TR 38.922.

- ITU-R M.2101 methodology is sufficient for MIMO modelling in a pure LOS channel.

- Discussion

Proposal 1: The simulation methodologies and results based on R4-2416281 in #112bis 3GPP RAN4 meeting is provided in the Annex to facilitate the work to put all the simulation methodologies and results with corresponding assumptions in a dedicated subclause in TR 38.922.

It should be noted that although ITU-R Rec.M.2101 methodology assumes that LOS channel model between BSs and UEs, the AAS beam formed by this methodology points to randomly deployed UEs in the whole BS cell region. Considering different UE locations introduce different departure angles from the BSs side, it is equally the same to random NLOS signal paths departure angles from statistic interference side. M.2101 model is also a highly efficient model, which can be used to evaluate the aggregate interference comes from hundreds of thousands or even millions of BSs to satellites. Compared to M.2101, RAN1 system-level simulation with NLOS conditions can only support 19 x 3 cells small scale network due to its high complexity and time consuming. There is also no standard NLOS channel model between BS and UE for sharing and compatibility study in ITU. 3GPP TR38901 model may be used, but it’s mainly used for IMT system evaluation instead of sharing studies.

Proposal 2: ITU-R M.2101 methodology is the most suitable and sufficient modelling in NLOS condition for coexistence study between BS and UE.

3 Annex: Simulation methodologies and results of AAS Modelling

Zero-forcing (ZF) beamforming is an appealing solution as it has a moderate computational complexity while achieving spatial multiplexing. This technique decouples the multi-user channel into multiple independent subchannels by nullifying unwanted signals in the direction of other users, ensuring that each user receives their intended signal with minimal interference. Mathematically, the ZF precoder can be derived as the pseudo-inverse of the channel matrix:

| WZF=HHHHH-1 |  |
| --- | --- |

where H represents the channel matrix with its number of rows corresponding to the number of Rx antennas (i.e., UEs) and its number of columns corresponding to the number of Tx antennas (BS antenna ports), and HH is the conjugate transpose of H.

In this paper, Tx antenna number Ntx is 32 (4V8H in Attachment 1Table 3). On the receiving side, the system comprises of a set of 3 UEs, each having 1 antenna. The total number of receive antennas Nrx for all users is 3. H is the 3 × 32 matrix representing the MIMO channel matrix. Each element of H is a complex value that represents the propagation effect. hjk is the propagation coefficient between the kth transmit antenna and the i th receive antenna. WZF is 32 × NRx_MU, where NRx_MU can be 1, 2 or 3 depending on the conditions to allow UEs to join ZF precoding. Traditionally, precoders are designed subject to a total power constraint of the form. In this study, we normalize ZF precoder by diving its column wise Fresenius normwrx_idxF=Tr(wrx_idx.wrx_idx*)1/2.

wrx_idxnormalized=wrx_idx wrx_idx

The total power constraint simplifies the design problem and leads to simple and efficient precoders. Nonetheless, in practice, many systems are subject to individual per antenna power constraints. Additionally, individual per-antenna power constraints are also considered as the sensitivity analysis since each transmit antenna has its own power amplifier limitation in practise.

Channel matrix H between BS and UE link is normally assumed line-of-sight and then AAS beam is formed towards to its serving UEs in ITU-R sharing study. In the paper, the channel matrix H used for ZF beamforming follows similar assumptions and assume all BS and UE links are line-of-sight. Channel matrix H can be generated reflecting the Azimuth/ Zenith angle Of Departure (AoD/ZoD) from each BS to its serving UEs.

More details assumption aligns to ITU-R 5D/372, see Attachment 2.

Figure 1

Geometry distribution of AAS with multiple columns array

dH

z

u

dv

φ

θ

x

y

……

In order to assess the necessity and accuracy of interference comes from IMT systems for ITU-R sharing and compatibility studies, the IMT BS gain towards a number of potential interfered-with receivers located in the upper hemisphere are simulated. The angular separation between sample points is assumed 10 degrees.

Figure 2

10-degree sample points in the upper hemisphere above the BS

Deployment related parameters of BSs and UEs are based on the agreed technical and operational characteristics in the document 5D/716 (Annex 4.4 - WRC-23), and beamforming antenna characteristics are referred to 3GPP liaison statement on parameters for 4400 to 4800 MHZ of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, see 5D/136. More detail simulation parameter assumptions are provided in the Attachment 1.

2.1 Methodology 1 results (based on Attachment 2)

The cumulative distribution functions (CDFs) curves of the IMT BS gain towards the sample points of the different scenarios are shown in Figures 3 with additional individual per-antenna power constraint. It can be seen that the difference between ITU-R M.2101 and ZF gains are minor.

Figure 3

BS gain towards all sample points (composite) with individual per-antenna power constraint

To assess the impact on long- and short-term statistics, the 80th percentile and 99.9th percentile are considered respectively. Table 1 contains the summary of the BS antenna gain difference between ITU-R M.2101 model and ZF beamforming model with & without additional individual per-antenna power constraint. Additional individual per-antenna power constraint influences the simulation results very little. In addition, it shows that ZF beamforming get slightly less antenna gain than M.2101 towards to the sample points with individual per-antenna power constraint:

TABLE 1

BS antenna gain difference between ITU-R M.2101 beamforming model and ZF beamforming (Method 1)

| C band Urban (with subarrays) Scenario | Long-term (80th percentile) | Short-term (99.9th percentile) |
| --- | --- | --- |
| M.2101 | -13.720 | 20.861 |
| ZF without individual per-antenna power constraint | -13.587 | 20.855 |
| ZF with individual per-antenna power constraint | -13.779 | 20.719 |
| M.2101- ZF difference (dB) (without individual per-antenna power constraint) | -0.133 | 0.006 |
| M.2101- ZF difference (dB) (with individual per-antenna power constraint) | 0.059 | 0.142 |

2.2 Methodology 2 results (based on Attachment 3)

The cumulative distribution functions (CDFs) curves of the IMT BS gain towards the target UE are shown in Figures 4. It can be seen that this method can also get reasonable target antenna gain compared to M.2101.

Figure 4

BS gain towards to target UEs

The cumulative distribution functions (CDFs) curves of the IMT BS gain towards the sample points of the different scenarios are shown in Figures 6 without additional individual per-antenna power constraint. It can be seen that the difference between ITU-R M.2101 and ZF gains are minor.

Figure 6

BS gain towards all sample points (composite) without individual per-antenna power constraint (Method 2)

TABLE 2

BS antenna gain difference between ITU-R M.2101 beamforming model and ZF beamforming (Method 2)

| C band Urban (with subarrays) Scenario | Long-term (80th percentile) | Short-term (99.9th percentile) |
| --- | --- | --- |
| M.2101 | -20.810 | 19.527 |
| ZF (Method 2) without individual per-antenna power constraint | -20.992 | 19.197 |
| M.2101- ZF (Method 2) difference (dB) (without individual per-antenna power constraint) | 0.182 | 0.33 |

Attachment 1: Parameters and interim results

1	IMT BS AAS related parameters

Table 2 contains the deployment related parameters of BSs and UEs based on the agreed technical and operational characteristics in the document 5D/716 (Annex 4.4 - WRC-23), and Table 3 contains the beamforming antenna characteristics in 3GPP liaison statement on parameters for 4400 to 4800 MHZ of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, see 5D/136.

TABLE 2

IMT deployment-related parameters

|  | Urban macro |
| --- | --- |
| Base station |  |
| Carrier frequency | 4.5 GHz |
| Channel bandwidth | 100 MHz |
| BS Antenna height | 20 m |
| Cell radius | 400 m |
| Sectorization | 1 sector (Note 1) |
| Frequency reuse | 1 |
| User terminal |  |
| UE height | 1.5 m |
| UE density for terminals that are transmitting simultaneously | 3 UEs per sector |
| Note 1: This study considers only a single-entry scenario. |  |

TABLE 3

IMT base station beamforming antenna characteristics for IMT

|  | Urban Macro |
| --- | --- |
| Antenna pattern | Refer to the extended AAS model in 5D/716 (Annex 4.4 - WRC23) |
| Element gain (incl. Ohmic loss) (dBi) | 6.4 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 4 × 8 (Note 1) |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H 2.1 of wavelength for V |
| Number of element rows in sub-array | 3 |
| Vertical radiating element spacing in sub-array | 0.7 of wavelength |
| Pre-set sub-array downtilt (degrees) | 3 |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-100 (Note 2) |
| Mechanical downtilt (degrees) | 6 |
| Note 1: For the sake of completeness, additional results are provided with no subarrays. By keeping the same physical dimensions, an antenna array of size 12 × 8 elements (Row × Column) is considered. Note 2: The vertical coverage range includes the mechanical downtilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

An extended version of the AAS array antenna model is created to support vertical sub-array geometries with fixed sub-array down-tilt. The model equations are summarized in below Table 4.

Table 4

Extended AAS model

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan Where M and N is corresponding to (Row × Column). |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ Where M and N is corresponding to (Row × Column). |

The IMT network consists of a single site with a single sector. In each snapshot of the Monte Carlo simulation, 3 UEs are uniformly distributed within the sector, and the BS serves each UE with a dedicated beam.

Figure 7

Uniform UE distributions

A minimum 35m BS-UE distance along the ground is simulated and the vertical angle distribution is plotted below.

Figure 8

Probability density function (PDF) of the elevation angle from the BS to the UEs

The max vertical steering angle is considered if the deployed UEs are outside of this limitation provided in Table 3.

3	Configuration of minimum angular separation for ZF

Assuming there are only two UEs served by a BS (UE #1 always at the BS boresight and UE #2 located with an angular separation of 20, 10, or 5 degrees from UE #1), Figure 9 shows the radiation patterns towards UE #1 of a ULA with 8 antenna elements and a spacing of 0.5 wavelengths. A UE minimum angular separation of 20 degrees is deemed appropriate as the antenna gain reduction is negligible.

Figure 9

Radiation patterns of a ULA (8 antenna elements and a spacing of 0.5λ) with and without ZF beamforming

In order to evaluate the influence of minimum azimuth angular separation between serving UEs in Monte Carlo simulation, the UEs are deployed randomly, and AAS beamforming gain towards to target serving UE under different minimum angular separation assumptions is simulated as below.

It’s obvious ZF will decrease the target serving UE RX gain, the larger minimum angular separation the less influence.

Figure 10

AAS beamforming gain towards to target serving UE

Minimum 5 degrees angular separation        Minimum 10 degrees angular separation

Minimum 20 degrees angular separation

In order to avoid obvious antenna gain decrease, this study set minimum 20 degrees as the default assumption.

4	Effectiveness of the ZF beamforming technique

In this section, we validate the effectiveness of the ZF beamforming technique, Figure 11 shows the BS gain towards the co-scheduled UEs when serving a specific UE. As expected, ZF reduces the BS gain towards the co-scheduled UEs.

Figure 11

BS gain towards the co-scheduled UEs when serving a specific UE

Attachment 2: Methodology 1 of ITU-R WP5D/372

- For the k-th UE, generate the uniform planar array (UPA) coefficients, Ukn,m, using the weighting formulation in Rec. ITU-R M.2101 – Table 4, to reflect the UE angles from the BS antenna array in both azimuth and elevation (Note 1):

Ukn,m=expj2πn-1dvλcosθk+m-1dhλsinθksinφk

where, for the k-th UE, θk and φk are the angles of elevation and azimuth, respectively. The ranges of θk and φk are 0,π and [-π,π] radians, respectively, as defined in Rec. ITU-R M.2101. The size of the matrix Ukn,m is N×M as defined in M.2101, where N is the number of rows and M is the number of columns of the antenna array.

- Assess the angular separation between the K UEs in the cell (ITU-R assumption, K = 3) (Note 2).

- For each UE (Note 3), if there is at least another UE which with the minimum angular separation requirement is fulfilled, create a concatenated P×MN channel matrix,  Hk (Note 4):

Hk=hkT …hpTT

where T is the transpose operator and hk corresponds to the vectorization of Ukn,m resulting in a vector of size MN×1 (Note 5). Note that, for simplicity, hkT is always the first row of Hk, and hpT represent the UE(s) that can be paired with the k-th UE.

- For each UE, the zero-forcing beamforming (precoding) matrix WkZF (MN x P) is:

WkZF=HkHHkHkH-1

where Hk represents the channel matrix with its number of columns corresponding to the number of Rx antennas (i.e., UEs) and its number of rows corresponding to the number of Tx antennas (BS antenna ports), and HkH is the conjugate transpose of Hk.

- The entire WkZF matrix can be normalised by diving it by its Frobenius norm, or, doing a column wise normalisation by diving each column with its norm (Note 6).

- The actual precoding weights for each UE (wk) are obtained by selecting the first column of WkZF. The size of wk is MN×1.

- Obtain the individual Zero Forcing beam pattern: The precoding weights for each beam wk of size MNx1 may be used to obtain the k-th zero forcing beam pattern by reordering the column vector wk of size MNx1 into a matrix Wk of size NxM which can be used as element weight in the formulation in Rec. ITU-R M.2101:

Akθ,ϕ=AEθ,ϕ+10log10(|m=1NHn=1NVwkn,m⋅νn,m​2)

where wkn,m refers to the entry in the n-th row and m-th column of weighting matrix Wk and νn,m is the superposition vector as given in the Rec. ITU-R M.2101.

Note that for AAS with sub-arrays, AEθ,ϕ is updated by Asubθ,ϕ:

Akθ,ϕ=Asubθ,ϕ+10log10(|m=1NHn=1NVwkn,m⋅νn,m​2)

where Asubθ,ϕ calculation follows the method defined in the extended AAS modelling in Annex 4.4 to Working Party 5D Chairman’s Report ANNEX 3.

- The resulting composite ZF beam pattern may be obtained by summing up the individual Zero Forcing beam patterns in linear scale (and transforming them back to dB):

AZFθ,ϕ=10log10p=1P10Akθ, ϕ10

where AZFθ,ϕ refers to the composite ZF beam pattern and Akθ,ϕ refers to the k-th individual zero forcing beam pattern.

Note 1: For the computation of Ukn,m, θk and  φk are in local coordinate system (LCS) which accounts for the mechanical downtilt.

Note 2: The total 3D angular separation should be considered to determine the fulfillment of the minimum angular separation requirement. Additionally, only the azimuth angular separation may be considered in cases where the elevation angle does not significantly contribute to the total 3D angular separation.

Note 3: Hk must be constructed for each UE separately, since in some instances, one UE can be paired with all other UEs while another UE could be paired with only a single other UE. The next figure depicts this situation: UE1 can be paired with UE2 and UE3 since the min. angular separation is fulfilled (as shown in (a)), UE2 can only be paired with UE1 (as shown in (b)), and UE3 can only be paired with UE1 (as shown in (c)).

Note 4: Hk only includes the UEs that fulfill the minimum angular separation requirement depending on the size of the BS antenna, so Hk may have two or three columns (P = 2 or 3) in each Monte Carlo snapshot depending on the geometry of the BS and the UEs. For the UEs that cannot be paired with any other UEs (P = 1), Hk is not constructed, and the current beamforming model in Rec. ITU-R M.2101 is used.

Note 5: The vectorization must ensure that the vector of size 1×MN correspond to the concatenation of the rows of Ukn,m:

u11⋯u1m⋮⋱⋮un1⋯unm→concatenation→u11…u1m u21…u2m… un1…unm

Note 6: The total power constraint simplifies the design problem and leads to simple and efficient precoders. However, the magnitude of some elements of wk may be larger than 1/(MN), which can introduce per-antenna port transmit power larger than 1/(MN). In practice, many systems are subject to individual per-antenna port power constraints.

Attachment 3: Methodology 2 of ITU-R WP5D/380

Considering the presented analysis and results, New Zealand has two proposals to ITU-R WP 5D:

1)	Performing a pseudo-inverse for the calculation of the ZF beamforming matrix should be based on the product of wn,mvn,m, as considering  wn,m alone does not consider the complete (entire) AF of the UPA, and therefore, will likely not result in the same pseudo-inverse due to the underlaying differences in the respective quantities of wn,mvn,m vs. wn,m. Therefore, we propose that any pseudo-inverse calculation for ZF beamforming should be based on the product wn,mvn,m.

2)	Any assessment of the impact of ZF beamforming on IMT BS antenna array gain must be based on the complete AF of the UPA. Once this proposal is agreed, at the next meeting of ITU-R WP 5D meeting, New Zealand will provide a cumulative distribution function (CDF) of the IMT BS antenna array gain when the pseudo-inverse is computed with wn,mvn,m relative to wn,m alone.

Reference

[1] ITU-R Recommendation M.2101 Modelling and simulation of IMT networks and systems for use in sharing and compatibility studies, 02/2017

[2] ITU-R 5D/158 Assessment of the effect of zero-forcing on gain distribution for active antenna of IMT BS, France, New Zealand, 11 June 2024

[3] ITU-R 5D/197 Consideration for the active antenna array radiation pattern model for IMT Base Stations, Telefon AB - LM Ericsson, 14 June 2024

[4]	TR 37.842	3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS) (Release 13)

[5]	TR 37.840	3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study of Radio Frequency (RF) and Electromagnetic Compatibility (EMC) requirements for Active Antenna Array System (AAS) base station (Release 12)

[6] ITU-R 5D/372 Proposed methodology to model multi-user beamforming in IMT systems for assessment of its impact on sharing and compatibility studies, Telefon AB - LM Ericsson, etc, 20 Sep 2024

[7] ITU-R 5D/380 Important considerations for pseudo-inverse calculations in zero-forcing beamforming for IMT base stations with AAS , New Zealand , 21 Sep 2024

[8] R4-2413281, “AAS modelling considerations for IMT base stations”, Huawei, HiSilicon, RAN4 112, Maastricht, NL, August, 2024

[9] R4-2416281, “Further consideration and update simulation results for AAS modelling”, Huawei, HiSilicon, RAN4 112bis, Anhui, China, October, 2024