3GPP TSG-RAN WG4 Meeting #113	R4-2419253

# Orlando, US, November 18th – 22th 2024

Source:	Ericsson

Title:	Simulation results for Rel-19 SCM for demodulation

Agenda item:	7.14.2

Document for:	Information

# Introduction

In RAN4#112bis meeting, companies agreed to focus on SU-MIMO PDSCH and PMI cases to compare the performance between different channel models and SCM approaches. The CSI-ports and BS antenna configurations are also discussed in detail and some candidate options are agreed for simulations.

In this contribution, we deliver some performance curves between existing TDL, 901 CDL model and CDL extension model in R4-2415283.

# Simulation

The simulated test cases are 8Tx8Rx with 8 layers PDSCH demodulation.

## 2.1	Parameters

| Parameter |  | Value |
| --- | --- | --- |
| Simulated Channel |  | PDSCH |
| Duplex mode |  | FDD DL only |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |
| UE receiver type |  | SU MMSE |
| (BB) Antenna configuration |  | 8 Tx / 8 Rx |
| Waveform |  | CP-OFDM with normal CP |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |
| MCS |  | 17 (for both CWs) for low correlation 7 for medium correlation |
| Channel model |  | CDL-C Uma FR1 in TR38.827 TDLC300-100 low |
| Active DL BWP index |  | 1 |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PDSCH aggregation factor | 1 |
|  | PRB bundling type | N/A |
|  | PRB bundling size | N/A |
|  | Resource allocation type | Type 0 |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |
|  | Transmission Rank | Fixed to 8 (2CW) |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) |
|  | DMRS ports indexes | {1000-1007} for Rank8 |
| Codebook configuration | CodebookType | typeI-SinglePanel |
|  | CodebookMode | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) | (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) | (4,1) |
|  | RI Restriction | 10000000 rank 8 |
| PDSCH & PDSCH DMRS Precoding | Common parameters | Wideband |
|  | Random | Random precoder |
| Analog impairments |  | None (No TO, no FO, no PA model, no TxEVM, no RxEVM, no IQ imbalance, etc.) |
| Number of HARQ Processes |  | 4 |
| Maximum HARQ transmissions |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  | 2 |
| UE speed |  | 30 km/h |

Following BS and UE antenna configurations are used for simulation.

- CDL (as defined in R4-2415283) with Antenna Array Virtualisation (AAV) using subarray configuration

  - (M,N,P,Ms,Ns)  = (8,8,2,8,2) for 8Tx CSI-RS Ports (option X)

  - (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports (option Y)

- CDL (as defined in R4-2415283), i.e., with AAV without subarray (fully connected)

  - (M,N,P) = (8,8,2) for 8Tx CSI-RS Ports

- CDL (as defined in R4-2415283) without AAV with the following configuration:

  - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

Following are the beam forming configurations:

- Use following 4 beam directions for transmission:

  - 1st strongest beam: AoD = -7.27o, ZoD = 100o.

  - 2nd strongest beam: AoD = -21.82o, ZoD = 100o.

  - 3rd strongest beam: AoD = -35.98o, ZoD = 100o.

  - 4th strongest beam: AoD = -50.97o, ZoD = 100o.

- Full connection: create 4 beam directions above for 8 layers transmission

- 1st strongest beam: Transmit 8 layers by the 1st strongest beam direction above.

- Boresight: Transmit 8 layers by the beam direction: AoD = 0o, ZoD = 100o

## 2.2	Simulation results

### 2.2.1	Total performance comparison

#### BS antenna with 128AE

#### BS antenna with 64AE

#### BS antenna with 8AE

### 2.2.2	Per CW performance comparison

#### BS antenna with 128AE

#### BS antenna with 64AE

#### BS antenna with 8AE

#### Comparison with TDL model

# Conclusions

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

- No table of figures entries found.

# References

[1] 	R4-2416591, Way Forward for [112bis][318] FS_NR_demod_SCM, Nokia

[2]