3GPP TSG-RAN WG4 Meeting #113	R4-2419260

# Orlando, US, 18th – 22nd November, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on the core maintenance of SCell activation delay reduction

Agenda Item:	5.11.1

Document for:	Approval

- Introduction

The core part has been basically finished and good progress were achieved. Still some small maintenance issues with respect to the core part were proposed during last meeting but no consensus. In this paper, we provide our views on the remaining issues of core part.

- Discussion

- Uncertainty part for SCell activation requirement with L3 reporting:

During last meeting, this issue was raised by company since it is relevant how to capture the uncertainty delay in the requirements. In the existing requirements, three uncertainty parts are involved in, i.e. Tuncertainty_MAC, Tuncertainty_RRC and Tuncertainty_SP. Currently the unsure case is when UE sends L1 reports earlier than L3 reports and L3 report is later than TCI activation command.

Under this case, actually NW determines the TCI state based on L1 reports, but UE is unknown about this, so UE is still preparing L3 report and wait for the UL resource scheduling within the waiting window, as long as UE derives UL resource then UE sends the L3 reports. The timeline of this case was shown as below in [1].

Figure 1  Timeline of R18 unknown SCell activation delay reduction with L3 reporting

If following the definition of Tuncertainty_MAC, Tuncertainty_RRC and Tuncertainty_SP defined in 8.3.17 as below, they should be negative value.

- Defined in 8.3.17: - Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to - - First valid L3-RSRP reporting for unknown case, when UE reports valid L3-RSRP - - First valid L1-RSRP reporting for unknown case, when UE does not report L3-RSRP results - Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to - - First valid L3-RSRP reporting for unknown case, when UE reports valid L3-RSRP - - First valid L1-RSRP reporting for unknown case, when UE does not report L3-RSRP results - Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to - - First valid L3-RSRP reporting for unknown case, when UE reports valid L3-RSRP - - First valid L1-RSRP reporting for unknown case, when UE does not report L3-RSRP results

But if following the definition in 8.3.2 as below, they should be positive value.

- Defined in 8.3.2: - Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to - - SCell activation command for known case; - - First valid L1-RSRP reporting for unknown case. - Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to - - SCell activation command for known case; - - First valid L1-RSRP reporting for unknown case. - Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to - - SCell activation command for known case; - - First valid L1-RSRP reporting for unknown case.

- Observation 1: Regarding the value of Tuncertainty_MAC, Tuncertainty_RRC and Tuncertainty_SP,

- If following the definition in 8.3.2, they should be positive value;

- If following the definition in 8.3.17, they should be negative value.

The delay component with negative value is not appropriated and meaningless, so if following the definition in 8.3.17, some additional revision is needed to specify these uncertainty part as zero at this unsure case.

Besides these uncertainty part definition, regarding the requirements of whole delay for this unsure case, if following legacy 8.3.2, then for the delay from TCI state indication to L3 reporting, actually no component to capture this part. While within 8.3.17, the component of TL3, report has already included this part. But under this case, this part actually exists, if we want to reuse the requirement in 8.3.2, which implies the UE would not send L3 reporting after receiving the TCI state indication. The UE behavior and the requirement should be consistent.

So another alternative is to allow the L3 reporting event after receiving the TCI state indication, accordingly, the requirement in 8.3.17 should be followed, the UE behavior and the requirement are consistent. Under this case, all these uncertainty part should be defined as zero.

- Observation 2: If following the requirement of 8.3.2, the UE behavior and requirement is not consistent, the the delay from TCI state indication to L3 reporting actually exists, but no component captures this part in the requirement.

- If following the requirement of 8.3.17, the UE behavior and requirement is consistent.

On the other hand, in the beginning of 8.3.17, it is clearly captured when to apply the requirements defined in 8.3.17 as below:

8.3.17 SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation. The requirements in this clause shall apply for UE supporting l3-MeasUnknownSCellActivation-r18 and reporting valid L3 measurement results after receiving the SCell activation command for unknown SCell. The requirements in this clause shall apply for the UE configured with one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated. Clause 8.3.2 is applied for UE who does not report L3 measurement results after receiving SCell activation command for unknown SCell. - ...

So no matter whether the L1 reporting sent or not, as long as the L3 reporting sent during the window, the requirements defined in 8.3.17 should be followed, unless no L3 reporting sent by UE.

- At this maintenance stage of the WID, Alt 2 is preferred by us as no need to change the UE behavior, just to refine the definition of Tuncertainty_MAC, Tuncertainty_RRC and Tuncertainty_SP is enough. We provide one CR to capture this refining in [2].

- Proposal 1: Two alternatives to address this case:

- Alt 1: Reuse the requirement defined in 8.3.2, the uncertainty part Tuncertainty_MAC, Tuncertainty_RRC and Tuncertainty_SP are positive value under this case. UE would not report L3 reporting after receiving the TCI state indication.

- Alt 2: Follow the requirement defined in 8.3.17, add some clarification on the uncertainty part Tuncertainty_MAC, Tuncertainty_RRC and Tuncertainty_SP are zero under this case.

- Alt 2 is preferred by us due to less spec impact.

- Conclusion

In this contribution, we have the following proposals for the core part maintenance of SCell activation enhancements:

- Observation 1: Regarding the value of Tuncertainty_MAC, Tuncertainty_RRC and Tuncertainty_SP,

- If following the definition in 8.3.2, they should be positive value;

- If following the definition in 8.3.17, they should be negative value.

- Observation 2: If following the requirement of 8.3.2, the UE behavior and requirement is not consistent, the the delay from TCI state indication to L3 reporting actually exists, but no component captures this part in the requirement.

- If following the requirement of 8.3.17, the UE behavior and requirement is consistent.

- Proposal 1: Two alternatives to address this case:

- Alt 1: Reuse the requirement defined in 8.3.2, the uncertainty part Tuncertainty_MAC, Tuncertainty_RRC and Tuncertainty_SP are positive value under this case. UE would not report L3 reporting after receiving the TCI state indication.

- Alt 2: Follow the requirement defined in 8.3.17, add some clarification on the uncertainty part Tuncertainty_MAC, Tuncertainty_RRC and Tuncertainty_SP are zero under this case.

- Alt 2 is preferred by us due to less spec impact.

- Reference

- R4-2412599 “Discussion on maintenance of RRM requirements for R18 RRM_enh3”, vivo

- R4-2419279 “CR on core part of R18 SCell activation enhancement”, ZTE