3GPP TSG RAN#4 Meeting #113						R4-2419283

Orlando, 18-22 November 2024

Agenda Item:	11.x.x – Proposals led by RAN4 (spectrum related)

Source:	SES

Title:	Considerations on Satellite service allocated “Extended C-band” as NTN band

Document for:	Information

Contact:	joel.grotz@ses.com, abbas.karaki@ses.com

# Introduction

In this paper, the study of the satellite service allocated [3.3 – 4.2 GHz] frequency band is proposed in view of a potential definition as “NTN” band. This paper presents the motivation to consider this frequency band (also called “C band” in satellite eco-system).

# Discussion

Enabling the deployment/operation of NR-NTN based satellite access network in additional bands below 6GHz would help to address the ubiquitous connectivity ambition, as defined in [1].

We consider the quite general use cases of mobile handheld devices as well as moving platforms (e.g. vehicles, airplanes, boats) mounted devices, which may roam over large areas with possible TN coverage gaps and require however reliable and broadband connectivity. Services on such UEs are relying on increased coverage and bandwidth availability enabled by an NTN complement to the terrestrial network.

An NTN component could expand service availability to devices and vehicles in areas which cannot be covered or are difficult to cover with a terrestrial network. We note the following essential requirements for connectivity:

- Wide area coverage and availability

- High data rates to these devices

- Fast and lossless transitions to terrestrial network coverage connectivity

- Compact and cost-effective RF front end antenna for the moving platform mounted user equipment

NR-NTN access links hold the potential to address these coverage requirements (as per objectives in [1]), and the addition of NTN bandwidth in the FR1 range and specifically within the C-band, would help address these requirements.

For all handheld or moving platform mounted devices, it is proposed to define as NTN band, part of the satellite service allocated C-band which would overlap with the 3GPP defined TN band n77 [3.3 – 4.2 GHz].

In addition, for many applications, it is important that the RF front-end of the NTN capable UE can access a TN network as well as an NTN network to enable a seamless transition and leverage the NTN network global coverage capability.

To accommodate the increasing usage of mobile services and the coverage required, the usage of this C-band is proposed for mobile NTN services to respond to the required coverage and bandwidth need of future NTN services. Identifying additional spectrum for NR-NTN applications is important to ensure sufficient bandwidth for mobile broadband applications is available.

# Conclusions

Proposal 1: RAN to consider as part of the Rel-20 work plan the study of the [3.3 – 4.2 GHz] for NR-NTN based satellite networks deployment/operation in view of the definition of a related NTN band. This includes:

- Analysis of the applicable requirements in ITU Regions 1, 2 and 3

- Identification of the relevant co-existence scenarios between NTN and TN

- Undertake preliminary coexistence analysis between NTN and TN (“terrestrial” band n77) considering different duplex mode options for the NTN component, including TDD and FDD

# References

[1]	 “ITU-R Recommendation M.2160 - Framework and overall objectives of the future development of IMT for 2030 and beyond”