3GPP TSG-RAN WG4 Meeting #113	R4-2419330

Orlando, US, 18th – 22nd November 2024

Agenda Item:	7.16.2

Source:	Ericsson

Title:	On MMSE-IRC receiver for interference mitigation with 8Rx

Document for:	Discussion

# 1	Introduction

In RAN#104 meeting, the revised WID on NR demodulation performance: Phase 5 was approved [1]. According to the WID, following are the objectives for UE part:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline

Following the objectives of the WID, companies first discussed the test scope and made following agreements:

Test scope - RAN4 to introduce UE demodulation requirements for both TDD and FDD - Not to cover PDSCH absolute physical layer throughput requirements for 8Rx - Candidate options on whether to cover CQI requirements for ICI scenario: - Option 1: CQI reporting requirements under ICI should be cover for 8Rx - Option 2: No need to cover such requirements

By default, PDSCH requirements will be defined for both inter-cell interference and intra-cell inter-user interference, unless any technical issues are identified. To establish parameter assumptions for the test cases, it was encouraged to share initial evaluations during this meeting to assess the performance gain of the IRC receiver compared to the MMSE receiver. In this contribution, we present our perspectives along with evaluations to facilitate better determination of the parameter assumptions.

# 2	Inter-cell interference

In this section, we discussed the scenario of inter-cell interference cancellation with 8Rx reception.

## 2.1	PDSCH requirements with inter-cell interference

As a result of last RAN4 #112bis meeting’s discussion, following parameter assumptions are confirmed for the final requirements:

Interference profile and power levels Agreement: - Reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx. Interference signal modulation order Agreement: - Reuse the same interference signal modulation order as 2/4Rx requirements, i.e., random 16QAM symbols. Network type and SCS/CBW Agreement - FDD: 15kHz/10MHz - TDD: 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) Antenna and MIMO correlation Agreement - Rank 1: 2T8R ULA Low - Rank 2: 2T8R ULA Low - Rank 4: 4T8R ULA Low

Except the parameters above, there are still some options for the Rank, Propagation condition, MCS that need to be down selected based on the simulation results submitted by interested companies. In order to narrow down from multiple candidates, several evaluation cases have been agreed so that interested companies can provide their simulation results for studying the PDSCH performance.

According to the agreed WF [1], following parameters are with multiple candidates that need to be further discussed:

Propagation conditions Agreement: As a starting point, use: - TDLC300-100 for 2 interfering cells with rank 1 (HomNet) - TDLA30-10 for all other configurations. Rank of serving cell Agreement - Evaluate rank 1,2 & 4 in initial simulation analysis. - TBD: Down-selection for requirement definition. MCS of serving cell Agreement: Evaluate the following configurations for initial analysis: - Rank 1: MCS17 - Rank 2: MCS17 - Rank 4: MCS13 In case one of the above configurations is found not feasible consider alternative MCS. Interrested companies can provide simulations results with: - Rank 1: MCS19 - Rank 2: MCS13 / MCS19

Following are the expected simulation cases and results from our side.

Detailed assumptions and results can be found in our companion paper [2].

Table 2.1-1 PDSCH simulation results for 2 and 1 interference cell (baseline)

| Test num | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS (Table 1) | Propagation condition | Correlation matrix and antenna configuration | Reference value 70% max Tput |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 2 interference cells |  |  |  |  |  |  |  |  |
| 1 | 1 | 10 / 15 | MCS17 | TDLC300-100 | 2x8, ULA Low | 8.65 | 11.07 | 2.42 |
| 2 | 2 | 10 / 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 7.26 | 13.22 | 5.96 |
| 3 | 4 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 9.21 | 14.47 | 5.26 |
| 4 | 1 | 10 / 15 | MCS19 | TDLC300-100 | 2x8, ULA Low | 10.89 | 12.40 | 1.51 |
| 5 | 2 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 4.02 | 10.27 | 6.25 |
| 6 | 2 | 10 / 15 | MCS19 | TDLA30-10 | 4x8, ULA Low | 9.03 | 14.53 | 5.50 |
| 1 interference cell |  |  |  |  |  |  |  |  |
| 7 | 1 | 10 / 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 4.93 | 7.90 | 2.97 |
| 8 | 2 | 10 / 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 6.20 | 11.10 | 4.90 |
| 9 | 4 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 7.30 | 12.83 | 5.53 |
| 10 | 1 | 10 / 15 | MCS19 | TDLA30-10 | 2x8, ULA Low | 6.50 | 9.23 | 2.7 |
| 11 | 2 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 2.76 | 8.27 | 5.5 |
| 12 | 2 | 10 / 15 | MCS19 | TDLA30-10 | 4x8, ULA Low | 7.53 | 12.85 | 5.3 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Sub carrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |

Observing from the results of both 2 interference cells and 1 interference cell, MCS17 and MCS19 are both acceptable for the final requirements since the SNRs of 70% max Tput for IRC receiver are within a reasonable range and the gain over MMSE receiver is quite large. However, for MCS13, the SNR at 70% of maximum throughput is 4.02 dB, corresponding to an SINR of approximately -7 dB, which is too low to be considered feasible at the cell edge. In this case, we propose the following:

Observation 1: MCS17 and MCS19 are all feasible for rank 1 and rank 2 cases

Observation 2: MCS13 with rank 2 is not feasible to be selected since the corresponding SINR is -7dB

Proposal 1: Consider following parameter assumptions for the final requirements:

Propagation conditions

- TDLC300-100 for 2 interfering cells with rank 1 (HomNet)

- TDLA30-10 for all other configurations.

Rank of serving cell

- 1,2 & 4

MCS of serving cell

- Rank 1: MCS19

- Rank 2: MCS17

- Rank 4: MCS13

## 2.2	CQI reporting

It has been agreed to include CQI reporting requirements for inter-cell interference scenario. In Rel-17, RAN4 has defined CQI reporting requirements for ICI scenario assuming fading condition for the target cell and AWGN condition for the interference cell. We propose to reuse the previous propagation condition, specifically:

- TDLA30-5 for the serving cell

- AWGN for the interference cell.

Proposal 2: Reuse the same configuration for Rel-17 2/4Rx:

- TDLA30-5 for the serving cell

- AWGN for the interference cell

Regarding the interference model, we propose to reuse the assumptions for Rel-17 2/4Rx:

Proposal 3: Reuse the following assumptions for Rel-17 2/4Rx

- One interfering cell

- CSI-IM overlaps with PDSCH from interference

- NZP CSI-RS on target cell overlaps with NZP CSI-RS from interference

We suggest reusing the test metric as well:

- The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ γ;

- when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Proposal 4: Reuse the same test metric for Rel-17 2/4Rx

As for the antenna configuration and rank, we propose to study the performance with at least rank 2 and 4 to further confirm the feasibility.

Proposal 5: Study the performance for at least rank 2 and rank 4

# 3	Intra-cell inter-user interference

In this section, we discussed the scenario of intra-cell inter-user interference cancellation with 8Rx reception.

## 3.1	PDSCH requirements with intra-cell inter-user interference

As a result of last RAN4 #112bis meeting’s discussion, following parameter assumptions are confirmed for the final requirements:

Network Type and SCS/CBW Agreement: - FDD: 15kHz/10MHz (as priority for initial simulation analysis). - TDD: 30kHz/40MHz, 7D1S2U (S=6D+4G+4U) Interference modelling Agreement: - Beamforming model: MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4 - MO of paired UE: Random 16QAM Number of coUEs Agreement: - Consider only 1 coUE Propagation condition Agreement: - 1+1: TDLA30-10. Reconsider TDLC300-100 if TDLA30-10 found not feasible - 2+2: TDLA30-10 - 4+4: TDLA30-10 Precoding method for the target UE Agreement: - Random precoder DMRS ports Agreement: - Rank 1+1 and 2+2, reuse same configuration as Rel-17 - Rank 4+4, use different CDM group for 4+4 case. Serving UE with ports {1000, 1001,1004, 1005} and co-scheduled UE with ports {1002, 1003, 1006, 1007} as a starting point. Signal power assumptions Agreement: Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal SNR assumptions Agreement: SNR = (STargetUE+ SInterfUE)/N Codebook type Agreement: Type I Single Panel only - 4Tx: (N1, N2) = (2,1) - 8Tx: (N1, N2) = (4,1) PRB bundling size and precoding granularity Agreement: 2PRBs for PRB bundling size and precoding granularity PDCCH and PDSCH allocation in time domain Agreement: - Use symbols #0 and #1 of each slot for PDCCH - PDSCH mapping type A, Start symbol 2, Duration 12 PDSCH allocation in frequency domain Agreement: Full PRB

Except the parameters above, there are still some options for the Rank, precoding method for the co-scheduled UE, MCS that need to be down selected based on the simulation results submitted by interested companies. In order to narrow down from multiple candidates, several evaluation cases have been agreed so that interested companies can provide their simulation results for studying the PDSCH performance.

According to the agreed WF [1], following parameters are with multiple candidates that need to be further discussed:

Precoding method for the co-scheduled UE Agreement: For initial simulation alignment use: - Rank 1+1: random and orthogonal. - Rank 2+2: orthogonal. - Rank 4+4: orthogonal. For orthogonal precoding, select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH Antenna, rank (target UE + co-scheduled UEs) configuration and MIMO correlation Agreement: For initial simulation analysis: - Rank 1+1 for 2T8R ULA Low - Rank 2+2 for 4T8R ULA Low - Rank 4+4 for 8T8R ULA Low - Note: Will require DMRS len=2, hence optional feature, which means if introduced requirements would also be needed for one mandatory configuration (i.e. either 1+1 or 2+2) - Note: TBD: Down-selection of configurations. MCS Agreement: Evaluate: - Rank 1+1: MCS13 - Rank 2+2: MCS13 - Rank 4+4: MCS13 as starting point. Interrested companies can provide results for - Rank 1+1: MCS17 - Rank 2+2: MCS19

Following are the expected simulation cases and results from our side.

Detailed assumptions and results can be found in our companion paper [2].

Table 3.1-1 PDSCH simulation results

| Test num | Rank combination (target+ co-scheduled) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value (70% max Tput) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 1 | 1+1 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 2x8, ULA Low | 2.13 | 3.08 | 0.95 |
| 2 | 2+2 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 4x8, ULA Low | 5.64 | 13.03 | 7.39 |
| 3 | 4+4 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 8x8, ULA Low | 13.9 | N/A | INF |
| 4 | 1+1 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 2x8, ULA Low | 3.98 | 10.63 | 6.65 |
| 5 | 1+1 | 10 / 15 | MCS17 | Random 16QAM symbols | TDLA30-10 | 2x8, ULA Low | 5.31 | 8.64 | 3.33 |
| 6 | 1+1 | 10 / 15 | MCS17 | Random 16QAM symbols | TDLA30-10 | 2x8, ULA Low | 7.28 | 26.31 | 19.03 |
| 7 | 2+2 | 10 / 15 | MCS19 | Random 16QAM symbols | TDLA30-10 | 4x8, ULA Low | 10.73 | N/A | INF |

Test number 4 and 6 were configured with random precoding for the target UE. It can be observed that for rank 1+1 case, the gain from the performance with orthogonal precoding is quite limited.

Observation 1: For the rank 1+1 case, the gain from the performance with orthogonal precoding is quite limited

For rank 1+1 case, the gain from the performance with random precoding is significantly large no matter for MCS13 or MCS17.

Observation 2: For the rank 1+1 case, the gain from the performance with random precoding is significantly large no matter for MCS13 or MCS17

In the meantime, MCS17 should be considered instead for the rank 1+1 case if RAN4 agreed on orthogonal precoding for such case.

Proposal 6: Consider following combinations

- MCS17 for rank 1+1

- MCS13 for rank 2+2

- MCS13 for rank 4+4

More evaluation results can be found in our companion simulation result paper [2].

# 4	Summary

In this contribution, we shared our views and results on the  introduce PDSCH and CQI reporting requirement. Our proposals are summarized as follows:

Observation 1: MCS17 and MCS19 are all feasible for rank 1 and rank 2 cases

Observation 2: MCS13 with rank 2 is not feasible to be selected since the corresponding SINR is -7dB

Proposal 1: Consider following parameter assumptions for the final requirements:

Propagation conditions

- TDLC300-100 for 2 interfering cells with rank 1 (HomNet)

- TDLA30-10 for all other configurations.

Rank of serving cell

- 1,2 & 4

MCS of serving cell

- Rank 1: MCS19

- Rank 2: MCS17

- Rank 4: MCS13

Proposal 2: Reuse the same configuration for Rel-17 2/4Rx:

- TDLA30-5 for the serving cell

- AWGN for the interference cell

Proposal 3: Reuse the following assumptions for Rel-17 2/4Rx

- One interfering cell

- CSI-IM overlaps with PDSCH from interference

- NZP CSI-RS on target cell overlaps with NZP CSI-RS from interference

Proposal 4: Reuse the same test metric for Rel-17 2/4Rx

Proposal 5: Study the performance for at least rank 2 and rank 4

Observation 3: For the rank 1+1 case, the gain from the performance with orthogonal precoding is quite limited

Observation 4: For the rank 1+1 case, the gain from the performance with random precoding is significantly large no matter for MCS13 or MCS17

Proposal 6: Consider following combinations

- MCS17 for rank 1+1

- MCS13 for rank 2+2

- MCS13 for rank 4+4

# References

[1]	RP-2416555: WF for [112bis][320] NR_demod_Ph5_Part2_UE, Nokia, RAN4 #112bis, Hefei, China

[2]	R4-2419331: Simulation results for MMSE-IRC receiver for 8Rx reception, RAN4 #113, Orlando, US