3GPP TSG-RAN WG4 Meeting #113							R4-2419370

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.5.2.3

Source: 	OPPO

Title: 	On intra-band non-collocated other aspects

Document for:	Approval

- Introduction

In RAN4#112 meeting, the reference UE architecture has been discussed based on the LTE intra-band non-contiguous CA assumptions. Based on the understanding, the type 1 UE architecture is proposed to be modified by some of the companies. In this paper, we give our understanding and try to figure out the solution for the UE architecture table.

Discussion

The reference UE architecture topic has been discussed during the RAN4#112-bis meeting and the WF agreement is captured as below.

Way Forward:

- Continue further discussion on the following aspects for reference UE architecture in the next meeting.

- Comments of proposed reference UE architecture table

- Partially/fully shared

- Differentiate Type 1a and 1b

- If identified, other additional issues

During the RAN4#112-bis meeting, the reference receiver architecture that is used for LTE intra-band non-contiguous CA has been quoted as below from TR 36.823:

Figure 1 Reference receiver architecture for NC-intra-band CA for LTE

Indeed, this reference architecture has been agreed also as the baseline assumption of NR intra-band non-contiguous CA. In such case, from the figure 1 we can observe, one shared LNA has been used for the two RX chains for the two non-contiguous carriers. Furthermore, the diversity receiver duplicates the main receiver components to provide 2 layers for each carrier.

Observation 1: For LTE intra-band NC CA, one shared LNA has been used for the two RX chains for the two non-contiguous carriers.

Observation 2: For NR intra-band NC CA, similar architecture of LTE has been agreed as baseline architecture.

Figure 1 LTE intra-band NC CA baseline RF architecture

The LTE baseline RF architecture for intra-band NC CA only considers 2 layer for each CC. While for NR intra-band NC CA, 4 layer for each CC is mandatory below figure 2 shows the baseline RF architecture.

Figure 2 NR intra-band NC CA baseline RF architecture

In this case, the LNA is shared while the rest of the RX chain starting from mixer to digital baseband are all separate. This is quite similar to the type 3b UE architecture as listed in the table 1 below for UE architecutre.

Observation 3: based on the LTE UE intra-band NC CA architecture baseline, the baseline for NR intra-band NC CA is shown as figure 2 and marked as type 3b.

The above observation 1 and 2 are currently used in the fragmented carriers WID and specially to define the frequency gap between two non-contiguous carriers. However, the intra-band non-collocated WID are defining the requirements in different perspective as the requirement for power imbalance of the two non-collocated carriers.

Observation 4: Different to the fragmented carriers WID, the intra-band non-collocated WID is defining requirement for the power imbalance of the two non-collocated carriers.

Looking back to the whole non-collocated WID progress, the UE type table is listed below for information. The 25dB power imbalance is based on the totally separated RX chain architecture for type 2 UE as well as type 4a/4b UE. This has nothing to do with the type 1 UE. The type 1 UE with 6dB power imbalance capability is just to have a basic understanding of the legacy UE.

Table 1 UE architecture

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

Observation 4: The 25dB power imbalance is based on the totally separated RX chain architecture for type 2 and type 4a/4b UE.

Currently, the question is the type 1 UE with the comment column set as baseline architecture which is not aligned with the baseline assumption for intra-band NC CA. Type 1 UE is one of the possible UE architecture but for sure it is not the baseline as discussed above with the observation 3. But we don’t need to further modify the type 1 UE which will make the table more complex and make no sense to better understanding the power imbalance requirement for non-collocated deployment.

Furthermore, the baseline architecture for intra-band NC CA is not needed for the intra-band non-collocated deployment requirements considering observation 1,2 and 4. Currently the UE architecture for type 2 and type 4a/4b is totally correct and does not depend on the type 1 UE architecture.

Hence. from our understanding the comment column of the table is not needed. Removing the whole column can also remove the ambiguity. We don’t need to further modify the UE architecture table and the type 3a and type 3b for handheld UE can be further discussed in the future.

Proposal 1: To delete the “comment” column to update the UE architecture table.

Table 2 updated UE architecture description

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range |
|  | 2 |  |  |  |  | 4Rx |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |

Another topic is the UE type definition for NR-CA and EN-DC. The agreement in WF [1] is captured as below:

<Issue 3-2: UE Type definition for NR-CA>

Way Forward:

- The following table is a starting point, and continue further discussion the following table in the next meeting.

- The table will be added into TS 38.101-1: 7.10A	Power imbalance for CA.

- Table 2: NR-CA UE Capability types

| [UE capability types] | Type description | dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and MRTD requirements according to clause 7.6.4-1 in 38.133 | N/A |
| Type 2 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | intraBandNR-CA-non-collocated-r18 |
| Type 4 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | TBD |

For simplicity, we start from NR-CA table as captured above table 2.

As this table will be captured in TS 38.101-1 sub-clause 7.10A Power imbalance for CA, this table will only apply to the power imbalance requirement. For type 1 UE, as discussed offline and also through the email discussion, the 6dB PSD imbalance should be a minimum capability and baseline assumption. However, this 6dB PSD power imbalance is not captured as a UE RF requirement, so this baseline assumption might not be appropriate to be captured in the TS 38.101-1. Indeed, a legacy UE that has fulfil the TS 38.101-1 already defined requirement should naturally fulfil this 6dB PSD imbalance. Hence we propose to directly deleted the wording in the “()” for type 1. And for sure this type 1 legacy UE should have no UE capability related to the intra-band non-collocated deployment.

Observation 5: A type 1 UE fulfil current TS 38.101-1 requirement should naturally fulfil the 6dB PSD imbalance baseline.

Proposal 2: For type 1 UE type description, delete the description of 6dB PSD imbalance.

3	Conclusions

In this paper, we give further discussion on the UE reference architecture, the observations and proposals are as below:

Observation 1: For LTE intra-band NC CA, one shared LNA has been used for the two RX chains for the two non-contiguous carriers.

Observation 2: For NR intra-band NC CA, similar architecture of LTE has been agreed as baseline architecture.

Observation 3: based on the LTE UE intra-band NC CA architecture baseline, the baseline for NR intra-band NC CA is shown as figure 2 and marked as type 3b.

Observation 4: The 25dB power imbalance is based on the totally separated RX chain architecture for type 2 and type 4a/4b UE.

Observation 5: A type 1 UE fulfil current TS 38.101-1 requirement should naturally fulfil the 6dB PSD imbalance baseline.

Proposal 1: To delete the “comment” column to update the UE architecture table.

Proposal 2: For type 1 UE type description, delete the description of 6dB PSD imbalance.

4 References

[1] R4-2417115 	WF on rest issues for intra-band non-collocated EN-DC/NR-CA, KDDI