3GPP TSG-RAN WG4 Meeting #113							R4-2419372

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.3.1

Source: 	OPPO

Title: 	On intra-band CCA and NCCA A-MPR

Document for:	Approval

- Introduction

During the previous meeting, the sidelink intra-band CCA and NCCA MPR has been fully discussed. Also, some of the discussion papers have already give some analysis on the A-MPR requirement such as [1] and [2]. Hence, we would like to further discuss the A-MPR requirement and to prepare for the A-MPR simulation.

- Discussion

In current TS 38.101-1, NS_33 has been used for EU regulation for ITS band n47. Current requirement is for single carrier with only 10MHz channel bandwidth. While furthermore, currently only 10MHz for one channel is applicable in the EU regulation and the detail spectrum is shown as below.

So currently, the road ITS for EU regulation is the 5875 to 5915 with 40MHz in total. For easier discussion, we mark the 40MHz with 4 channels as channel 1 to 4.

For intra-band NCCA, there are three cases for bandwidth configuration, only 10MHz + 10MHz can be supported based on the current EU regulation. In such case, Channel 1+ Channel 3; Channel 1+ Channel 4 and Channel 2 + Channel 4 are the 3 cases that needs to be considered.

The NS_33 consist of additional SEM and also the additional spurious emission requirements. The requirement is captured as below from TS 38.101-1. We see the ASEM as captured below is for 10MHz each carrier. In such case, for intra-band NCCA, the combined SEM is used for the two carriers, which means for the overlapping part of the two SEM of two carriers, the higher limit applies. We believe that the same principle can be used for the A-SEM for NS_33.

Proposal 1: For intra-band NCCA, the A-SEM of two non-contiguous 10MHz carriers, the overlapping part of the A-SEM for each carrier will use the higher limit as the combined A-SEM.

For the A-SE, this is an absolute value and hence we can directly apply the A-SE. In [2], it has been discussed and consider that if the IM3 of the two non-contiguous 10MHz carriers fall into the A-SE region to meet the very stringent EIRP limit as -30dBm/MHz. Indeed currently the MPR simulation, most of the IM3 falls into the -30dBm/MHz region and hence we can further check after A-MPR simulation is done.

Proposal 2: To use the A-SE limit to derive the derive the A-MPR requirement.

For intra-band CCA, in our opinion, the Channel 1+ Channel 2; Channel 2+ Channel 3 and Channel 3+ Channel 4 is applicable bandwidth configurations. For A-SEM of the CCA, it is proposed that only consider the A-SEM of one side of the 10MHz carrier.

Proposal 3: For intra-band CCA A-SEM, only consider the A-SEM of one side of the 10MHz carrier.

3	Conclusions

In this paper, we give further discussion on intra-band NR SL CCA and NCCA A-MPR simulation requirements, the observations and proposals are as below:

Proposal 1: For intra-band NCCA, the A-SEM of two non-contiguous 10MHz carriers, the overlapping part of the A-SEM for each carrier will use the higher limit as the combined A-SEM.

Proposal 2: To use the A-SE limit to derive the derive the A-MPR requirement.

Proposal 3: For intra-band CCA A-SEM, only consider the A-SEM of one side of the 10MHz carrier.

4 References

[1] R4-2408048 UE RF requirements for NR SL Intra-band contiguous CA, Meta Ireland

[2] R4-2412014 NR side link requirements and European regulation for band n47, LG Electronics