3GPP TSG-RAN WG4 Meeting #113                                           R4-2419377

Orlando, US, 18th -22th Nov, 2024

Source: 	Huawei, HiSilicon

Title: 	A-MPR re-evaluation on n1 for updated emission requirements

Agenda Item:	4.2.1

Document for:	Approval

Introduction

Recently, Japan's regulations on spurious emissions related to PHS have been updated, as background information shows in [1]. This contribution provides a preliminary evaluation of the A-MPR requirements in light of the updated emissions to protect DECT and the cessation of PHS operations.

Discussion

According to [1], the changed additional emission requirements for "NS_05" and “NS_05U” are:

Table 6.5.3.3.4-1: Additional requirements for "NS_05" and “NS_05U”

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |  |
| --- | --- | --- | --- |
|  | 5, 10, 15, 20 |  |  |
| 1884.5 f 1915.7 | -41 | 300 kHz |  |

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 |  |
| 1884.5 ≤ f ≤ 1910 | -30 | 1 MHz |
| 1910 ≤ f ≤ 1915.7 | -25 |  |
| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
|  | 10, 15, 20 |  |
| 1884.5 ≤ f ≤ 1906.6 | -30 | 1 MHz |
| 1906.6 ≤ f ≤ 1915.7 | -25 |  |

The evaluation cases based on operators’ spectrum allocations are outlined as below.

Figure 1: Evaluation cases for band n1 – in 1920-1940MHz [1]

The existing A-MPR requirement for NS_05 and NS_05U (PC3) are reproduced as below:

Table 6.2.3.4-2: A-MPR for NS_05 and NS_05U (Power Class 3)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer |  |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 10 | ≤ 5 | ≤ 4 |  |
|  | QPSK | ≤ 10 | ≤ 5 | ≤ 4.5 |  |
|  | 16 QAM | ≤ 10 | ≤ 5 | ≤ 6 |  |
|  | 64 QAM | ≤ 11 | ≤ 5 | ≤ 6 |  |
|  | 256 QAM | ≤ 13 | ≤ 5 | ≤ 7 |  |
| CP-OFDM | QPSK | ≤ 10 | ≤ 5 | ≤ 7.5 |  |
|  | 16 QAM | ≤ 10 | ≤ 5 | ≤ 7.5 |  |
|  | 64 QAM | ≤ 11 | ≤ 5 | ≤ 8 |  |
|  | 256 QAM | ≤ 13 |  | ≤ 10 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |

Table 6.2.3.4-3: A-MPR for NS_05 (Power Class 3)

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer |  | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 | N/A | ≤ 1 |  | ≤ 1 | N/A | ≤ 6 |
|  | QPSK |  |  | ≤ 1.5 |  | ≤ 1.5 |  | ≤ 6 |
|  | 16 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 64 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 6 |
| CP-OFDM | QPSK | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 6 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 6 |
|  | 64 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 6 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |

A-MPR values in RB allocation region A1 are noticeably higher compared to other regions. Previous studies suggest that these high values in A1 may occur in both outer and inner regions. The A-MPR values for NS_05 and NS_05U are specified for the regions outlined in the table below."

Figure 1 shows the outer and inner RB allocation regions for different CBWs, defined by RB_start and L_CRB values

Figure 1: A-MPR regions for different CBWs for NS_05 and NS_05U

Simulations are performed for case 1 (20MHz) and case 6 (10MHz) in A-MPR region A1 for both DFT-s-OFDM and CP-OFDM waveform.

Table 2: A-MPR evaluation for NS_05 case 1 and case 6 (PC3)

| Modulation/Waveform |  | A1 (dB) | A-MPR evaluation |  |
| --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Case1: 20MHz | Case 6: 10MHz |
| DFT-s-OFDM | QPSK | ≤ 10 | 3.0 | 2.3 |
| CP-OFDM | QPSK | ≤ 10 | 4.8 | 4.1 |

Due to time limitations, the simulation was conducted using only a small subset of RB allocation points.

Observation 1: Preliminary evaluation shows that the high A-MPR values in region A1 could potentially be reduced by approximately 5dB and 7dB for CP-OFDM and DFT-s-OFDM respectively. More study is needed.

Further evaluation is planned.

Proposal 1: The simulation assumptions should be approved firstly, if possible, simplify the simulation cases.

Proposal 2: The simulation campaign should be focused on PC3, if the evaluation for PC2 cannot be completed by the Feb meeting, a simple scaling approach could be adopted to estimate PC2 A-MPR values for NS_05 and NS_05U.

Conclusion

This contribution presents the preliminary evaluation of A-MPR values with changed emission requirements for NS_05 and NS_05U.

Observation 1: Preliminary evaluation shows that the high A-MPR values in region A1 could potentially be reduced by approximately 5dB and 7dB for CP-OFDM and DFT-s-OFDM respectively. More study is needed.

Proposal 1: The simulation assumptions should be approved firstly, if possible, simplify the simulation cases.

Proposal 2: The simulation campaign should be focused on PC3, if the evaluation for PC2 cannot be completed by the Feb meeting, a simple scaling approach could be adopted to estimate PC2 A-MPR values for NS_05 and NS_05U.

- References

[1] R4-2418439, NR A-MPR reevaluation scenarios on n1 for Japan, KDDI, NTT DOCOMO

- Annex: Simulation assumptions

- CIM3: 60dBc.

- CIM5: 70dBc

- Focus evaluations first on QPSK DFT-s-OFDM with Image and carrier leakage of 28dB for up to 64QAM.

- Evaluation of Higher order modulations is not precluded, for 256QAM image is 35dB,

- PA calibration for 20 MHz, SCS 15 kHz, QPSK, DFT-S-OFDM, 100 RB0 waveform to meet ACLR (PC3: 30dB, PC2:31dB) at 1 dB MPR based on [4~5]dB post PA losses,

- Post PA insertion loss: [4~5] dB

- PC2 studies assume the same filter rejection than those used for PC3 studies

- Both 1Tx and 2Tx can be considered for PC2 evaluation