3GPP TSG-RAN WG4 Meeting #113									                        R4-2419380

Orlando, US, November 18 – 22, 2024

Agenda item:			7.1.1.3.4

Source:	Ericsson

Title:	On the SRS IL imbalance reporting details

Document for:	Approval

# Introduction

In RAN#104, a revised WID has been approved [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” WI. One of the three main parts of the WI is to specify 6Rx UE RF requirements for both handheld and FWA UEs, and in this contribution we only focus on the study of the issue of insertion loss imbalance across SRS ports:

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - …

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

Due to divergent views from different companies, this topic still hasn’t made a big progress so far. However, in the last meeting, in the approved WF [2] a candidate solution which captures different proposals and observations and combines them into one has been approved for further analysis.

In this contribution, we will not repeat the detailed analysis on the general considerations for the SRS IL imbalance issue nor the justification for the introduction of reporting. Instead, we will only repeat the main observations and refer to the corresponding contributions where more details can be found.

Regarding the existing UE behavior, according to the current specifications, the UE shall compensate for the IL below the maximum configured output power for a resource, as additional Tx insertion loss is not accounted for in the SRS power control equations (see [3] for more details).

Regarding the impact of SRS IL imbalance on NW performance degradation, based on system-level simulation results, we have demonstrated that the SRS IL imbalance without reporting/compensation results in a non-negligible DL MIMO performance loss (see [3] for more details).

In our contribution for the previous meeting [4], we have provided lab testing results of a commercial UE to verify the reciprocity of the Tx and Rx imbalance. We observed that the Tx imbalance is much larger than and non-reciprocal to the Rx imbalance.

We have also observed that contrary to TS38.213 and TS38.215, the total power at the antenna connectors is not split equally for multi-port SRS resources (e.g. 2T4R), and there is no specified test which verifies the expected behavior. Hence, in our view, it would be very beneficial to introduce testing of the SRS output power at least at the maximum power for multi-port SRS resources. In our contribution [5] (as a part of TEI-17), we have proposed new RMCs that should be introduced to TS38.101-1, where the rest could be handled by RAN5.

# Discussion

As stated in the introduction, in the approved WF [2] the following candidate solution has been captured, where the details are FFS:

- Candidate solutions - Option 1: - UE performs self-compensation of SRS IL up to its maximum Tx power capabilities - UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - Details of SRS IL self-compensation and assistance framework are FFS - Other options are not precluded

According to the WF, companies are “encouraged to provide expected benefit/performance gains and draftCRs so that the impacts to RAN1, RAN2, and RAN4 specifications (for RAN1 and RAN2, a description of the necessary changes is sufficient if draftCRs are not possible) can be further discussed”.

In the following, we will propose the details of the SRS IL self-compensation and assistance framework, and then we will discuss the impact on the different specifications (on high-level).

First, we make the following proposals:

Proposal 1: Introduce reporting for the SRS insertion loss imbalance issue in Rel-19.

Proposal 2: The introduced solution for the SRS insertion loss imbalance issue should be applicable to all 2Rx/4Rx/6Rx/8Rx cases.

## Details of the proposed candidate solution

### Details of the SRS IL self-compensation part of solution

As discussed in the introduction, the UE shall perform self-compensation for the IL below the maximum configured output power for a resource, as additional Tx insertion loss is not accounted for in the SRS power control equations. Since PCMAX takes into account the insertion loss by allowing the maximum relaxation ΔTRxSRS in its lower bound and is defined for each SRS resource separately, each SRS transmission power below PCMAX shall be feasible. Thus, the UE should perform the self-compensation of the SRS IL up to the maximum configured output power (rather than up to its maximum Tx power capability).

Observation 1: The proposed solution shall mandate the UE to perform the self-compensation of the SRS IL up to its configured maximum output power, which is consistent with the current specifications.

### Details of the assistance framework part of solution

Since the UE cannot compensate for the IL at the maximum configured output power, it should provide assistance to the network on the SRS IL compensation.

One candidate reporting parameter is the “SRS insertion loss value (per SRS resource or per UE)”. In our understanding, this is related to the actual insertion loss of different branches/SRS ports. In general, this could be a promising solution as long as the imbalance between the actual SRS transmission powers is equal to the actual insertion loss differences, which should be the case if the UE transmits at the maximum configured output power PCMAX at each SRS port.

However, as it can be concluded from the online discussions, the UE behavior in the field on the IL compensation may be unpredictable. If the network takes into account the reported actual “SRS insertion loss value” and the UE compensates for the insertion loss, that may introduce even more inaccuracy for the DL CSI estimation.

In addition, there is no guarantee that the UE will set the maximum configured output power according to the actual insertion loss (for example, the chosen PCMAX could be “overoptimistic”), and in that case only a partial compensation of the IL may occur.

Observation 2: Reporting of the actual insertion loss (or the actual IL imbalance), i.e. “SRS insertion loss value reporting”, is in general insufficient for the accurate DL CSI estimation.

Our proposed solution for the “UE assistance to the network” part is that reporting should include the configured maximum output power per SRS resource and the power headroom (PH) per SRS resource as well as the information on the power class fallback or the return to the supported power class (ΔPPowerClass). In that way the network can have a relatively precise information on each SRS output power at the connector. Note that reporting of all three of these parameters is already part of “Single Entry PHR MAC CE” and “Multiple Entry PHR MAC CE”.

The configured maximum output power includes the actual insertion loss (Tx/Rx) imbalance (for resources for which this is allowed) on top of the MPR used for the SRS resource. If the MPR is the same between the different ports (which is expected), by inspecting the differences between the configured maximum output power per SRS resource the information on the IL imbalance can be obtained. On the other hand, by transmitting in addition the power headroom (PH) report per SRS resource, the network can be aware of whether a UE operates at its maximum power and it can obtain the information on the actual output power per SRS resource by subtracting the power headroom from the configured maximum output power (in that way it can observe whether the IL compensation has been performed). So, both cases when UEs perform or do not perform a compensation for the ILs are covered by the proposed solution.

One of the motivations to include ΔPPowerClass in the reporting is the discussion under the Rel-18 maintenance agenda for “Further RF requirements enhancement for NR frequency range 1 (FR1)” WI, where the ΔPPowerClass requirements for AS for 4Tx are currently still incomplete. To see a detailed argumentation from our side, one could check our contribution [6]. Here, we will only reiterate that the solution similar to the one specified for 2Tx would be sub-optimal (4Tx adds a lot of complexity compared with 2Tx) and that the issue could be optimally solved only by introducing the reporting of ΔPPowerClass.

The expected benefit from such solution is the maximization of the accuracy of the DL CSI estimation, which is of key importance for the performance of the DL MIMO, both Single User and Multi-User.

Proposal 3: The following SRS IL imbalance reporting solution should be specified:

- UE performs self-compensation of SRS IL up to its configured maximum output power.

- UE provides dynamic assistance to the network on SRS IL compensation:

  - Per SRS resource power headroom reporting

  - Configured maximum output power per SRS resource reporting

  - ΔPPowerClass reporting

Observation 3: The expected benefit from the proposed solution is the maximization of the accuracy of the DL CSI estimation, which is of key importance for the performance of both SU and MU MIMO.

### Detailed impact on the specifications

#### RAN4 specifications: TS38.101-1

With the proposed solution (Proposal 3) there would be no/very limited impact on TS38.101-1 specification.

#### RAN1 specifications: TS38.213

The Type 3 power headroom report is defined in clause 7.7.3 of TS38.213[7].

The only required change would be to allow Type 3 power headroom report for the UE configured for PUSCH transmissions on carrier f of serving cell c (for both an actual and a reference SRS transmission). In the following, we highlight the exact places in the specifications where the applicability of Type 3 power headroom is limited only to “PUSCH-less” carriers, e.g. SDL. This restriction should be lifted at least for a new MAC-CE, named for example “SRS resource PHR” (see clause 2.1.3.3), which should be straightforward to incorporate into the text.

If a UE determines that a Type 3 power headroom report for an activated serving cell is based on an actual SRS transmission then, for SRS transmission occasion on active UL BWP of carrier of serving cell c and if the UE is not configured for PUSCH transmissions on carrier of serving cell c and the resource for the SRS transmission is provided by SRS-Resource, the UE computes a Type 3 power headroom report as [dB] where , , , , and are defined in clause 7.3.1 with corresponding values provided by SRS-ResourceSet. If the UE determines that a Type 3 power headroom report for an activated serving cell is based on a reference SRS transmission then, for SRS transmission occasion on UL BWP of carrier of serving cell c, and if the UE is not configured for PUSCH transmissions on UL BWP of carrier of serving cell c and a resource for the reference SRS transmission is provided by SRS-Resource, the UE computes a Type 3 power headroom report as [dB] where is an SRS resource set corresponding to SRS-ResourceSetId = 0 for UL BWP and , , and are defined in clause 7.3.1 with corresponding values obtained from SRS-ResourceSetId = 0 for UL BWP . is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB and TC =0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2], [8-3, TS 38.101-3] and [8-5, TS 38.101-5].

One possible way to do it is the following:

If a UE determines that a Type 3 power headroom report for an activated serving cell is based on an actual SRS transmission then, for SRS transmission occasion on active UL BWP of carrier f of serving cell c and if the UE is not configured for PUSCH transmissions on carrier f of serving cell c and the resource for the SRS transmission is provided by SRS-Resource [or if the UE is configured for PUSCH transmissions on carrier c of serving cell f and the resource for the SRS transmission is provided by SRS-Resource with usage antennaSwitching], the UE computes a Type 3 power headroom report as

#### RAN2 specifications: TS38.321

Power Headroom (PH) framework specified in TS38.321 [8] could be used for the proposed reporting solution. PH used for the SRS resource can be a Type 3 but used for a new MAC-CE, for example called “SRS resource PHR”. A similar approach as for the “Multiple Entry PHR MAC CE” could be used (clause 6.1.3.9 in [8]), where multiple PH (power headroom), PCMAX (maximum configured output power) and DPC (ΔPPowerClass) fields (each set corresponding to a different SRS resource) could be transmitted in a single MAC CE. The maximum number of entries could be eight, which corresponds to 8 Rx case and 1T8R AS, where an SRS Resource Set consists of eight single port SRS resources. A new MAC-CE specified for SRS power reporting should be dynamic since SRS AS power levels can change.

The “SRS resource PHR MAC CE” could be defined similar to following:

-	Ci: This field indicates the presence of a PH field for the SRS resource with index i. The Ci field set to 1 indicates that a PH field for the SRS resource with index i is reported. The Ci field set to 0 indicates that a PH field the SRS resource with index i is not reported;

-	V: This field indicates if the PH value is based on a real transmission or a reference format. The V field set to 0 indicates real transmission on SRS and the V field set to 1 indicates that an SRS reference format is used. Furthermore, the V field set to 0 indicates the presence of the octet containing the associated PCMAX,f,c field and the DPC field, and the V field set to 1 indicates that the octet containing the associated PCMAX,f,c field and the DPC field is omitted;

-	Power Headroom (PH): This field indicates the power headroom level for the SRS resource i. The length of the field is 6 bits. The reported PH and the corresponding power headroom levels are shown in TBD;

-	P: This field indicates whether power backoff is applied due to power management (as allowed by P-MPRc as specified in TS 38.101-1, TS 38.101-2, and TS 38.101-3). The MAC entity shall set the P field to 1 if the corresponding PCMAX,f,c field would have had a different value if no power backoff due to power management had been applied;

-	PCMAX,f,c i: If present, this field indicates the PCMAX,f,c (as specified in TS 38.213) for the SRS resource i used for calculation of the preceding PH field. The reported PCMAX,f,c and the corresponding nominal UE transmit power levels are shown in TBD (the corresponding measured values in dBm for the NR Serving Cell are specified in TS 38.133);

-	DPC: If dpc-Reporting-FR1 is configured, and the Serving Cell operates on FR1, this field indicates the ΔPPowerClass for SRS resource i, as specified in TS 38.101-1[14] and TS 38.101-3 [16]. This field indicates an index to Table 6.1.3.8-4 and the corresponding measured values of DPC levels in dB are specified in TS 38.133 [11], the DPC field is set to 0 if the criteria to report DPC is not met. The length of the field is 2 bits.

| C7 | C6 | C5 | C4 | C3 | C2 | C1 | C0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P | V | PH (Type 3, SRS Resource 0) |  |  |  |  |  |
| DPC0 |  | PCMAX,f,c 0 |  |  |  |  |  |
| P | V | PH (Type 3, SRS Resource 1) |  |  |  |  |  |
| DPC1 |  | PCMAX,f,c 1 |  |  |  |  |  |
| P | V | PH (Type 3, SRS Resource 2) |  |  |  |  |  |
| DPC2 |  | PCMAX,f,c 2 |  |  |  |  |  |

…

| P | V | PH (Type 3, SRS Resource 7) |
| --- | --- | --- |
| DPC7 |  | PCMAX,f,c 7 |

Figure 1: Possible configuration of “SRS resource PHR MAC CE” with the highest number of SRS resources equal to 8 (corresponding to 1T8R AS)

The exact details of the PHR triggering conditions would also have to be discussed.

# Conclusion

In the previous sections, we made the following observations:

Observation 1: The proposed solution shall mandate the UE to perform the self-compensation of the SRS IL up to its configured maximum output power, which is consistent with the current specifications.

Observation 2: Reporting of the actual insertion loss (or the actual IL imbalance), i.e. “SRS insertion loss value reporting”, is in general insufficient for the accurate DL CSI estimation.

Observation 3: The expected benefit from the proposed solution is the maximization of the accuracy of the DL CSI estimation, which is of key importance for the performance of both SU and MU MIMO.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Introduce reporting for the SRS insertion loss imbalance issue in Rel-19.

Proposal 2: The introduced solution for the SRS insertion loss imbalance issue should be applicable to all 2Rx/4Rx/6Rx/8Rx cases.

Proposal 3: The following SRS IL imbalance reporting solution should be specified:

- UE performs self-compensation of SRS IL up to its configured maximum output power.

- UE provides dynamic assistance to the network on SRS IL compensation:

  - Per SRS resource power headroom reporting

  - Configured maximum output power per SRS resource reporting

  - ΔPPowerClass reporting

# References

- RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN#104.

- R4-2417209, “WF on 6Rx UE requirements”, AT&T, RAN4#112-bis.

- R4-2413360, “On need for SRS insertion loss imbalance reporting”, Ericsson, RAN4#112.

- R4-2415313, “On the SRS insertion loss imbalance reporting”, Ericsson, RAN4#112-bis.

- R4-2418870, “Reference measurement channels for SRS transmissions”, Ericsson, RAN#113.

- R4-2419499, “(NR_ENDC_RF_FR1_enh2-Core) On DeltaP_PowerClass for SRS AS for 4Tx”, Ericsson, RAN4#113.

- 3GPP TS38.213 V18.4.0, “NR; Physical layer procedures for control”.

- 3GPP TS38.321 V18.3.0, “NR; Medium Access Control (MAC) protocol specification”.