3GPP TSG-RAN4 Meeting #113	R4-2419390

# Orlando, United States, 18th - 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.871 | CR | 0006 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On Absolute Probe Locations for Baseline Multi-RX System |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_FR2_OTA_enh |  |  |  |  |  | Date: |  |  | 2024-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Latest agreements/endorsements of absolute probe locations for Multi-RX system has not been incorporated yet |  |  |  |  |  |  |  |  |
| Summary of change: |  | Incorporate the latest agreements/endorsement of absolute probe locations from R4-2309469 in the TR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR is not clear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

## 5.1	General

This clause describes the UE RF testing methodology for multi-Rx chain DL reception for FR2. The following candidate measurement setups are investigated:

-	Measurement Setup with Full Degree of Rotation Freedom for Each AoA

-	Measurement Setup with Full Degrees of Freedom for AoA1 with Fixed Angular Offset(s) Between AoA1 and 		AoA2

-	Measurement Setup with Full Degrees of Freedom for AoA1 with Variable Angular Offset(s) between AoA1 and 		AoA2

-	Measurement Setup with Full Degrees of Freedom for AoA1 with Partial Freedom of Variable Angular Offset(s) 		between AoA1 and AoA2

-	Measurement Setup with Full Degrees of Freedom for AoA1 with Fixed AoA2/Anchor in NF

-	Measurement Setup with Test Modes

-	Measurement Setup to reduce sensitivity to UE orientation in holder with full degrees of freedom for AoA1 with 		fixed angular offset(s) between AoA1 and AoA2

Through the analysis of system complexity, chamber footprint, upgradeability of existing system, development lead time, measurement uncertainty, and test time aspects, the measurement setup with full degrees of freedom for AoA1 with fixed angular Offset(s) between AoA1 and AoA2 and with all probes located on the xz plane is selected as the baseline in Rel-18.

<<< Skip Unchanged Sections >>>

## 5.3	Test System Aspects

### 5.3.1	Absolute Probe Locations

A sample system with three different probe locations as shown in Figure 5.3.1-1 is investigated to provide detailed justification to define absolute probe locations/directions for Multi-AoA UE RF testing. The sample implementation has a 2-axis positioning system for the DUT to implement a 3D scan and three probes in the x, y, and z axes with the assumption that AoA1 is along z and AoA2 along x or y. The DL directions perceived by the DUT from each probe are illustrated with red, green, and red spheres on the DUT grid in the x, y, and z directions, respectively, for this initial test point of (q, f) = (0°, 0°).

Figure 5.3.1-1: Sample measurement setup with three fixed probe locations for initial test point of (0°, 0°)

For the sample multi-AoA system investigated, the angular difference between AoA1 (z direction) and AoA2 (x direction) and between AoA1 (z direction) and AoA2 (y direction) is 90° in both cases. One could seemingly argue that the roll motor of the DUT (rotation around the DUT z axis) makes it irrelevant whether AoA2 is along the y or the x axis. However, once the device is rotated in 3D, i.e., q and f, this argument is no longer applicable which is further analysed and illustrated below. For a second sample device orientation of (q, f) = (45°, -45°), the perceived DL directions are illustrated on the rotated DUT grid in Figure 5.3.1-2. Clearly, a 90° rotation of the device around its z axis would no longer map the green sphere onto the red sphere.

Figure 5.3.1-2: Sample measurement setup with three fixed probe locations for test point of (45°, -45°)

It is now assumed that the test points in 3D are based on a constant-step size grid with 266 unique grid points (Dq=Df=15°), i.e., as defined for PC1 & PC3 spherical coverage test cases for 1-DL in Clause 5.3 of [6], or on a constant density grid with 200 unique grid points, i.e., as defined for PC1 spherical coverage test cases for 1-DL in Clause 5.3 of [6]. These test/grid points are visualized in Table 5.3.1-1.

Table 5.3.1-1: Visualization of spherical coverage test/grid points

| Constant-step size grid with 266 unique grid points (Dq=Df=15°) | Constant density grid with 200 unique grid points |
| --- | --- |

When all spherical coverage grid points are sampled with 0°≤q≤180° and 0°≤f≤360°, the DL directions perceived by the UE from the various probes are plotted in Table 5.3.1-2. Clearly, these results show vastly different results.

-	For the probe aligned with the z axis, the perceived DL directions match the test points.

-	For the probe aligned with the y axis, the DUT only perceives the DL in a single plane only, i.e., the DUT’s xy plane. This is due to the turntable axis of the OTA positioner aligned with the y axis, i.e., the turntable rotation merely changes the polarization but not the DL direction with this probe. The roll motor, i.e., the rotation of the DUT around its z axis, only adjusts the DL direction within the xy plane.

-	For the AoA2 probe aligned with the x axis, the DL directions are perceived from different directions in (q, f) but limited to a single hemisphere only.

The plots of the perceived DL directions from different, fixed probes demonstrate the need to define the absolute probe locations/directions for UE RF systems so that different test systems yield the same results if the probes are placed arbitrarily and not within the xz plane.

Table 5.3.1-2: DL directions perceived by the DUT from probes in the x, y, and z axes.

| Probe Along Direction | DL Directions perceived by DUT with constant-step size grid with 266 unique grid points (Dq=Df=15°) | DL Directions perceived by DUT with constant density grid with 200 unique grid points |
| --- | --- | --- |
| z |  |  |
| y |  |  |
| x |  |  |

When probes are aligned on the xz plane while meeting the required relative angular offsets, absolute probe locations are no longer required for multi-Rx chain DL reception testing.

The effect of the 2 AoA DL polarizations perceived by the UE using orientation vectors will be demonstrated next; additional details regarding the orientation vectors can be found in Clause 5.3.5. The AoA1-AoA2 DL orientation vectors are the vectors between the DL orientations perceived by the UE from AoA1 to AoA2 and are visualized by plotting the AoA1 DL directions on a sphere with the black sphere while the direction towards the corresponding offset AoA2 DL direction is visualized with a single arrow (the length of the arrow corresponds to the value of the offset), e.g.,

Three different probe placement examples with probes on the xz plane are used next together with a 15° grid spacing. In the first example, a sample 4-probe system supporting {30°, 60°, 90°, 120°, and 150°} relative angular offsets is used as shown in Figure 5.3.1-3.

Figure 5.3.1-3: Example system implementation with P0 placed at q=0°

In the second example, as illustrated in Figure 5.3.1-4, the P0 probe is no longer aligned with the z axis but rotated by 30° towards the -x axis.

Figure 5.3.1-4: Example system implementation with P0 placed at q=-30°

In the third example, the AoA1&AoA2 probes are placed so that they are symmetric to the z axis as illustrated in Figure 5.3.1-5.

Figure 5.3.1-5: Example system implementation with probes symmetric to the z axis: left: P0 & P60, centre: P0 & P120, right: P0 & P150.

The AoA1-AoA2 DL orientation vectors for the two system/probe configurations are further illustrated in Table 5.3.1-3. Clearly, the vectors are all aligned, i.e., absolute probe locations are not necessary as long as the probes are placed on the xz plane.

Table 5.3.1-3: Vectors of DL orientations between AoA1 (P0) and AoA2 from three different system configurations with AoA2 to AoA1 probe offsets of 60°, 120°, and 150° with a constant-step size grid with Dq=Df=15°). Full rotation in q and half rotation in f.

|  | Probes in the xz plane with P0 at q=0° | Probes in the xz plane with P0 at q=-30° | Probes in the xz plane symmetric to z axis |
| --- | --- | --- | --- |
| System Configuration |  |  |  |
| Orientation between Probes P0 and P60 |  |  |  |
| Orientation between Probes P0 and P120 |  |  |  |
| Orientation between Probes P0 and P150 |  |  |  |

<<< END OF CHANGES >>>